3GPP TSG-RAN5 Meeting #109	R5-255906r2

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4189 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TT to NR NTN TC 14.1.11 and 14.1.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT analysis and test applicability of NR NTN TC 14.1.11 and 14.1.12 are finished while TT related information are missing in 38.533. The titles of TC14.1.11 and 14.1.12 are not aligned with the work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting the title of TC 14.1.11 and 14.1.12 as per the work plan. - Deleting related editors notes. - Updating NR NTN TCs including TT. - Editorial correcting to 14.1.11.4.2 step 8. - Editorial correcting to 14.1.12.2. - Removing some incorrect uses of “TT” in Table 14.1.11.5-1 and 14.1.11.5-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 14.1.11 and 14.1.12 can’t be finished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.11, 14.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.903 CR 1102 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1：Correct some relevant values associated with "TT" in Tables 14.1.11.5-1, Table 14.1.11.5-2 and Table 14.1.12.5-1. r2： Correct ‘EUTRA’ or ‘E-UTRA’ to ‘E-UTRAN’ for the title of 14.1.11 and text contents, unified format; Correct the editorial issues for test case 14.1.12, align the formatting of the subclauses to H6. Updated the Io value in Table 14.1.12.5-1 |  |  |  |  |  |  |  |  |

==============First change==============

14.1.11	NR SA FR1 inter-RAT Cell reselection to E-UTRAN TN carrier for Satellite Access

14.1.11.1	Test purpose

This test is to verify the requirement for the NR NTN to E-UTRAN TN inter-RAT cell reselection requirements specified in TS 38.133 [6] clause 4.2C.2.11 when the E-UTRAN cell is of higher priority.

14.1.11.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access.

14.1.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.11.

14.1.11.4	Test description

The test scenario comprises of one NR cell and one E-UTRAN cell as given in Tables 14.1.11.4.1-1, 14.1.11.4.1-3, 14.1.11.5-1 and 14.1.11.5-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. NR Cell 1 is already identified by the UE prior to the start of the test. E-UTRAN Cell 2 is of higher priority than Cell 1.

14.1.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.11.4.1-1.

Table 14.1.11.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 14.1.11-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 14.1.11-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 14.1.11-3 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| 14.1.11-4 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases, and the UE is only required to be tested in one of the supported test configurations of the applicable scenario (GSO or NGSO). |  |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.11.4.1-2.

Table 14.1.11.4.1-2: Initial conditions for NR NTN to E-UTRAN cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.11.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.11.4.1-3.

2.	Message contents are defined in clause 14.1.11.4.3.

3.	The test scenario comprises of one NR NTN cell and one E-UTRAN cell. Cell 1 is the NR NTN cell and Cell 2 is the E-UTRAN neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.11.4.1-3:  General test parameters for NR NTN to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1-4 | Cell 1 | The UE camps on Cell 1 in the initial phase and during T2 period the UE reselects to Cell 2. |
| T2 end | Active cell |  | 1-4 | Cell 2 | The UE shall perform reselection to cell 2 during T2. |
| condition | Neighbour cell |  | 1-4 | Cell 1 |  |
| Access Barring Information |  | - | 1-4 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1-4 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1-4 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1-4 | 53 | As specified in Table 5.7.1-2 in TS 36.211 [24] |
| T1 |  | s | 1-4 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1-4 | 70 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.11.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR NTN Cell (Cell 1) and an E-UTRAN TN neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T1 in Table 14.1.11.5-1 and 14.1.11.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.11.5-1 and 14.1.11.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority cell, Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	The TE shall switch off and on the UE and set the parameters according to T1 in Table 14.1.11.5-1 and 14.1.11.5-28.

8.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.11.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.1.11.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 2 and higher priority Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

14.1.11.5	Test requirement

Tables 14.1.11.5-1 and 14.1.11.5-2 define the primary level settings including test tolerances for higher priority E-UTRAN cell re-selection test case.

Table 14.1.11.5-1: Cell specific test parameters for NR Cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Satellite information |  | 1,3 | SSC.1 |  |
|  |  | 2,4 | SSC.2 |  |
| TDD configuration |  | 1-4 | N/A |  |
| PDSCH parameters |  | 1-4 | SR.1.1 FDD |  |
| RMSI CORESET parameters |  | 1-4 | CR.1.1 FDD |  |
| Dedicated CORESET parameters |  | 1-4 | CCR.1.1 FDD |  |
| SSB parameters |  | 1-4 | SSB.1 FR1 |  |
| NR SMTC parameters |  | 1-4 | SMTC.2 |  |
| OCNG Pattern |  | 1-4 | OP.1 defined in A.2.1 |  |
| Initial DL BWP configuration |  | 1-4 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1-4 | ULBWP.0.1 |  |
| RLM-RS |  | 1-4 | SSB |  |
| Qrxlevmin | dBm/SCS | 1-4 | -140 |  |
|  | dBm/SCS | 1-4 | -98 |  |
|  | dBm/15 kHz | 1-4 | -98 |  |
| SS-RSRP | dBm/SCS | 1-4 | -84 | -84 |
|  | dB | 1-4 | 14 | 14 |
|  | dB | 1-4 | 14 | 14 |
| Io | dBm/9.36 MHz | 1-4 | -55.88 | -55.88 |
| Treselection | s | 1-4 | 0 |  |
| SnonintrasearchP | dB | 1-4 | 50 |  |
| Threshx, highP (Note 2) | dB | 1-4 | 48 |  |
| Threshserving, lowP | dB | 1-4 | 44 |  |
| Threshx, lowP | dB | 1-4 | 50 |  |
| Propagation Condition |  | 1-4 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRAN target cell |  |  |  |  |

Table 14.1.11.5-2: Cell specific test parameters for E-UTRAN Cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRAN RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [23] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2 OP.2 FDD for test configuration 3, 4 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -infinity | -84.4 |
|  | dB | -infinity | 13.6 |
|  | dB | -infinity | 13.6 |
| TreselectionE-UTRAN | s | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP (Note 2) | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, Low which is included in E-UTRAN system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRAN,

Where:

Thigher_priority_search	See TS 38.133 [6] clause  4.2C.2.9

Tevaluate, NR_inter	See TS 38.133 [6] clause 4.2C.2.11TSI-E-UTRAN	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

14.1.12	NR SA FR1-FR1 Cell reselection to TN carrier for Satellite Access

14.1.12.1	Test purpose

This test is to verify the requirement for the inter frequency NR NTN to TN cell re-selection requirements specified in clause TS 38.133 [6] clause 4.2C.2.11.

14.1.12.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access.

14.1.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.12.

14.1.12.4	Test description

The test scenario comprises of 2 cells on 2 different NR carriers, including NR NTN cell 1 on RF channel 1 and NR TN cell 2 on RF channel 2 respectively as given in Tables 14.1.12.4.1-1, 14.1.12.4.1-2 and14.1.12.4.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Cell 1 is already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

14.1.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.12.4.1-1.

Table 14.1.12.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 14.1.12-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 14.1.12-2 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 14.1.12-3 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 14.1.12-4 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 14.1.12-5 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 14.1.12-6 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases, and the UE is only required to be tested in one of the supported test configurations of the applicable scenario (GSO or NGSO). |  |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.12.4.1-2.

Table 14.1.12.4.1-2: Initial conditions for NR SA FR1-FR1 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.12.4.1-3.

2.	Message contents are defined in clause 14.1.12.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 1 and Cell 2 are configured according to Annex C.1.2.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.12.4.1-3: General test parameters for inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1-6 | Cell 1 |  |
| T2 end condition | Active cell |  | 1-6 | Cell 2 |  |
|  | Neighbour cells |  | 1-6 | Cell 1 |  |
| RF Channel Number |  |  | 1-6 | 1,2 | Cell 1 is on RF channel 1 Cell 2 is on RF channel 2 |
| Time offset between cells |  |  | 1-6 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1-6 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1-6 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1-6 | SMTC.6 | Configured in SIB4 for Cell 1 and Cell 2 |
| DRX cycle length |  | s | 1-6 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1-6 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1-6 | Not configured |  |
| Ephemeris information |  |  | 1-6 | [TBD] | The detailed configuration is specified in SIB19 |
| T1 |  | s | 1-6 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1-6 | 70 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| Note 1: Detailed ephemeris information is provided in TS 38.508-1 [14] |  |  |  |  |  |

14.1.12.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Cell 1 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in [state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.12.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.12.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

7.	The TE shall switch off and on the UE and set the parameters according to T2 in Table 14.1.11.5-1 and 14.1.11.5-2.

8.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 1.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.12.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.1.12.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |

14.1.12.5	Test requirement

Tables 14.1.12.5-1 defines the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 14.1.12.5-1: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1,2,3 | SSC.1 |  | N/A |  |
|  |  | 4,5,6 | SSC.2 |  |  |  |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2,5 |  |  | TDDConf.1.1 |  |
|  |  | 3,6 |  |  | TDDConf.2.1 |  |
| PDSCH RMC |  | 1,4 | SR.1.1 FDD |  | SR.1.1 FDD |  |
| configuration |  | 2,5 |  |  | SR.1.1 TDD |  |
|  |  | 3,6 |  |  | SR.2.1 TDD |  |
| RMSI CORESET |  | 1,4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| RMC configuration |  | 2,5 |  |  | CR.1.1 TDD |  |
|  |  | 3,6 |  |  | CR.2.1 TDD |  |
| Dedicated CORESET |  | 1,4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| RMC configuration |  | 2,5 |  |  | CCR.1.1 TDD |  |
|  |  | 3,6 |  |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1-6 | OP.1 defined in clause A.2.1 |  | OP.1 defined in clause A.2.1 |  |
| Initial DL BWP configuration |  | 1-6 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1-6 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1-6 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1,2,4,5 | -140 |  | -140 |  |
|  |  | 3,6 |  |  | -137 |  |
| Pcompensation | dB | 1-6 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1-6 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1-6 | 14 | 14 | -infinity | 13.6 |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  | -98 |  |
|  |  | 3,6 |  |  | -95 |  |
| Note2 | dBm/15 kHz | 1-6 | -98 |  |  |  |
|  | dB | 1-6 | 14 | 14 | -infinity | 13.6 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4,5 | -84 | -84 | -infinity | -84.4 |
|  |  | 3,6 |  |  | -infinity | -84.4 |
| Io | dBm/Ch BW | 1,2,4,5 | -55.88 | -55.88 | -70.05 | -56.26 |
|  |  | 3,6 |  |  | -66.96 | -50.17 |
| Treselection | s | 1-6 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1-6 | 50 |  | 50 |  |
| Threshx, highP | dB | 1-6 | 48 |  | 48 |  |
| Threshserving, lowP | dB | 1-6 | 44 |  | 44 |  |
| Threshx, lowP | dB | 1-6 | 50 |  | 50 |  |
| Propagation Condition |  | 1-6 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_inter + TSI-NR,

Where:

Thigher_priority_search	See TS 38.133 [6] clause 4.2C.2.9

Tevaluate, NR_inter	See Table 4.2C.2.4-1 in TS 38.133 [6] clause 4.2C.2.10

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority.

==============End of change==============