3GPP TSG-RAN5 Meeting #108	R5-255919r1

# Dallas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5194 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC 12.2.11.1 Inter-concurrent operation / DRX / Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Test Purpose and Test procedure of SL DRX are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Test Purpose and Test procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Deleted the steps related to timer and added a Note at step 9B. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.2.11.1	Inter-concurrent operation / DRX / Transmission

12.2.11.1.1	Test Purpose (TP)

(1)

with { UE is in RRC_CONNECTED state and its serving cell broadcasts SIB12 including sl-ConfigCommonNR and sl-DRX-ConfigCommonGC-BC }

ensure that {

when { UE is configured with sl-ScheduledConfig and received an UEAssistanceInformationSidelink message from the associated peer UE and did not transmit SidelinkUEInformationNR message with sl-DRX-InfoFromRxList previously }

then { UE sends a SidelinkUEInformationNR message with sl-DRX-InfoFromRxList to report the sidelink DRX assistance information }

}

(2)

with { UE in RRC_CONNECTED state and configured with sl-ScheduledConfig and sl-FreqInfoToAddModList in sl-ConfigDedicatedNR }

ensure that {

when { UE receives RRCReconfiguration message containing sl-ConfigDedicatedNR with sl-DRX-ConfigUC-ToAddModList }

then { UE sends an RRCReconfigurationSidelink message with sl-DRX-ConfigUC-PC5 }

}

(3)

with { UE is in RRC_CONNECTED state and its serving cell broadcasts SIB12 including sl-FreqInfoList and sl-DRX-ConfigCommonGC-BC }

ensure that {

when { UE received a sidelink DRX configuration in the RRCReconfigurationSidelink message for NR sidelink unicast reception from the associated peer UE and did not transmit SidelinkUEInformationNR message with sl-RxDRX-ReportList previously }

then { UE sends a SidelinkUEInformationNR message with sl-RxDRX-ReportList to report the accepted sidelink DRX configuration }

}

(4)

with { UE is in RRC_CONNECTED state and its serving cell broadcasts SIB12 including sl-ConfigCommonNR and sl-DRX-ConfigCommonGC-BC }

ensure that {

when { UE is configured with sl-ScheduledConfig and received an UEAssistanceInformationSidelink message with the changed sidelink DRX assistance information from the associated peer UE }

then { UE sends a SidelinkUEInformationNR message with sl-DRX-InfoFromRxList to report the changed sidelink DRX assistance information }

}

(5)

with { UE is in RRC_CONNECTED state and its serving cell broadcasts SIB12 including sl-ConfigCommonNR and sl-DRX-ConfigCommonGC-BC }

ensure that {

when { UE is configured with sl-ScheduledConfig and received an RRCReconfigurationCompleteSidelink message including the sl-DRX-ConfigReject from the associated peer UE and did not transmit SidelinkUEInformationNR message with sl-FailureList previously }

then { UE sends a SidelinkUEInformationNR message with sl-Failure to report the sidelink DRX configuration reject information }

}

(6)

with { UE is in RRC_CONNECTED state and having established PC5 RRC connection with peer UE and SL DRX is configured }

ensure that {

when { Data arrives at logical channel and sl-drx-Cycle is configured and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle) = sl-drx-StartOffset }

then { UE transmits the SCI and data after sl-drx-SlotOffset from the beginning of the subframe when sl-drx-onDurationTimer is running }

}

12.2.11.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.8.9.6.3, 5.8.3.1, 5.8.3.2, 5.8.3.3, 5.3.5.3, 5.3.5.14, 5.8.9.1.1, 5.8.9.1.2, 5.8.9.1.9, TS 38.321, clause 5.28.3 and 5.28.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.8.9.6.3]

For sidelink unicast, when a UE is in RRC_CONNECTED and is performing sidelink operation with resource allocation mode 1, it may report the sidelink DRX assistance information received within the UEAssistanceInformationSidelink from its peer UE to the network as specified in 5.8.3. For sidelink unicast, when a UE is in RRC_CONNECTED and is performing sidelink operation with resource allocation mode 2 or is in RRC_IDLE or RRC_INACTIVE or out of coverage, regardless of whether the UE has obtained the sidelink DRX assistance information from the UEAssistanceInformationSidelink transmitted from its peer UE or not, it may determine the sidelink DRX configuration SL-DRX-ConfigUC for its peer UE.

NOTE:	When UE determines the sidelink DRX configuration for its peer UE, it may take the sidelink DRX assistance information received from its peer UE into account.

[TS 38.331, clause 5.8.3.1]

The purpose of this procedure is to inform the network that the UE:

-	…

-	is reporting the accepted sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception,

-	is reporting the sidelink DRX assistance information received from the associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig,

-	…

-	is reporting DRX configuration reject information from its associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig,

[TS 38.331, clause 5.8.3.2]

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception, upon accepting the sidelink DRX configuration from the associated peer UE. A UE capable of NR sidelink communication that is configured with sl-ScheduledConfig and is performing sidelink unicast transmission may initiate the procedure to report the sidelink DRX assistance information or the sidelink DRX configuration reject information received from the associated peer UE, upon receiving either of them from the associated peer UE.

…

Upon initiating this procedure, the UE shall:

1>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

2>	ensure having a valid version of SIB12 for the PCell;

…

2>	if configured by upper layers to perform NR sidelink reception on the frequency included in sl-FreqInfoList in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

3>	if the UE received a sidelink DRX configuration in the RRCReconfigurationSidelink message for NR sidelink unicast reception from the associated peer UE and the UE accepted the sidelink DRX configuration:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxDRX-ReportList; or if the information carried by sl-RxDRX-ReportList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX configuration in accordance with 5.8.3.3;

3>	else:

4>	if the last transmission of the SidelinkUEInformationNR message included sl-RxDRX-ReportList:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the sidelink DRX configuration is no longer used in accordance with 5.8.3.3;

…

2>	if configured by upper layers to perform NR sidelink transmission on the frequency included in sl-FreqInfoList in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs and if the UE is configured with sl-ScheduledConfig:

3>	if the UE received a sidelink DRX assistance information or a sidelink DRX configuration reject information from the associated peer UE for NR sidelink unicast transmission:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-DRX-InfoFromRxList or sl-FailureList; or if the information carried by sl-DRX-InfoFromRxList or sl-FailureList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX assistance information or the sidelink DRX configuration reject information in accordance with 5.8.3.3;

NOTE:	After including the SL-DRX reject information in sl-FailureList in the last transmission of the SidelinkUEInformationNR message, it is up to UE implementation to consider another sidelink DRX rejection of a new SL DRX configuration from the same associated peer UE as "change" of sl-FailureList.

[TS 38.331, clause 5.8.3.3]

The UE shall set the contents of the SidelinkUEInformationNR message as follows and shall include all concerned information, irrespective of what triggered the procedure:

…

1>	if the UE initiates the procedure to report to the network the sidelink DRX configuration for NR sidelink unicast reception; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission; or

…

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

…

3>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

…

4>	if configured by upper layers to perform NR sidelink transmission and configured with sl-ScheduledConfig:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay and/or sl-FailureList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-InfoFromRxList to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

6>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

7>	set sl-Failure as drxReject-v1710 for the associated destination for the NR sidelink communication transmission;

…

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

[TS 38.331, clause 5.3.5.14]

Upon initiating the procedure, the UE shall:

…

1>	if sl-DRX-ConfigUC-ToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each SL-DestinationIndex included in the received sl-DRX-ConfigUC-ToReleaseList that is part of the current UE configuration:

3>	remove the entry with the matching SL-DestinationIndex from the stored NR sidelink DRX configuration information;

1>	if sl-DRX-ConfigUC-ToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is part of the current stored NR sidelink DRX configuration:

3>	reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink DRX configuration information;

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is not part of the current stored NR sidelink DRX configuration:

3>	add a new entry for this sl-DestinationIndex to the stored NR sidelink DRX configuration.

NOTE 2:	The UE is expected to update the mapping between the Destination Layer-2 ID and the destination index for the stored NR sidelink DRX configuration after the UE updates the destination list and reports to the gNB.

[TS 38.331, clause 5.8.9.1.1]

Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

…

-	the (re-)configuration of the peer UE to perform sidelink DRX;

[TS 38.331, clause 5.8.9.1.2]

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

…

1>	set the sl-DRX-ConfigUC-PC5 as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED and if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

4>	set the sl-DRX-ConfigUC-PC5 according to stored NR sidelink DRX configuration information for this destination;

NOTE 2:	If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, or in RRC_CONNECTED and sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration, it is up to UE implementation to set the sl-DRX-ConfigUC-PC5.

[TS 38.331, clause 5.8.9.1.9]

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied.

2>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

3>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication.

[TS 38.321, clause 5.28.3]

The UE transmitting SL-SCH Data should keep aligned with its intended UE receiving the SL-SCH Data regarding the SL DRX Active time as specified in clause 5.28.2.

Furthermore, the UE transmitting SL-SCH Data determines the SL DRX Active time based on SL DRX timers that are running (e.g., sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer, sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer, sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer) or will be running in the future (e.g., sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer, sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer, sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer) at the UE(s) receiving SL-SCH data. The UE may select resource for the initial transmission of groupcast within the time when sl-DRX-GC-BC-OnDurationTimer or sl-DRX-GC-InactivityTimer of the destination is running.

[TS 38.321, clause 5.28.2]

The MAC entity shall:

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI).

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID and the cast type indicator in the SCI is set to groupcast:

4>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is indicated in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else if PSFCH resource is configured for the SL grant associated to the SCI:

4>	set the sl-drx-HARQ-RTT-Timer based on sl-drx-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is enabled, or based on sl-drx-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is disabled, for resource pool configured with PSFCH.

3>	else (i.e., if PSFCH resource is not configured for the SL grant associated to the SCI):

4>	set the sl-drx-HARQ-RTT-Timer as 0 slots.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization.

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process.

12.2.11.1.3	Test Description

12.2.11.1.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

-	NR-SS-UE

-	NR-SS-UE1 operates as NR sidelink communication device on the resources that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.2.11.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.2.11.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | service no. 119 V2X is available |  |
| EFVST |  | Service no.2 V2X policy configuration data over PC5 is supported, i.e. value is '01 02' HEX |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in 38.508-1 [4] Table 4.10.1-1 |  |

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast) and GNSS Sync (On) as defined in TS 38.508-1 [4], subclause 4.5.1 using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.11.1.3.2	Test procedure sequence

Table 12.2.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS-NW transmits an RRCReconfiguration message with sl-ConfigDedicatedNR to configure transmission resources and network scheduling-based sidelink transmission. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The NR-SS-UE1 transmits an UEAssistanceInformationSidelink message. | <-- | PC5-RRC: UEAssistanceInformationSidelink | - | - |
| 4 | Check: Does the UE send a SidelinkUEInformationNR message to report to the network the sidelink DRX assistance information received from step 3? | --> | NR RRC: SidelinkUEInformationNR | 1 | P |
| 5 | SS-NW transmits an RRCReconfiguration message with sl-ConfigDedicatedNR to configure the sidelink DRX configurations. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | Check: Does the UE transmit an RRCReconfigurationSidelink message? | --> | PC5-RRC: RRCReconfigurationSidelink | 2 | P |
| 8 | The NR-SS-UE1 transmits an RRCReconfigurationCompleteSidelink message. | <-- | PC5-RRC: RRCReconfigurationCompleteSidelink | - | - |
| 9 | Check: Does the UE send a SidelinkUEInformationNR message to report to the network the accepted sidelink DRX configuration? | --> | NR RRC: SidelinkUEInformationNR | 3 | P |
| 9A | The SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 9B | Check: Does the UE transmit the first MAC PDU after sl-drx-SlotOffset from the beginning of the subframe when sl-drx-onDurationTimer is running? NOTE: The UE sends multiple packets. Only the first one is considered to achieve the verdict, the rest are discarted. | --> | MAC PDU | 6 | P |
| 9C | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 10 | The NR-SS-UE1 transmits an UEAssistanceInformationSidelink message with changed sidelink DRX assistance information. | <-- | PC5-RRC: UEAssistanceInformationSidelink | - | - |
| 11 | Check: Does the UE send a SidelinkUEInformationNR message to report to the network the changed sidelink DRX assistance information received from step 10? | --> | NR RRC: SidelinkUEInformationNR | 4 | P |
| 12 | SS-NW transmits an RRCReconfiguration message with sl-ConfigDedicatedNR to configure the sidelink DRX configurations. | <-- | NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 14 | Check: Does the UE transmit an RRCReconfigurationSidelink message? | --> | PC5-RRC: RRCReconfigurationSidelink | 2 | P |
| 15 | The NR-SS-UE1 transmits an RRCReconfigurationCompleteSidelink message with sl-DRX-ConfigReject-v1720. | <-- | PC5-RRC: RRCReconfigurationCompleteSidelink | - | - |
| 16 | Check: Does the UE send a SidelinkUEInformationNR message to report to the network the sidelink DRX configuration reject information? | --> | NR RRC: SidelinkUEInformationNR | 5 | P |

12.2.11.1.3.3	Specific message contents

Table 12.2.11.1.3.3-1: SIB12-IEs (Preamble and all steps)

Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A with condition SL_DRX

Table 12.2.11.1.3.3-2: RRCReconfiguration (Steps 1, 5 and 12, Table 12.2.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ConfigDedicatedNR with condition SCHEDULING |  | Step 1 |
|  |  | SL-ConfigDedicatedNR with condition SL_DRX |  | Step 5 and 12 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-3: SL-ConfigDedicatedNR (Table 12.2.11.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |  |
| sl-ScheduledConfig-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ScheduledConfig as defined in TS 38.508 [4] Table 4.6.6-29. |  | Step 1 |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PHY-MAC-RLC-Config-v1700 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-DRX-Config-r17 |  | SL-DRX-Config with condition SL_DRX_UC_AddMod | where SL-DRX-ConfigUC is specified in Table 12.2.11.1.3.3-7 | Step 5 and 12 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-4: UEAssistanceInformationSidelink (Steps 3 and 10, Table 12.2.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-9 with condition RX |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformationSidelink-r17 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformationSidelink-r17 SEQUENCE { |  |  |  |  |
| sl-PreferredDRX-ConfigList-r17 SEQUENCE (SIZE (1..maxNrofSL-RxInfoSet-r17)) OF SL-DRX-ConfigUC-SemiStatic-r17 { |  | 1 entry |  |  |
| SL-DRX-ConfigUC-SemiStatic-r17[1] |  | SL-DRX-ConfigUC-SemiStatic | entry 1, Table 12.2.11.1.3.3-10 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-5: SidelinkUEInformationNR (Steps 4 and 11, Table 12.2.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28A with condition SIDELINK_TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SidelinkUEInformationNR-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| sidelinkUEInformationNR-r16 SEQUENCE { |  |  |  |
| sl-TxResourceReqList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16 { | 1 entry |  |  |
| SL-TxResourceReq-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-DestinationIdentity-r16 | SL-DestinationIdentity-r16 |  |  |
| sl-CastType-r16 | unicast |  |  |
| sl-RLC-ModeIndicationList-r16 | Not checked |  |  |
| sl-QoS-InfoList-r16 | Not checked |  |  |
| sl-TypeTxSyncList-r16 | Not checked |  |  |
| sl-TxInterestedFreqList-r16 | Not checked |  |  |
| sl-CapabilityInformationSidelink-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-TxResourceReqList-v1700 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-v1700 { | 1 entry |  |  |
| SL-TxResourceReq-v1700[1] SEQUENCE { |  | entry 1 |  |
| sl-DRX-InfoFromRxList-r17 SEQUENCE (SIZE (1..maxNrofSL-RxInfoSet-r17)) OF SL-DRX-ConfigUC-SemiStatic-r17 { | 1 entry |  |  |
| SL-DRX-ConfigUC-SemiStatic-r17[1] | SL-DRX-ConfigUC-SemiStatic | entry 1, Table 12.2.11.1.3.3-10 |  |
| } |  |  |  |
| sl-DRX-Indication-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-RxDRX-ReportList-v1700 | Not checked |  |  |
| sl-RxInterestedGC-BC-DestList-r17 | Not checked |  |  |
| sl-RxInterestedFreqListDisc-r17 | Not checked |  |  |
| sl-TxResourceReqListDisc-r17 | Not checked |  |  |
| sl-TxResourceReqListCommRelay-r17 | Not checked |  |  |
| ue-Type-r17 | Not checked |  |  |
| sl-SourceIdentityRemoteUE-r17 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.11.1.3.3-6: RRCReconfigurationSidelink (Steps 7 and 14, Table 12.2.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition TX and SL_DRX |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-DRX-ConfigUC-PC5-r17 CHOICE { |  |  |  |
| setup | SL-DRX-ConfigUC | Table 12.2.11.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.11.1.3.3-7: SL-DRX-ConfigUC (Tables 12.2.11.1.3.3-3 and 12.2.11.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-9C |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigUC-r17 ::= SEQUENCE { |  |  |  |  |
| sl-drx-onDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms200 |  |  |
| } |  |  |  |  |
| sl-drx-InactivityTimer-r17 |  | ms1280 |  |  |
| sl-drx-HARQ-RTT-Timer1-r17 |  | Not Present |  |  |
| sl-drx-HARQ-RTT-Timer2-r17 |  | Not Present |  |  |
| sl-drx-RetransmissionTimer-r17 |  | sl16 |  |  |
| sl-drx-CycleStartOffset-r17 CHOICE { |  |  |  |  |
| ms10240 |  | 0 |  |  |
| } |  |  |  |  |
| sl-drx-SlotOffset |  | 0 |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-8: RRCReconfigurationCompleteSidelink (Step 8, Table 12.2.11.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-4 with condition RX

Table 12.2.11.1.3.3-9: SidelinkUEInformationNR (Step 9, Table 12.2.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SidelinkUEInformationNR-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| sidelinkUEInformationNR-r16 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-TxResourceReqList-v1700 | Not checked |  |  |
| sl-RxDRX-ReportList-v1700 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxDRX-Report-v1700 { | 1 entry |  |  |
| SL-RxDRX-Report-v1700[1] SEQUENCE { |  | entry 1 |  |
| sl-DRX-ConfigFromTx-r17 | SL-DRX-ConfigUC-SemiStatic | Table 12.2.11.1.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |
| sl-RxInterestedGC-BC-DestList-r17 | Not checked |  |  |
| sl-RxInterestedFreqListDisc-r17 | Not checked |  |  |
| sl-TxResourceReqListDisc-r17 | Not checked |  |  |
| sl-TxResourceReqListCommRelay-r17 | Not checked |  |  |
| ue-Type-r17 | Not checked |  |  |
| sl-SourceIdentityRemoteUE-r17 | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.11.1.3.3-10: SL-DRX-ConfigUC-SemiStatic (Tables 12.2.11.1.3.3-4, 12.2.11.1.3.3-5 and 12.2.11.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-9D |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DRX-ConfigUC-SemiStatic-r17 ::= SEQUENCE { |  |  |  |  |
| sl-drx-onDurationTimer-r17 CHOICE { |  |  |  |  |
| milliSeconds |  | ms200 |  | Steps 3, 4 and 9 |
|  |  | ms80 |  | Steps 10 and 11 |
| } |  |  |  |  |
| sl-drx-CycleStartOffset-r17 CHOICE { |  |  |  |  |
| ms10240 |  | 0 |  |  |
| } |  |  |  |  |
| sl-drx-SlotOffset-r17 |  | 0 |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-11: RRCReconfigurationCompleteSidelink (Step 15, Table 12.2.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-4 with condition RX |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationCompleteSidelink ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationCompleteSidelink-r16 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| dummy |  | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-DRX-ConfigReject-v1720 |  | true |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.11.1.3.3-12: SidelinkUEInformationNR (Step 16, Table 12.2.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28A with condition SIDELINK_TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SidelinkUEInformationNR-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| sidelinkUEInformationNR-r16 SEQUENCE { |  |  |  |
| sl-TxResourceReqList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16 { | 1 entry |  |  |
| SL-TxResourceReq-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-DestinationIdentity-r16 | SL-DestinationIdentity-r16 |  |  |
| sl-CastType-r16 | unicast |  |  |
| sl-RLC-ModeIndicationList-r16 | Not checked |  |  |
| sl-QoS-InfoList-r16 | Not checked |  |  |
| sl-TypeTxSyncList-r16 | Not checked |  |  |
| sl-TxInterestedFreqList-r16 | Not checked |  |  |
| sl-CapabilityInformationSidelink-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-FailureList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-Failure-r16 { | 1 entry |  |  |
| SL-Failure-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-DestinationIdentity-r16 | SL-DestinationIdentity-r16 |  |  |
| sl-Failure-r16 | drxReject-v1710 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>