3GPP TSG-RAN5 Meeting #109	R5-255989r1

# Dallas, November 17 - 21 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3646 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the description of reducing CA BW combinations in tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of how to reduce the number of BW combinations in test frequency tables is only valid for non contiguous CA, including UL. It needs to be updated to include also contiguous CA tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description moved to new clause and description updated to also include intra band contiguous CA tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description on how to reduce tables for contiguous CA test frequency tables. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to discussion paper R5-255986 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Correction of clause numbering. - Addition of Note to make sure signalling frequencies are included in tables. |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

### C.2.4.3	Determination of test frequencies for NR Intra-band Non-Contiguous CA

#### C.2.4.3.1	General

The default test frequencies in clause 4.3.1 for NR Intra-band Non-Contiguous CA are based on maximum Wgap between the carrier components of the different bands taking the UE capability of maximum supported frequency separation between the lower edge of lowest component carrier and upper edge of highest component carrier for UL for FR1, and UL and DL for FR2.

Test frequencies with Wgap different from maximum Wgap are specified in the specific test cases using them.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NSB | Number of sub-block in the for NR Intra-band non-contiguous configuration |
| SBCBWDL(i) | Downlink channel bandwidth (MHz) of sub-block SB(i), where i = 1 to NSB |
| maxFsBW | Maximum frequency separation bandwidth between the lower edge of lowest component carrier and upper edge of highest component carrier. |
| maxWgap | maxWgap is the maximum separation in MHz between each sub-block in the NR Intra-band non-contiguous configuration within maxFsBW for a given sub-block combination. |

#### C.2.4.3.1A	Selection of maximum frequency separation for FR1

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1A-1:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | Min(Full NR bandwidth,100 MHz) |
|  | >=100 MHz and <200 MHz | Min(Full NR bandwidth,200 MHz) |
|  | >=200 MHz and <600 MHz | Min(Full NR bandwidth,600 MHz= |
|  | =600 MHz | Full NR bandwidth |

#### C.2.4.3.1B	Selection of maximum frequency separation for FR2

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1B-1:

Table C.2.4.3.1B-1: Selecting maxFsBW for FR2

| Type of configuration | Aggregated CBW | maxFsBW (Note 1) |
| --- | --- | --- |
| With non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz | N/A |
| Without non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz and <1600 MHz | 1600 MHz |
|  | >=1600 MHz and <1800 MHz | 1800 MHz |
|  | >=1800 MHz and <2000 MHz | 2000 MHz |
|  | >=2000 MHz and <2200 MHz | 2200 MHz |
|  | >=2200 MHz and <2400 MHz | 2400 MHz |
| Note 1: For FR2 intra-band non-contiguous CA configurations with non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for UL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. For FR2 intra-band non-contiguous CA configurations without non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for DL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. |  |  |

#### C.2.4.3.2	Determination of test frequencies for a sub-block combination

Editor's note: The number of test points for FR2 intra-band non-contiguous CA configurations is under investigation, e.g. “Low” and “High”, or “Mid”.

1.	Calculate the maxWgap value between sub-blocks for the sub-block combination:

| maxWgap = (maxFsBW – ∑k=1 to NSB SBCBWDL(k)) / (NSB-1) | C.2.4.3.3-Eq1 |
| --- | --- |

2.	Calculate test frequencies for all sub-blocks in the sub-block combination:

If the maxFsBW is smaller than the full bandwidth of the NR band then calculate test frequencies for both Low and High range. Else only for the Low range.

For Low range the test frequencies are calculated such that the lower edge of the lowest component carrier of the lowest frequency sub-block is located at the lower edge of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

For High range the test frequencies are calculated such that the upper edge of the highest component carrier of the highest frequency sub-block is located at the upper edge frequency of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

Within each sub-block the test frequencies and parameters of the sub-block are calculated based on the location of the sub-block and the relevant principles in clause C.2, C3 and C4 for the type of component carriers in the sub-block.

The test frequencies for CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 is calculated without CORESET#0 as specified in C.3.3. CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 can only be used for NR Intra-band Non-Contiguous CA configurations with mixed numerologies.

#### C.2.4.3.3	Void

#### C.2.4.3.4	Void

### C.2.4.4	Determination of CBW combinations to add in initial test frequency tables for Intra band CA

There is a need, for some bands, to limit the number of initial CBW combinations in the tables for the test frequencies for intra band non-contiguous and contiguous CA.  If the total number of CBW combination exceeds a value of 35 combinations, the number of combinations should be reduced according to the steps described in this clause. The intention is to include the CBW combinations that is the most likely to be needed for testing.

The determination of which CBW combinations to add in the initial tables, is based on two statements stated in test cases:

1.	Initial conditions for test: Lowest NRB_agg,  Highest NRB_agg.

2.	If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested.

The determination of initial CBW combinations to add in new tables for the test frequencies shall follow below steps:

1.	One, and only one, combination for each possible value of aggregated CBW for a specific BCS. The maximum aggregated BW in table will be limited to Maximum supported Aggregated CBW as stated in 38.521-1 [14], Table 5.5A.2-1. The aggregated BW of the combinations will be restricted to even 10 MHZ values.

2.	Each combination according to step 1 above shall be configured to use the highest NRB_PCC (CBW) possible, to comply with statement 2 above.

Note:	Since tables might be referenced to define default test frequencies for signalling tests in clause 6.2.3, BW combinations that are needed for signalling tests shall be added to the table in addition to the combinations defined above, if needed.

## C.2.5	Frequency determination for supplemental uplink

##### << End of changes>>