3GPP TSG-RAN5 Meeting #109	R5-256035r1

Dallas, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0019 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.1.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK, Ericsson, FirstNet, Element |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case ‘6.1.23 On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Pre-established session / Preconfigured Group Use Only’ included in the Work Plan, is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case ‘6.1.23 On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Pre-established session / Preconfigured Group Use Only’ according to the Work Plan. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case ‘6.1.23 On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Pre-established session / Preconfigured Group Use Only’ included in the Work Plan will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.23 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.579-4 CR 0007 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated TP. Updated core requirements. Updated clase 6.1.23.3.2, Test procedure sequence. |  |  |  |  |  |  |  |  |

Table 6.9.1.3.3-11: MCData-Regroup in SIP MESSAGE (Table 6.9.1.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-2, condition REMOVE

### 6.1.23	On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Pre-established session / Preconfigured Group Use Only

6.1.23.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a group SDS session using the media plane and using the pre-established session to a group configured for preconfigured group use only  }

then { UE (MCDATA Client) indicate to the MCDATA user that group standalone SDS message  is not allowed on the indicated group and does not send a SIP INVITE request }

}

6.1.23.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 9.2.5.3.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 9.2.5.3.1]

Upon receiving a request from an MCData user to initiate group SDS session within the pre-established session, the MCData client shall determine whether the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element. If a <preconfigured-group-use-only> element exists and is set to the value "true", then the MCData client:

1)	should indicate to the MCData user that SDS sessions are not allowed on the indicated group; and

2)	shall skip the remainder of this procedure.

Upon receiving a request from an MCData user to initiate group SDS session within the pre-established session, the MCData client shall generate a SIP REFER request outside a dialog as specified in IETF RFC 3515 [51] as updated by IETF RFC 6665 [36] and IETF RFC 7647 [52], and in accordance with the UE procedures specified in 3GPP TS 24.229 [5], with the clarifications given below.

The MCData client:

1)	shall set the Request URI of the SIP REFER request to the session identity of the pre-established session;

2)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [51] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [33] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [18], and with the Content-ID header field set to this "cid" URL;

3)	shall include in the application/resource-lists MIME body a single <entry> element containing a "uri" attribute set to the MCData group identity, extended with the following parameters in the headers portion of the SIP URI:

NOTE:	Characters that are not formatted as ASCII characters are escaped in the following parameters in the headers portion of the SIP URI.

a)	an hname "body" parameter populated with:

i)	an application/sdp MIME body containing an SDP offer with media attributes specified in clause 9.2.3.2.1, if a group standalone SDS message is requested;

ii)	an application/vnd.3gpp.mcdata-info MIME body with:

A)	if a group standalone SDS message is requested, the <request-type> element set to a value of "group-sds". If a group SDS session is requested, the <request-type> element set to a value of "group-sds-session";

B)	the <mcdata-request-uri> element set to the MCData group identity;

C)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

D)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP REFER request, the <functional-alias-URI> element set to the URI of the used functional alias; and

E)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

3A)	if the MCData user has requested the origination of an MCData emergency group communication or is originating an MCData group communication and the MCData emergency state is already set:

a)	if this is an authorised request for an MCData emergency group communication as determined by the procedures of clause 6.2.8.1.8, shall execute the procedures in clause 6.2.8.1.1; and

b)	if this is an unauthorised request for an MCData emergency group communication as determined in step a) above, should indicate to the MCData user that they are not authorised to initiate an MCData emergency group communication;

3B)	if the MCData client emergency group state for this group is set to "MDEG 2: in-progress" or "MDEG 4: confirm-pending", shall include the Resource-Priority header field and execute the procedures in clause 6.2.8.1.2;

3C)	if the MCData user has requested the origination of an MCData imminent peril group communication:

a)	if this is an authorised request for an MCData imminent peril group communication as determined by the procedures of clause 6.2.8.1.8, shall execute the procedures in clause 6.2.8.1.9; and

b)	if this is an unauthorised request for an MCData imminent peril group communication as determined in step a) above, should indicate to the MCData user that they are not authorised to initiate an MCData imminent peril group communication;

3D)	if the MCData client imminent peril group state for this group is set to "MDIG 2: in-progress" or "MDIG 4: confirm-pending", shall include the Resource-Priority header field and execute the procedures in clause 6.2.8.1.12;

4)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), according to IETF RFC 6050 [7];

5)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

6)	shall include the following according to IETF RFC 4488 [53]:

a)	the option tag "norefersub" in the Supported header field; and

b)	the value "false" in the Refer-Sub header field;

7)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [54] identifying the pre-established session;

8)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP REFER request according to IETF RFC 3840 [16]; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [5].

On receiving a final SIP 2xx response to the SIP REFER request, the MCData client:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15].

On receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to the SIP REFER request:

1)	if the MCData emergency group communication state is set to "MDEGC 2: emergency-communication-requested" or "MDEGC 3: emergency-communication-granted" or	if the MCData imminent peril group communication state is set to "MDIGC 2: imminent-peril-communication-requested" or "MDIGC 3: imminent-peril-communication-granted", the MCData client shall perform the actions specified in clause 6.2.8.1.5 and shall skip the remaining steps.

On receiving a SIP re-INVITE request within the pre-established session targeted by the sent SIP REFER request, the MCData client:

0)	if the sent SIP REFER request was a request for an MCData emergency group communication or an MCData imminent peril group communication, the MCData client:

a)	shall perform the actions specified in clause 6.2.8.1.16;

b)	shall check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [5];

c)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

d)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [5], based upon the parameters already negotiated for the pre-established session; and

e)	shall send the SIP 200 (OK) response towards the participating MCData function according to rules and procedures of 3GPP TS 24.229 [5].

1)	if the <mcdata-communication-state> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "establish-success":

i)	shall notify MCData user about successful the MCData communication establishment;

2)	if the <mcdata-communication-state> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "establish-fail":

i)	shall notify MCData user about the MCData communication establishment failure; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15].

On communication release by interaction with the media, if the sent SIP REFER request was a request for an MCData emergency group communication or an MCData imminent peril group communication, the MCData client shall perform the procedures specified in clause 6.2.8.1.17.

On receiving a SIP INFO request where the Request-URI contains an MCData session ID identifying an ongoing group session, the MCData client shall perform the procedures specified in clause 6.2.8.1.13.

6.1.23.3	Test description

6.1.23.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2 with the following clarifications:

-	The <max-payload-size-sds-cplane-bytes> element of the MCData Service Configuration document shall be set to 0 to force the MCData client to send the data using the media plane.

-	The UE has performed procedure 'MCX pre-established session establishment' as described in TS 37.579-1 [2] clause 5.3.3.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

-	A pre-established session is established.

6.1.23.3.2	Test procedure sequence

Table 6.1.23.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send a group session SDS message using the pre-established session with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP REFER request to establish a MCData call within the next 5s? | - | REFER | 1 | F |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication to the MCData user that group session SDS messagesare not allowed on the indicated group. | - | - | 1 | P |
| 3a1 | IF pc_MCX_DisplayInfoSDSNotAllowedOnGroup THEN Check: Does the UE (MCData client) indicate to the MCData user that groupSDS is not allowed on the indicated group? (NOTE 1) | - | - | - | 1 |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.23.3.3	Specific message contents

Table 6.1.23.3.3-1: Group Configuration document (Preamble)

| Derivation Path: TS 37.579-1 [2], Table 5.5.7.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:preconfigured-group-use-only | true |  | TS 24.481 [31] clause 7.2.4.2 |  |