3GPP TSG-RAN WG5 Meeting #109	R5-256051r1

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3567 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Adjacent channel leakage ratio for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 6.5J.2.3 Adjacent channel leakage ratio for ATG UE is missing and need to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.5J.2.3 Adjacent channel leakage ratio for ATG UE has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5J.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.905 CR 1097 TS 38.522 CR 0673 TS 38.521-1 CR 3568 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related TP analysis CR is R5-256048(CR 1097). The related applicability CR is R5-256055(CR 0673). The related Annex F MU/TT updated CR is R5-256052(CR 3568). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Update the upper frequency limit for the TT to 6GHz since there is no ATG band beyond 6GHz. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.5J.2.3	Adjacent channel leakage ratio for ATG

Editor’s Note: The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.5J.2.3.1	Test purpose

Same test purpose as in 6.5.2.4.1.1.

6.5J.2.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.5J.2.3.3	Minimum conformance requirements

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. For ATG UE, the requirements for ACLR are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5J.2.3.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value 30dBc.

Table 6.5J.2.3.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5J.2.3.

6.5J.2.3.4	Test description

Same test description as in clause 6.5.2.4.1.4 with the following exceptions:

Power is measured as the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array)

6.5J.2.3.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2.2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5J.2.3.5-2.

Table 6.5J.2.3.5-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Table 6.5J.2.3.5-2: NR ACLR requirement

NR ACLR

30 - TT dB

NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5J.2.3.5-3.

Table 6.5J.2.3.5-3: Test Tolerance (NR ACLR)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

6.5J.3	Spurious emissions for ATG

6.5J.3.0	General

<< END OF CHANGES >>