3GPP TSG-RAN5 Meeting #109	R5-256103r2

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0111 | rev | - | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of band n252 to UE conformance Tx/Rx specification – clause 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network, Terrestar, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 finalized the normative work on defining NTN band n252 in RAN#108. Consequently RAN5 agreed the revised WID: New Rel-19 NR licensed bands and extension of existing NR bands (UID-1080080) which include this band. This CR is needed to progress the RAN5 UE conformance Tx/Rx requirement specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to clause 5: Update to operating band, UE channel bandwidth, asymmetric channel bandiwidth, channel raster and Tx-Rx frequency separation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band n252 UE conformance specification can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 5.2	Operating bands

### 5.2.1	General

NTN satellite covers FR1-NTN operating bands in the present specification.

### 5.2.2	Operating bands with conducted requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1,980MHz – 2,010 MHz | 2,170 MHz – 2,200 MHz | FDD |
| n255 | 1,626.5 MHz – 1,660.5 MHz | 1,525 MHz – 1,559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| n252 | 2000 – 2020 MHz | 2180 – 2200 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

==============Next change==============

## 5.3	UE channel bandwidth

<<Unchanged parts are omitted >>

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in Table 5.3.2-1 and Table 5.3.2-2 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n256 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n255 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
| n254 | 15 | 5 | 10 | 15 |  |
|  | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |
| n252 | 15 | 5 | 10 | 15 | 20 |
|  | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| NTN satellite band | SCS (kHz) | UE channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 2001 |  |
|  | 120 | 50 | 100 | 2001 | 4001 |
| n511 | 60 | 50 | 100 | 2001 |  |
|  | 120 | 50 | 100 | 2001 | 4001 |
| n510 | 60 | 50 | 100 | 2001 |  |
|  | 120 | 50 | 100 | 2001 | 4001 |
| NOTE 1: This UE channel bandwidth is optional in Rel-18. |  |  |  |  |  |

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 | 0 |
|  | 15 | 20 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

==============Next change==============

## 5.4	Channel arrangement

<<Unchanged parts are omitted >>

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NREF-Offs are given in Table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3,000 | 5 | 0 | 0 | 0 – 599,999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For the uplink of FDD FR1 NTN bands defined in Table 5.2.2-1 except bands n247, n248

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3.

<<Unchanged parts are omitted >>

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NTN satellite operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NTN satellite operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396,000 – <20> – 402,000 | 434,000 – <20> – 440,000 |
| n255 | 100 | 325,300 – <20> – 332,100 | 305,000 – <20> – 311,800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

For NTN operating bands with 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-2 is given as <2>.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN for enhanced channel raster

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 | Yes |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 | Yes |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 | Yes |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 | Yes |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |  |

For FR2-NTN satellite operating bands, ΔFRaster = I1 × ΔFGlobal for UL channel and ΔFRaster = I2 × ΔFGlobal for DL channel, where (I1, I2) ϵ {(1,4) or (2, 8)}. But (I1, I2) = (2, 8) only applies under the condition that 120kHz SCS is configured in the channel and SSB SCS is equal to or larger than 120kHz. In this case, every I1th NR-ARFCN for UL channel and I2th NR-ARFCN for DL channel are applicable for the UL and DL channel raster correspondingly within the operating band and the <(UL_step size, DL_step size)> for the UL and DL channel raster in Table 5.4.2.3-3 is given as <(I1, I2)> for FR2-NTN.

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |

<<Unchanged parts are omitted >>

### 5.4.3	Synchronization raster

<<Unchanged parts are omitted >>

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1 and table 5.4.3.3-2. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1 for FR1-NTN and table 5.4.3.3-2 for FR2-NTN.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5,429 – <1> – 5,494 |
| n255 | 15 kHz | Case A | 3,818 – <1> – 3,892 |
|  | 30 kHz | Case B | 3,824 – <1> – 3,886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6220 – <1> – 6238 |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

<<Unchanged parts are omitted >>

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n252 | 180 MHz1 165 to 195 MHz2 |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

==============End of change==============