3GPP TSG RAN WG5#109	R5-256346r1

# Dallas, USA, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3663 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CodebookConfig-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Information elements for CodebookConfig-r18 for new PMI test cases are missing in 38.508-1. These elements are PMI test case specific and could therefore reside under clause 5.4.2, Radio resource control information elements for Demodulation Performance and CSI reporting tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added 3 new tables for CodebookConfig-r18, N1-N2-CBSR-r18 and TD-DD-Config-r18 - A placeholder for CodebookConfig-r17 added in Table 5.4.2.5-20 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CodebookConfig-r18 will still be missing in 38-508-1, resulting in confusion in new PMI test cases and unnecessary duplication of information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Corrected meeting date on cover page |  |  |  |  |  |  |  |  |

#### << Start of changes >>

Table 5.4.2.5-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  | 2Tx, 4Tx, 8Tx |
|  | aperiodic-ZP-CSI-RS-ResourceSetsToAddModList |  | 16Tx, 32Tx |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet |  | 2Tx, 4Tx, 8Tx |
|  | Not present |  | 16Tx, 32Tx |
| } |  |  |  |

CodebookConfig-r16

Table 5.4.2.5-19: CodebookConfig-r16

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig-r16 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type2 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeII-r16 SEQUENCE { |  |  |  |
| N1-n2-codebookSubsetRestriction-r16 |  |  |  |
| Four-two | 0x 7FF FFFF FFFF FFFF FFFF |  |  |
| } |  |  |  |
| typeII-RI-Restriction-r16 | 0010 |  |  |
| } |  |  |  |
| } |  |  |  |
| numberOfPMI-SubbandsPerCQI-Subband-r16 | 1 |  |  |
| paramCombinatin-r16 | 6 | (L =4, pν =1/2, β=1/2 ) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.4.2.5-20

Table 5.4.2.5-21: CodebookConfig-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig-r18 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type2 CHOICE { |  |  |  |
| typeII-Doppler-r18 SEQUENCE { |  |  |  |
| n1-n2-codebookSubsetRestrictionList-r18 | N1-N2-CBSR-r18 |  |  |
| paramCombination-Doppler-r18 | 7 |  |  |
| td-dd-config-r18 | TD-DD-Config-r18 |  |  |
| numberOfPMI-SubbandsPerCQI-Subband-r18 | 1 |  |  |
| predictionDelay-r18 | n1 |  |  |
| typeII-RI-Restriction-r18 | 0010 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.4.2.5-22: N1-N2-CBSR-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| N1-N2-CBSR-r18 ::= CHOICE { |  |  |  |
| four-two-r18 CHOICE { |  |  |  |
| cbsr-r18 | 0x 7FF FFFF FFFF |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.4.2.5-23: TD-DD-Config-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TD-DD-Config-r18 ::= SEQUENCE { |  |  |  |
| vectorLengthDD-r18 | n1 |  |  |
| unitDurationDD-r18 | m1 |  |  |
| aperiodicResourceOffset-r18 | 2 |  |  |
| tdCQI-r18 | Not present |  |  |
| } |  |  |  |

#### 5.4.2.6	Message contents for RI reporting requirements

#### << End of changes >>