3GPP TSG-RAN5 Meeting #109	R5-256398r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3608 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of TCs 6.3J.3 for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test tolerances missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test tolerances fo TC 6.3J.3.3 and 6.3J.3.4. Updated editor notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_ATG-UEConTest work item cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J.3.3 6.3J.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Table 6.3J.3.3.5-2 and Table Table 6.3J.3.3.5-3 removed instead a Note added in Table 6.3J.3.3.5-1 - Table 6.3J.3.4.5-2 and Table 6.3J.3.4.5-3 removed instead a Note added in Table Table 6.3J.3.4.5-1 |  |  |  |  |  |  |  |  |

==============First change==============

#### 6.3J.3.2	Transmit power time mask for slot and short or subslot boundaries for ATG

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can’t provide enough resolution to identify non-conformant UEs. Therefore, the minimum requirement is not testable.

#### 6.3J.3.3	PRACH time mask for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-  MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

-  MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.3J.3.3.1	Test purpose

Same test purpose as in 6.3.3.4.1.

6.3J.3.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.3.3.3	Minimum conformance requirements

The PRACH time mask requirements as specified in clause 6.3.3.4.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.3

6.3J.3.3.4	Test description

Same test description as in clause 6.3.3.4.4 with following exceptions:

-	The ON power is measured as sum of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

-	The OFF power is measured at each transmit antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

6.3J.3.3.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3J.3.3.5-1.

Table 6.3J.3.3.5-1: PRACH time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+R+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Expected PRACH Transmission ON Measured Power for PRACH Format 0 and PRACH Format A3 for SCS 30kHz | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm |
| Expected PRACH Transmission ON Measured Power for PRACH Format A3 for SCS 15kHz and SCS 60kHz | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm |
| ON Power Tolerance | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT and Relaxation (R) for each frequency and channel bandwidth of OFF power is specified in Table 6.3J.2.5-2 and Table 6.3J.2.5-3 respectively NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2J.1.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.3J.3.4	SRS time mask for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-  MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

-  MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.3J.3.4.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3J.3.4.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.3.4.3	Minimum conformance requirements

The SRS time mask requirements as specified in clause 6.3.3.6.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

not apply during this transient period.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.3.

6.3J.3.4.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	The ON power is measured as sum of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

-	The OFF power is measured at each transmit antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

6.3J.3.4.5	Test requirement

The measured SRS time mask power shall not exceed the values specified in Table 6.3J.3.4.5-1.

Table 6.3J.3.4.5-1: SRS time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
| Transmit OFF power |  | ≤ -50+R+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Table 6.2.1.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT and Relaxation (R) for each frequency and channel bandwidth of OFF power is specified in Table 6.3J.2.5-2 and Table 6.3J.2.5-3 respectively NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2J.1.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

==============End of change==============

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)