3GPP TSG-SA WG6 Meeting #67	S6-252487

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252053)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0010 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for Update digital asset media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In Table 8.4.3.5-1: Update digital asset media request, there is List of digital assets. But in Table 8.4.3.6-1: Update digital asset media response, there is Digital asset identifier. It is confused. - In clause 8.2 Digital asset profile management & 8.3 Digital asset discovery, for the procedures and information flows, the operations just for one Digital asset. - So it is suggested that keep one Digital asset for Update digital asset media operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 8.4.3.5 Update digital asset media request, no need parameter List of digital assets. S6-251376 has already changed parameter Digital asset profile identifier to Digital asset identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with Digital asset profile management & Digital asset discovery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.4.3.5	Update digital asset media request

Table 8.4.3.5-1 describes the information elements for an update digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.5-1: Update digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset profile identifier | M | Identifier of the digital asset profile |
| List of digital asset media | M | List of digital asset media to be updated |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

8.4.3.6	Update digital asset media response

Table 8.4.3.6-1 describes the information elements for the update digital asset media response sent from a DA server to a requestor (e.g. DA client).

Table 8.4.3.6-1: Update digital asset media response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure cause is provided. |
| Digital asset identifier | M | An identifier for the digital asset. |
| Cause | O | The cause information when the status is fail. |

# * * * * End of changes * * * *