3GPP TSG-SA3 Meeting #121	S3-251696

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025   merge of S3-251329, S3-251452, S3-251499

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.545 | CR | 0002 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Some terminology corrections in TS 33.545 clause 5.2.1 and 5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto_Sec |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clauses 5.2.1 and 5.2.2 have some inconsistencies in terms “NR Femto NF” used in the figure but has missing descriptions, and “5FMTF” used in descriptions but this abbreviation is not explained. This CR proposes to use a consistent term : “NR Femto Authentication Proxy (NRFAP)” which is as per the functionality of this function. - Clause 5.5 “HNB” should be replaced with “H(e)NB” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced “5FMTF” with “NRFAP” in descriptions and figure in clauses 5.2.1 and 5.2.2. - Replaced “HNB” with H(e)NB in clause 5.5 - Updated abbreviations list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in terminologies and bbreviations can cause confusions in understanding clause 5.2.1 and 5.2.2 and maybe incorrectly interpreted in the future. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 5.2.2, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of First Change *************

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

5GS	5G System

AAA	Authentication, Authorization, and Accounting

AKA	Authentication and Key Agreement

AUSF	Authentication Server Function

CAG	Closed Access Group

CSG	Closed Subscriber Group

eNB	Evolved Node-B

EAP	Extensible Authentication Protocol

H(e)NB	Home NodeB or Home eNodeB

H(e)MS	Home NodeB Management or Home eNodeB Management System

HeMS	Home eNodeB Management System

HeNB	Home eNodeB

HMS	Home NodeB Management System

HNB	Home NodeB

HP	Hosting Party

HPM	HP Module

HSS	Home Subscriber Server

IKE	Internet Key Exchange

L-GW	Local Gateway

MSK	Master Session Key

NR	New Radio

NRFAP	NR Femto Authentication Proxy

SeGW	Security Gateway

UDM	Unified Data Management

********** End of First Change *************

********** Start of Second Change *************

### 5.2.1	General

Device Authentication of the NR Femto by the SeGW can be followed with a hosting party authentication. The security features for hosting party authentication specified in TS 33.320 [5] clause 5.3 and clause 7.3 can be derived with the following changes:

-	Replace H(e)NB with NR Femto;

-	Replace HSS with UDM;

-	EAP-AKA'-based method can be used for NR Femto hosting party authentication exchange.

NR Femto Authentication Proxy (NRFAP) can be co-located with SeGW or an independent function deployed in the 5GC.

NR Femto hosting party can plays the role of CAG owner to operate (add/delete/modify) on the allowed CAG list of the users after authenticated and authorized by the 5GC. The security aspects of NEF specified in clause 12 of TS 33.501[7] can be reused when NR Femto hosting party operating on the allowed list in the UDM. Refer to clause 5.5 of the present document for relevant access control mechanisms.

### 5.2.2	Combined device and hosting party authentication procedure

NRFAP can act as the proxy for NR Femto authentication. NRFAP can be co-located with SeGW or an independent function deployed in the 5GC. The procedure between the NR Femto, SeGW and EAP authentication server is shown in Figure 5.2.2.1.

Figure 5.2.2.1: NR Femto hosting party authentication using EAP-AKA' authentication method

1. The NR Femto establishes connection between the NR Femto and the SeGW.

2. The SeGW sends an EAP Identity/Request to the NR Femto.

3. The 5G NR Femto sends an EAP Response/Identity message. The NR Femto uses the SUCI in NAI format as its identity.

4. The EAP Response/Identity message is routed over SeGW towards the NRFAP based on the realm part of the SUCI.

5. The NRFAP sends the message Nausf_UEAuthentication_Authenticate Request with SUCI, Access Network Identity and NRFemto_indicator towards the AUSF. NRFemto_indicator is used to indicate to the AUSF that the authentication request is for NR Femto purposes. The NRFAP sets the Access Network Identity to "5G: NRFemto".

6. Based on the NRFemto_indicator, the AUSF (acting as the EAP authentication server) sends a Nudm_UEAuthentication_Get Request to the UDM, including SUCI and the Access Network Identity and NRFemto_indicator.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF. SIDF de-conceal SUCI to gain SUPI before UDM can process the request. Based on the NRFAP indicator and if NRFAP is allowed based on the subscription data, the UDM/ARPF selects the EAP-AKA' authentication method and generate an authentication vector using the Access Network Identity as the KDF input parameter. The UDM includes the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´) and may include SUPI to AUSF in a Nudm_UEAuthentication_Get Response message.

8. The AUSF stores XRES for future verification. The AUSF sends the EAP-Request/AKA'-Challenge message to the NRFAP in a Nausf_UEAuthentication_Authenticate Response message.

9. The NRFAP sends the EAP-Request/AKA'-Challenge message to the SeGW.

10. The SeGW forwards the EAP-Request/AKA'-Challenge message to the NR Femto.

11. At receipt of the RAND and AUTN in the EAP-Request/AKA'-Challenge message, the NR Femto derives CK' and IK' using the Access Network Identity as the KDF input parameter. The NR Femto may derive MSK from CK’ and IK’. When the NR Femto is performing NR Femto authentication, the KAUSF does not be generated by the 5G NR Femto.

12. The NR Femto sends the EAP-Response/AKA'-Challenge message to the SeGW.

13. The SeGW forwards the EAP-Response/AKA'-Challenge message to the NRFAP.

14. The NRFAP send the Nausf_UEAuthentication_Authenticate Request with EAP-Response/AKA'-Challenge message to AUSF.

15. The AUSF verifies if the received response RES matches the stored and expected response XRES. If the AUSF has successfully verified, it continues as follows to step 16, otherwise it returns an error to the NRFAP. The AUSF derives the required MSK key from CK’ and IK’, based on the NRFAP_indicator received in step 5. The AUSF does not generate the KAUSF.

16. The AUSF sends Nausf_UEAuthentication_Authenticate Response message with EAP-Success and MSK key to NRFAP. The AUSF may optionally provide the SUPI to NRFAP. The AUSF/UDM does not perform the linking increased home control to subsequent procedures.

17. The NRFAP sends the EAP-success and MSK to SeGW. The EAP-Success message is forwarded from SeGW to the NR Femto.

18. Upon receiving the EAP-Success message, the NR Femto derives the MSK, if it has not derived the MSK earlier. The NR Femto uses the first 256-bit of MSK as PMK to establish a secure connection with the SeGW.

********** End of  Second Change *************

********** Start of  Third Change *************

## 5.5	Access Control Mechanisms for Femto

Clause 8.2.1 in TS 33.320 [5] shall be reused by replacing H(e)NB with NR Femto.

The existing CAG concept defined for PNI-NPN in TS 23.501 [2] caluse 5.30.3 shall  be re-used for Femto access control.

********** End of  Third Change *************