3GPP TSG-SA WG1 Meeting #111 	S1-253096

Gotheborg, Sweden, 25 - 29 Aug 2025	(revision of S1-25xxxx)

Title:	Enhanced Access Control

Agenda Item:	8.1.2

Source:	Qualcomm

Contact:	Francesco Pica, fpica@qti.qualcomm.com

Abstract: This contribution proposes new potential requirements, for the 6G TR, on enhanced UE access control.

1. Introduction

With the advent of new 6G services, traffic and device types, this paper looks at potential enhancements of the 5G framework on Unified Access Control (UAC) [22.261], including the following areas:

- New Access Identities and/or Access Categories

- User/UE context related conditions

- Flexible per-user/UE access control policies

2. Reason for Change

Introduce potential service requirements related to UAC enhancements, to provide more granular, flexible and configurable access control for 6G.

3. Proposal

It is proposed to agree following text toward TR 22.870 Clause 5 (System and Operation aspects).

================================ pCR PROPOSAL ===============================

## 5.x	Enhanced Access Control

### 5.x.1	Description

New Access Identities and/or Access Categories

UAC defines a certain set of Access Categories and Identities [22.261].

With the expected variety of devices that will be supported by 6G, it would be beneficial to allow further differentiation of access control based on device type, e.g., for vehicles, drones, IOT, FWA. 
This may be achieved by introducing new Access Identities, which can be based on subscription and/or dedicated (per-UE) network authorization, as for MPS or MCS in UAC. Such Identities may be provisioned and stored in the ME or SIM.

6G will also support different services and traffic types, which may also benefit from specific access control parameters, e.g., for application data like background traffic, XR or Streaming (see, e,g, GSMA NG.141), or beyond-communication type of data, e.g. AI and Sensing. This may be achieved e.g., via introducing new 6G Access Categories.

When introducing enhanced Access Categories/Identities, corresponding connection establishment mapping and/or reject causes should be extended accordingly. CN overload control mechanisms can also become more granular, considering new and more flexible UE-type or service-based load indication, from CN to RAN, of which traffic to reject or bar.

UE context-aware conditions

The 5G UAC framework [22.261] includes, as part of the Access Categories definition, some “conditions related to UE”, but only a very limited granularity is defined (i.e. for delay tolerant traffic). As enhancement of that, it would be beneficial to consider new UE-context related conditions, for example to differentiate access/load control for traffic coming from a stationary/parked vehicle (e.g. EV charging) vs a moving vehicle (e.g. transmitting critical ADAS data), or between certain MO traffic from UEs that are moving at low-speed vs high-speed (e.g. devices on a highway, train).

More flexible per-user/UE access control configuration

In addition to standardized access categories, based on access attempt type, appling equally to all UEs, 5G UAC supports also some form of per-UE access control policies via the Operator-defined Access Categories, which include few criteria that can be set and provisioned by the network to a UE (stored in the ME), i.e. based on DNN, OS Id + OS App Id and S-NSSAI. 6G should be able to address more flexible scenarios, allowing new configuration and differentiation of per-UE/user access control policies and criteria, including:

- Dedicated mapping of traffic type to Access Category, e.g. the network may indicate to UE-1 that a certain access attempts should map to category-A, whilst UE-2 should apply category-B, for the same traffic. The dedicated mapping may also include a combination of Access Category and Identity.

- Different per-RAT access control policies based on UE/subscription type, e.g. the network can assign different access category mapping (to UE-1 and UE-2) for traffic originated over terrestrial vs satellite access. For example, the network may differentiate SMS access attempts from a specific IOT device or subscriber, when originated over NTN, compared to TN.

- Geographical area or time-of the day, e.g. certain UEs or users can be provisioned with dedicated Access Categories to be used, or not, in specific locations (e.g. home, office, stadium, mall), or times (e.g., low/high peak hours), different than other UEs or users, using common traffic-to-category mapping.

Those dedicated access control policies, provisioned by the network, can be stored in the ME, similarly to operator defined categories, and the network should be able to update them if/when needed, via dedicated/unicast communication. The network can use e.g. subscription data, UE capabilities, UE context information (available in the network or provided by the UE) and/or user preference to customize per-UE access control policies.

### 5.x.2	Existing requirements and functionalities

TS 22.261 [x] defines UAC Access Identities and Categories, as summarized below.

UAC Access Identities [22.261, Table 6.22.2.2-1]

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS). |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3 | UE for which Disaster Condition applies (note 4) |
| 4-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access Identity 1 is used by UEs configured for MPS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 1 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 2: Access Identity 2 is used by UEs configured for MCS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN or PLMNs equivalent to HPLMN and visited PLMNs of the home country. Access Identity 2 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose, the home country is defined as the country of the MCC part of the IMSI. NOTE 4: The configuration is valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN. |  |

UAC Access Categories [22.261, Table 6.22.2.3-1]

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |
| NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UEto a NB-IOT cell connected to 5GC when the UE is authorized to send exception data. |  |  |

5G NAS spec [24.501] includes the following details about Operator-defined access category:

Operator-defined access category definitions can be signalled to the UE using NAS signalling. Each operator-defined access category definition consists of the following parameters:

a)	a precedence value which indicates in which order the UE shall evaluate the operator-defined category definition for a match;

b)	an operator-defined access category number, i.e. access category number in the 32-63 range that uniquely identifies the access category in the PLMN or SNPN in which the access categories are being sent to the UE;

c)	criteria consisting of one or more access category criteria type and associated access category criteria type values. The access category criteria type can be set to one of the following:

1)	DNN;

2)	OS Id + OS App Id of application triggering the access attempt; or

3)	S-NSSAI;

NOTE: An access category criteria type can be associated with more than one access category criteria values.

d)	optionally, a standardized access category. This standardized access category is used in combination with the access identities of the UE to determine the RRC establishment cause as specified in subclause 4.5.6.

An access attempt matches the criteria of an operator-defined access category definition, if the access attempt matches all access category criteria types included in the criteria with any of the associated access criteria type values.

Each operator-defined access category definition has a different precedence value.

Several operator-defined access category definitions can have the same operator-defined access category number.

5G NAS specs [24.501] define also detailed mapping rules between access attempts and Access Categories/Identities as well as mapping with RRC establishment causes.

5G RAN specs describe access control / barring signalling based on Access Categories and/or Identities, over-the-air [38.331] as well as between CN and RAN (e.g. for CN overload control [38.413]).

### 5.x.3	Potential New Requirements

[PR 5.x.3-001] The 6G system shall be able to support requirements on Unified Access Control (UAC) defined for 5GS [22.261].

[PR 5.x.3-002] In addition to 5GS UAC, the 6G system shall be able to support enhanced access control mechanisms, including:

- New Access Identities to differentiate access control for specific device types, e.g., vehicles, drones, IOT, FWA;

- New Access Categories to differentiate access control for specific traffic types, e.g., XR, Streaming, AI, Sensing;

- New criteria related to UE contextual conditions, e.g., stationary/mobile, low/high speed;

- Configurability of per-user/UE dedicated access control policies, provisioned by the network, including e.g., mapping of traffic type to Access Category (and/or Identity), per-RAT access control policies (e.g. TN vs NTN), applicable geographical area or time.

NOTE 1: dedicated access control policies can be stored in the ME / USIM and can be updated by the network as needed, e.g. based on subscription, UE information, user preference or specific network conditions.

NOTE 2: new Access Identities / Categories can be used also to enhance CN overload control.