3GPP TSG-SA WG1 Meeting #111 	S1-253127

25-29 August 2025, Goteborg, Sweden	(revision of S1-250xxx)

Source:	China Mobile

pCR Title:	New use case on flexible traffic routing in 6G

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Huazhang Lv, lvhuazhang@chinamobile.com

Shimei Lu, lushimei@cmdi.chinamobile.com

Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This paper provides the use cases of flexible traffic routing in 6G.

1. Introduction

Some operators can provide nation wide services in a country. And in the country, taking into account the vast territory of some country, the operator always have the provincial basis or regional basis management of network, and it may cause that the operator may deploy multiple core network in different region or region. And in order to connect each core network together to support mobility, each core network in each region shall connect to the national backbone network (can also say national transmission network).

Considering that currently, that UE doesn’t support SSC mode 3, and if UE mobility happens between two core networks belongs to the same PLMN, how to handle traffic routing flexibly is a problem for operator.

2. Reason for Change

None

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1

* * * First Change, all new text * * * *

5.9.x	Use case on flexible traffic routing in 6G

5.9.x.1	Description

Some operators can provide nation wide services in a country. And in the country, taking into account the vast territory of some country, the operator always have the provincial basis or regional basis management of network, and it may cause that the operator may deploy multiple core network in different regions, see example in Figure 5.9.x.1-1 below. And in order to connect each core network together to support mobility, each core network in each region shall connect to the national backbone network (can also say national transmission network). But this may cause the issue of intra PLMN roaming, that when UE moves from a region of PLMN that UE registers into, moves to another region of the same PLMN that different to the location that UE registers into, the UE still receives service from the PLMN of the same country as that of the PLMN.

Figure 5.9.x.1-1: Example of provincial basis or regional basis deployment of core network

Clarification of intra PLMN roaming:

The user sometimes may have subscription with operator in one region, which can be seen as the core network that UE registers into. But if the UE moves to another region, which is different from the location that UE registers into (sometime a different province), this may cause intra roaming issues. When the UE moves to the new coverage of another core network of the same PLMN that different from the location of UE registers into, this may cause the intra PLMN roaming or inter region roaming internal PLMN.

For intra PLMN romaing, there exists some inconvinience of traffic rule configuration and application traffic routing:

-	For the current UE, the UE doesn’t support SSC mode 3, as indicated in section 5.6.9 of TS 23.501[x]. And if the UE performs intra PLMN roaming with power on status, although the UE connects to the core network in roaming location, but the UP anchor still remains unchanged in the core network that UE registers into. So, in order to configure the UE in new location, frequently interaction between two core networks of different region is needed. The same case as that UE move back to registration region with UE power on status.

-	For the UE that initially power on in the location that roaming, the UE firstly connects to the core network in roaming place. But in order to support the local offloading in intra PLMN roaming area, still two core networks interactions are needed. It is the situation that to insert the new user plane that cannot controlled by the core network that UE registers into, like the ETSUN cases in section 5.34 of TS 23.501[x]. Still exists two core networks frequently interactions for configuration of traffic routing rules.

Here are some detailed descriptions of the traffic rule configuration and application traffic routing.

Firstly, for the current UE design, only SSC mode 1 is supported. The commercial maturity of SSC mode 3 is low. So, if the UE performs roaming to region B, the user plane anchor still in region A. It means the traffic to Internet service still needs to route back to region A and increase the inter-region traffic volumn. See Figure 5.9.x-2 for details. In region A, the core network configures the routing rules to route the traffic to Internet via Gateway 1. But when UE moves into the scope of region B, due to the UE doesn’t support the SSC mode 3, the IP anchor still on Gateway 1 and the traffic to Internet should be routed back to Gateway 1. And the result is the UE can not directly communicate with server via Gateway 2 using short routing in region B. So the operator should have multiple traffic routing between Gateway 2 and Gateway 1, that brings traffic burden on backbone network.

Figure 5.9.x.1-2: Example of traffic routing back to home region

Secondly, due to the UE moves into region B and the IP anchor doesn’t change, if to route the traffic from UE to Internet locally, may have other configurations. But, always core network in region A doesn’t know the topology details in region B, so the two core network may have multiple interactions. And considering that only the region B has the topology and routing information, so it is not efficient that always the region A trigger the local traffic routing in intra PLMN roaming cases. And from deployment point of view, sometimes the local access configuration in different region is not the same.

Figure 5.9.x.1-3: Example of two core network interactions

At last, when UE moves into region B, that the UE may still have the requirements to have two traffic routing, one that locally offload to Internet and another one is traffic to data network owned by enterprise in region A.

5.9.x.2	Pre-conditions

Alice purchases a SIM-card of operator X-Mobile in region A. So, Alice has the subcription with operator X-Mobile. And Alice works for company XXX and company XXX deploys the enterprise network in region A.

When Alice in region A, the operator X-Mobile can route the traffic from Alice’s UE to either to Internet or to the enterprise network.

Now, Alice plans to go to region B for sightseesing. But, Alice still needs to visit the enterprise network for some of the working. In region B, a museum now has an exhibition and the museum deploys a dedicated network in museum to provice guide service for tourists.

5.9.x.3	Service Flows

1.	Alice takes the train from region A to region B for sightseeing. Alice uses her phone to watch video on YouTube. And when Alice moves into region B, intra PLMN roaming performs.

2.	When Alice moves into region B, Alice is not aware of this roaming and Alice’s phone always keep power on. And the core network in region B finds a new UE of the operator X-Mobile moves into and also finds that this UE doesn’t have the subscription in region B.  Due to Alice’s UE doesn’t support SSC mode 3, that the IP anchor is still in region A, so currently the video traffic is still be routed to the gateway in region A.

3.	When region B aware that a new UE moves in, in order to improve the user experience and save the traffic not routing back, the region B can configure the network and support to route the application traffic to YouTube server locally in region B.

Figure 5.9.x.3-1: Operator in region B configures the traffic from Alice’s phone local offloading to Internet

4.	Alice knows that in region B, there exists an excellent museum that has exhibition. So, tomorrow, Alice opens her phone and buys museum’s ticket to see the exhibition. Alice uses her phone to connect to the network in museum, and consumes the guide services that provided by the museum such as XRM, holographic image.

Figure 5.9.x.3-2: UE in dedicated location of region B can be directly offloaded to certain network

5.	The operator in region B directly routes the traffic from Alice’s phone to the network of museum without has any interaction with core network in region A, and Alice has a high UL/DL rate and has a good experience. During the procedure of Alice consumes the service provided by museum, the operator can perform the authentication and authorization to the phone, that to authorize the UE to consume the services.

6.	Suddenly, Alice receives a call from her colleagues Bob that needs Alice to login the enterprise network to handle the work. Without any configuration on Alice’s UE, Alice starts the enterprise dedicated application on UE, and with the help of operator X-Mobile, Alice logins in successfully and handle the work.

7.	Alice thoroughly enjoys the entire trip and then takes the train back to Region A. And when Alice roaming back to Region A, the Alice is not aware of this roaming and also the phone keeps power on. The operators in Region A will consider routing the Internet traffic accessed by Alice's mobile phone through Gateway A to the Internet.

5.9.x.4	Post-conditions

Alice in Region B has a good user experience of OTT service that the traffic from OTT application can be routed to data network locally.

Alice can access to the dedicated network of museum and enjoys the service such as XRM, holographic image provided by museum. The museum can also authorize and authenticate Alice’s phone to check whether Alice can consume the service or not.

In Region B, Alice can also login the enterprise network in Region A without any changes on configuration on Alice’ phone.

During the all procedure of intra PLMN roaming, Alice doesn’t make any changes or configurations on her phone and also doesn’t aware of this intra roaming.

5.9.x.5	Existing features partly or fully covering the use case functionality

In TS 22.261[14], some of the requirements that related to roaming and traffic offloading may have some relationship with the service flow.

1) Roaming:

In section 5.1.2.1 of TS 22.261[14]:

The 5G system shall support a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's 5G Home Mobile Network.

This requirements clarify the roaming between HPLMN and VPLMN, not the roaming internal HPLMN.

2) Local offloading:

In section 6.2.1 of TS 22.261[14], it clarifies the mobility management:

With the ever-increasing multimedia broadband data volumes, it is also important to enable the offloading of IP traffic from the 5G network onto traditional IP routing networks via an IP anchor node close to the network edge. As the UE moves, changing the IP anchor node can be needed in order to reduce the traffic load in the system, reduce end-to-end latency and provide a better user experience.

Also in section 6.5.2 of TS 22.261[14], it clarifies the efficient user plane:

Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a Service Hosting Environment for specific services, modifying the path as needed when the UE moves during an active communication.

The 5G network shall be able to interact with applications in a Service Hosting Environment for efficient network resource utilization and offloading data traffic to the most suitable Service Hosting Environment, e.g. close to the UE's point of attachment to the access network or based on usage information.

NOTE:	To accomplish offloading data traffic, usage information might be exposed to the Service Hosting Environment.

Based on operator policy, the 5G system shall provide a mechanism such that one type of traffic (from a specific application or service) to/from a UE can be offloaded close to the UE's point of attachment to the access network, while not impacting other traffic type to/from that same UE.

This requirements clarify the offloading data traffic to the most suitable Service Hosting Environment that in HPLMN. But for intra PLMN roaming situation, two core network belongs to two regions needs coordination. Because always the core network that UE roaming to has the topology of routing information, but now always the core network that UE registers into controls the UE. And this intra roaming cases are not considered in this requirements.

5.9.x.6	Potential New Requirements needed to support the use case

[PR 5.9.x.6-1] The 6G network shall support efficient traffic routing to route the application traffic from UE to data network based on UE location.

[PR 5.9.x.6-2] The 6G network shall support to route the application traffic from UE to data network via the gateway which location is different from the location that UE registration.

[PR 5.9.x.6-3] The 6G network shall support the authentication and authorization for the UE that consumes third-party services.

[PR 5.9.x.6-4] The 6G network shall support the configuration of parallel traffic routing rules for UE through a unified identifier.

* * * End of Change * * * *