3GPP TSG SA WG 1 Meeting #111 	S1-253345

Goteborg, Sweeden, 25-29 August 2025

Source:	Philips International B.V.

pCR Title:	New use case on energy management for resilient emergency services

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 5.8 Sustainability and Energy Efficiency

1. Introduction

Clause 5.6 in TR 22.870 focuses on resilience aspects. Clause 5.8 in TR 22.870 contain several use cases related to Sustainability and Energy Efficiency. This contribution proposes a new use case related to both topics that is proposed to be included in Clause 5.8.

2. Reason for Change

In emergency scenarios, the ability to transmit and receive critical messages is contingent on sufficient battery power in User Equipment (UE). However, many users experience issues such as battery depletion, particularly during extended power outages or network disruptions. This can result in the inability to make emergency calls or receive public warning system (PWS) messages, posing a risk to life and safety. Research and real-world examples indicate this is a non-negligible issue, with significant implications for individuals in vulnerable situations, such as the elderly or those in remote areas. To address this, the proposed use case envisions a mechanism where the 6G system actively performs energy management and savings, ensuring the resilience of emergency services even under adverse conditions. This approach aims to extend battery life in UEs and optimize energy resources in the network, enabling the transmission and reception of critical messages for longer durations and mitigating risks of emergency services disruption during emergency situations.

3. Proposal

Include the following use case in Clause 5.8.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[W]	Kongsiriwattana, Watcharachai, Paul Gardner-Stephen, and Matthew Lloyd. "Historical distribution of duration of unplanned power outages in queensland: Insights for sustaining telecommunications during disasters." 2017 IEEE Global Humanitarian Technology Conference (GHTC). IEEE, 2017.

[X]	National Emergency Number Association (NENA), https://www.nena.org/page/911statistics

[Y]	Journal of Emergency Medical Services (JEMS), https://www.jems.com/ems-management/does-this-911-call-require-immediate-response/

[Z]	Counterpoint Technology Market Research, https://www.counterpointresearch.com/insight/survey-72-smartphone-users-india-experience-low-battery-anxiety

* * * Next Change * * * *

---------- Use Case template ----------

## 5.6.X	Use case on energy management for resilient emergency services

### 5.6.X.1	Description

In the US, according to National Emergency Number Association [X], around 240 million calls are made to 911 each year, in many areas, 80% or more are from wireless devices. According to the Journal of Emergency Medical Services [Y], in the United States, 10% of 911 calls are time sensitive. A mobile phone can be used to make the emergency call, but of course, only if the phone has battery left. In India, 72% of smartphone users experience low-battery anxiety and about half charge their smartphones twice a day [Z]. In some cases, users may forget or not have a charger at hand, so it is likely that a user’s smart phone may not have battery left when an emergency situation arises. Assuming 30% of users experience battery depletion before the end of the day, this affects approximately 90 million people daily in the U.S. If the likelihood of needing to make an emergency call on any given day is 0.002% (based on annual emergency call frequency, i.e., 0.002% ~ 240 million calls per year / (300 million users * 365 days)), the number of people unable to make an emergency call due to battery depletion would be 450 people per day in the U.S assuming that only the emergency calls that need to be made at the end of the day, when the battery is empty.

This situation becomes even worse when the network itself experiences an outage, e.g., a power grid outage as happened in Spain in April 2025 when suddenly 60% of  Spain’s power generation vanished and it took almost 20 hours to restore the power. The situation is worse because when such a power outage happens, e.g., users cannot recharge the batteries of their phones or base stations may not have power to transmit PWS messages or receive emergency messages from the users.

Other research work has arrived at similar conclusions. For instance, W. Kongsiriwattana, et. Al.[W],  write: “the risk of death due to the inability to call an emergency ambulance in Queenlands alone, which we have estimated at 4.78 per year, is higher than the risk of dying due to being killed anywhere in Australia by Australia’s infamously snakes spiders and other native poisonous creatures... That is, inability to call emergency services due to the lack of mobile phone battery represent a small, but non-zero risk to life”. In addition to the risk of death as argued by W. Kongsiriwattana, et. al. [W], accidents such as falls or strokes at home can lead to (more) severe consequences, e.g., if an elderly person cannot call for help.

In the context of 6G, it is crucial to ensure that User Equipment can perform critical services such as the reception of Public Warning System (PWS) messages or the transmission of emergency messages in a resilient manner. To this end, the 6G system should provide means to guarantee that these important services are operational as long as possible even in the event of different types of outages, such as limited power budget on the user equipment or in the network.

This use case proposes service requirements wherein, subject to operator’s policy, regulatory requirements and user consent, the 6G network enables the operator to provide means to perform energy management and saving for the purpose of enhancing the resiliency of emergency services.

### 5.6.X.2	Pre-conditions

In this use case, our user, Pedro, is an elderly person, living in a small village around 100 Km from Madrid, Spain. Pedro has difficulties walking and suffers early Alzheimer, and sometimes forgets to recharge his mobile phone.

Pedro subscribes to an emergency service so that if he falls, or suffers an accident, he can make a call to get help.

Victoria, another user, is a younger person that uses her mobile phone quite a lot, so her mobile phone tends to get depleted every day.

### 5.6.X.3	Service flows

1. Pedro's 6G smart phone continuously monitors its battery level, charging status, and periodically reports its battery level, and charging status to the network.

2. The 6G network monitors its own outage state, the outage state of the power grid, and indicates the smart phone information in the case of a network outage or a predicted smart phone outage.

3. At 11.25 am, the network determines a low battery power (10%), and  requests Pedro to recharge the phone.

4. At 11.30 am, Pedro reads the message requesting to recharge the phone, and plugs the phone to a charger.

5. At 12.25 pm, the network determines the start of a nationwide power grid outage, and starts running on emergency power. The network broadcasts its emergency power outage status. Pedro’s phone moves to an emergency power saving mode to extend (1) the smart phone battery lifetime, even if the phone still has 40%, and (2) the energy resources in the network, with the aim of ensuring that, if needed, Pedro’s smart phone can use emergency services for a longer period of time.

6. At 8 pm, almost 8 hours later, Pedro does not have electricity in his house and Pedro is very anxious due to the nationwide power outage. He goes to the garden to see when the city lights will be switched on, and falls badly. His leg is broken and he cannot get up. Although the local terrestrial network is - in the meantime - down, Pedro’s 6G smart phone still has 20% battery left thanks to the emergency power saving mode, and the smart phone can send an emergency message via the 6G non-terrestrial network.

7. Around 8:30 pm, Victoria also suffers an emergency situation. Her smart phone’s battery is critically depleted, but luckily the network has stopped all non-critical services and communications with Victoria’s smart phone upon detecting a potential UE outage due to low battery, so that she can still make the emergency call.

### 5.6.X.4	Post-conditions

At 8.15 pm, Pedro receives help from a first responders' team, avoiding lying in the garden the whole night. Afterwards, at 8:40, the first responder’s team are able to help Victoria as well.

### 5.6.X.5	Existing features partly or fully covering the use case functionality

Clause 6.15.a in TS 22.261 summarizes requirements on Energy Efficiency as a Service Criteria. Some requirements are about energy savings, e.g., “Subject to regulatory requirements and operators’ policies, the 5G system shall enable an operator to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators.” but they are not focused on ensuring the resiliency of emergency services. They are also not focused on energy savings for the UEs.

Clause 6.1.2 in TR 22.883 summarizes energy related requirements implying service adjustments, e.g., “Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to provide means to degrade service performance (e.g. QoS, bitrate) to meet energy rationing constraints.” but such requirements lack also the focus on the resiliency of emergency services.

### 5.6.X.6	Potential New Requirements needed to support the use case

[PR 5.6.X.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall provide means to perform energy management and saving for the purpose of enhancing the resiliency of emergency services.

[PR 5.6.X.6-2] Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall provide means to perform energy management and saving to enable UEs with critical battery levels to make emergency calls.

[PR 5.6.X.6-3] Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall support subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services, and that enable sending recharge reminders for prolonging the UE’s ability to make emergency calls.

[PR 5.6.X.6-4] Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall support reporting network outages, reporting power grid outages, determining potential UE outages for the purpose of enhancing the resiliency of emergency services.

* * * End of Change * * * *