3GPP TSG-SA WG1 Meeting #112 	S1-254025

17-21 November 2025, DALLAS, USA	(revision of S1-25xxxx)

Title:	Discussion on KPIs related to the Immersive and AI

Agenda Item:

Source:	Moderator

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Facilitates discussion on TR22.870 Immersive & AI KPI consolidation. Limited to relevant TRs, TSs content mirrors.

* * * Begin of Copy * *

TS 22.261 clause 3 includes the related definition for performance requirements on immersive section.

activity factor: percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network.

area traffic capacity: total traffic throughput served per geographic area.

communication service: percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service.

NOTE 3:	The end point in "end-to-end" is the communication service interface.

NOTE 4:	The communication service is considered unavailable if it does not meet the pertinent QoS requirements. For example, the communication service is unavailable if a message is not correctly received within a specified time, which is the sum of maximum allowed end-to-end latency and survival time.

end-to-end latency: the time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

positioning service latency: time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface.

reliability: in the context of network layer packet transmissions, percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted.

synchronization threshold: A synchronization threshold can be defined as the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous.

NOTE 10:	This definition is based on [41].

user experienced data rate: the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

TS 22.261 includes the related terms for performance requirements on immersive section.

Service bit rate: user-experienced data rate: Refer to Table 7.6.1-1 KPI Table for additional high data rate and low latency service in 22.261.

Message size (byte): Refer to Table 7.11-1: Multi-modal communication service performance requirements

Influence quantity: Refer to Table 7.11-1: Multi-modal communication service performance requirements

TS 22.261 includes the following related KPI tables.

Copy from TS 22.261:

7.6	High data rate and low latency

7.6.1	AR/VR

Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on audio-visual feedback. In the use cases like VR and interactive conversation the latency requirements include the latencies at the application layer (e.g. codecs), which could be specified outside of 3GPP.

To support VR environments with low motion-to-photon capabilities, the 5G system shall support:

-	motion-to-photon latency in the range of 7 ms to 15ms while maintaining the required resolution of up to 8k giving user data rate of up to [1Gbit/s] and

-	motion-to-sound delay of [< 20 ms].

NOTE: 	The motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset. The motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears.

To support interactive task completion during voice conversation, the 5G system shall support low-delay speech coding for interactive conversational services (100 ms, one-way mouth-to-ear).

Due to the separate handling of the audio and video component, the 5G system will have to cater for the VR audio-video synchronisation in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization). To support VR environments the 5G system shall support audio-video synchronisation thresholds:

-	in the range of [125 ms to 5 ms] for audio delayed and

-	in the range of [45 ms to 5 ms] for audio advanced.

The 5G system shall support service continuity for AR/VR to support immersive user experience under high UE mobility.

When it comes to implementation of applications containing AR/VR components, the requirements on the 5G network could depend on architectural choices implementing these services. Note 3 in table 7.1-1 above gives an example on such dependences for a VR application in a 5G system. Table 7.6.1-1 below illustrates additional use cases and provides more corresponding requirements on the 5G system.

-	Cloud/Edge/Split Rendering – Cloud/Edge/Split Rendering is characterised by the transition and exchange of the rendering data between the rendering server and device.

-	Gaming or Training Data Exchanging – This use case is characterised by the exchange of the gaming or training service data between two 5G connected AR/VR devices.

-	Consume VR content via tethered VR headset – This use case involves a tethered VR headset receiving VR content via a connected UE; this approach alleviates some of the computation complexity required at the VR headset, by allowing some or all decoding functionality to run locally at the connected UE. The requirements in the table below refer to the direct wireless link between the tethered VR headset and the corresponding connected UE.

Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % in uplink and 99,9 % in downlink (note 4) | - | Stationary or Pedestrian (note 7) | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % (note 4) | ≤ [10] | Stationary or Pedestrian (note 7) | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consumption of VR content via tethered VR headset (note 6) | [5 to 10] ms (note 5) | 0,1 to [10] Gbit/s (note 5) | [99,99 %] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and can be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. NOTE 7: Similar user-experienced data rates may be achievable also at higher UE speeds. [50] |  |  |  |  |  |  |

7.11	KPIs for tactile and multi-modal communication service

The 5G system shall support tactile and multi-modal communication services with the following KPIs.

Table 7.11-1: immersive communication service performance requirements

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Message size (byte) | UE Speed | Service Area |  |
| Immersive multi-modal VR (UL: device application sever) | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [40] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs can be supported by the haptic device | Stationary or Pedestrian | typically < 100 km2 (note 5) | Haptic feedback |
|  | 5 ms | < 1Mbit/s | 99.99% [40] | 1500 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Sensing information e.g. position and view information generated by the VR glasses |
| Immersive multi-modal VR (DL: application sever device) | 10 ms (note1) | 1-100 Mbit/s | 99.9% [40] | 1500 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Video |
|  | 10 ms | 5-512 kbit/s | 99.9% [40] | 50 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Audio |
|  | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [40] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Haptic feedback |
| Remote control robot | 1-20ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% [40] | 2-8 (1 DoF) | high-dynamic (≤ 50 km/h) | ≤ 1 km2 | Haptic feedback |
|  | 20-100ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% [40] | 2-8 (1 DoF) | Stationary or Pedestrian | ≤ 1 km2 | Haptic feedback |
|  | 5 ms | 1-100 Mbit/s | 99.999% [40] | 1500 | Stationary or Pedestrian | ≤ 1 km2 | Video |
|  | 5 ms | 5-512 kbit/s | 99.9% [40] | 50-100 | Stationary or Pedestrian | ≤ 1 km2 | Audio |
|  | 5 ms | < 1Mbit/s | 99.999% [40] | - | Stationary or Pedestrian | ≤ 1 km2 | Sensor information |
| Skillset sharing low- dynamic robotics (including teleoperation) Controller to controlee | 5-10ms | 0.8 - 200 kbit/s (with compression) | 99,999% [40][45] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | Stationary or Pedestrian | 100 km2 | Haptic (position, velocity) |

| Skillset sharing low- dynamic robotics (including teleoperation) Controlee to controller | 5-10ms | 0.8 - 200 kbit/s (with compression) | 99,999% [40][45] | 1 DoF: 2-8 10 DoFs: 20-80 100 DoFs: 200-800 | Stationary or Pedestrian | 100 km2 | Haptic feedback |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10ms | 1-100 Mbit/s | 99,999% [40] [45] | 1500 | Stationary or Pedestrian | 100 km2 | Video |
|  | 10ms | 5-512 kbit/s | 99,9% [40] [45] | 50 | Stationary or Pedestrian | 100 km2 | Audio |
| Skillset sharing Highly dynamic/ mobile robotics Controller to controlee | 1-5ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99,999% (with compression) 99,9% (w/o compression) [40] [45] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | high-dynamic | 4 km2 | Haptic (position, velocity) |
| Skillset sharing Highly dynamic/ mobile robotics Controlee to controller | 1-5ms | 0.8 - 200 kbit/s | 99,999% (with compression) 99,9% (w/o compression) [40] [45] | 1 DoF: 2-8 10 DoFs: 20-80 100 DoFs: 200-800 | high-dynamic | 4 km2 | Haptic feedback |
|  | 1-10ms | 1-10 Mbit/s | 99,999% [40] [45] | 2000-4000 | high-dynamic | 4 km2 | Video |
|  | 1-10ms | 100-500 kbit/s | 99,9% [40] [45] | 100 | high-dynamic | 4 km2 | Audio |
| Immersive multi-modal navigation applications Remote Site Local Site (DL) | 50 ms [39] | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999 % [40] | 1 DoF: 2 to 8 10 DoF: 20 to 80 100 DoF: 200 to 800 | Stationary or Pedestrian | ≤ 100 km2 ( note 5) | Haptic feedback |
|  | <400 ms [39] | 1-100 Mbit/s | 99.999 % [40] | 1500 | Stationary/ or Pedestrian, | ≤ 100 km2 (note 5) | Video |
|  | <150 ms [39] | 5-512 kbit/s | 99.9 % [40] | 50 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Audio |
|  | <300 ms | 600 Mbit/s | 99.9 % [40] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | VR |
| Immersive multi-modal navigation applications Local Site Remote Site (UL) | <300 ms | 12 kbit/s [26] | 99.999 % [40] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Biometric / Affective |
|  | <400 ms [39] | 1-100 Mbit/s | 99.999 % [40] | 1500 | Workers: Stationary/ or Pedestrian, UAV: [30-300mph] | ≤ 100 km2 (note 5) | Video |
|  | <150 ms [39] | 5-512 kbit/s | 99.9 % [40] | 50 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | Audio |
|  | <300 ms | 600 Mbit/s | 99.9 % [40] | 1500 | Stationary or Pedestrian | ≤ 100 km2 (note 5) | VR |
| NOTE 1: Motion-to-photon delay (the time difference between the user’s motion and corresponding change of the video image on display) is less than 20 ms, and the communication latency for transferring the packets of one audio-visual media is less than 10 ms, e.g. the packets corresponding to one video/audio frame are transferred to the devices within 10 ms. NOTE 2: According to IEEE 1918.1 [40] as for haptic feedback, the latency is less than 25 ms for accurately completing haptic operations. As rendering and hardware introduce some delay, the communication delay for haptic modality can be reasonably less than 5 ms, i.e. the packets related to one haptic feedback are transferred to the devices within 10 ms. NOTE 3: Haptic feedback is typically haptic signal, such as force level, torque level, vibration and texture. NOTE 4: The latency requirements are expected to be satisfied even when multimodal communication for skillset sharing is via indirect network connection (i.e., relayed by one UE to network relay). NOTE 5: In practice, the service area depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. |  |  |  |  |  |  |  |

7.3	High-accuracy positioning

#### 7.3.2.2	Requirements for horizontal and vertical positioning service levels

The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

NOTE: 	The requirements do not preclude any type of UE, including specific UE such as for example V2X, MTC.

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

TS 22.156 includes the following related KPI tables.

Copy from TS 22.156:

6.2	Performance requirements

[R-6.2-001] The 5G system shall support various mobile metaverse services with the following KPIs.

NOTE: 	Unless stated otherwise, the "Max allowed end-to-end latency" refers to the maximum transmission delay expected between a UE and the mobile metaverse server or vice-versa.

Table 6.2-1:	Performance requirements for Mobile Metaverse Services

| Use Cases | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed | Service Area |  |
| 5G-enabled Traffic Flow Simulation and Situational Awareness (NOTE 2) | [5-20] ms (NOTE 1) | [10~100] Mbit/s [8] (NOTE 6) | > 99.9% | 1.14 Tbit/s/km2 (NOTE 5) | - | 20~100 ms (NOTE 3) | - | < 250 km/h | City or Country wide (NOTE 4) | UL |
| Collaborative and concurrent engineering | [≤10] ms [9] (NOTE 7) | [1-100] Mbit/s [9] | [> 99.9%] [9] | [1.55] Tbit/s/km2 (NOTE 8) | Video: 1500 Audio: 100 [9] | - | - | Stationary or Pedestrian | typically < 100 km2 (NOTE 9) | UL and DL audio/video |
|  | [5] ms UL [1-50] ms DL [9] (NOTE 7) | [<1] Mbit/s [9] | [> 99.9%] (without compression) [> 99.999%] (with compression (NOTE 10)) [10] | [2.25] Tbit/s/km2 (NOTE 8) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 [9] | 0.25-10 ms [9] |  |  |  | UL and DL haptic feedback |
| Metaverse-based Tele-Operated Driving (NOTE 16) | [100] ms [8] (NOTE 11) | [10~50] Mbit/s [8] | 99% [8] | [~360] Mbit/s/km2 (NOTE 14) | - | 20~100 ms [8] (NOTE 12) | [10] cm [8] | [10-50] km/h (vehicle) [8] Stationary or Pedestrian | Up to 10 km radius [8] (NOTE 13) | UL real-time vehicle data (video streaming and/or sensor data) [8] |
|  | [20] ms [8] | [0.1~0.4] Mbit/s [8] | 99.999% [8] | [~4] Mbit/s/km2 (NOTE 14) | Up to 8Kb [8] | 20 ms [8] (NOTE 12) | [10] cm [8] | [10-50] km/h (vehicle) [8] Stationary or Pedestrian | Up to 10 km radius [8] (NOTE 13) | DL control traffic (commands from the remote driver) [8] |
|  | 1-20 ms (NOTE 15) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) (NOTE 15) | 99.999% (NOTE 15) | [~20] Mbit/s/km2 (NOTE 14) | 2-8 (1 DoF) (NOTE 15) |  |  | Stationary or Pedestrian | Up to 10 km radius [8] (NOTE 13) | Haptic feedback |
| Viewports streaming from rendering device to AR glasses through direct device connection (tethered/relaying case) (NOTE 17) | 10 ms (i.e., UL+DL between AR Glasses display and the rendering UE) (NOTE 18) | [200-2000] Mbit/s | 99.9 % (NOTE 18) | - | - | - | - | Stationary or pedestrian (between rendering device and AR glasses) | Up to direct device connection ranging | Immersive AR interactive experience: tethered link |
| Pose information from AR glasses to rendering device through direct device connection (tethered/relaying case) (NOTE 17) | 5 ms (NOTE 18) | [100-400] Kbit/s (NOTE 18) | 99.9 % (NOTE 18) | - | - | - | - | Stationary or pedestrian (between rendering device and AR glasses) | Up to direct device connection ranging |  |
| Movie streaming from metaverse server to the rendering device (NOTE 20) | Only relevant for live streaming. [1-5] s in case of live streaming | [0.1-50] Mbit/s (i.e., covering a complete OTT ladder from low resolution to 3D-8K) (NOTE 19) | 99.9 % | - | - | - | - | Pedestrians and users in vehicles | - | Immersive AR interactive experience: NG-RAN multimodal communication link |
| Avatar information streaming between remote UEs (end to end) | 10 ms (i.e., 20ms between both UEs excluding metaverse server processing time) (NOTE 22) | [0.1-30] Mbit/s (NOTE 21) | 99.9 % | - | - | - | - | Pedestrians and users in vehicles | - |  |
| Interactive data exchange: voice and text between remote UEs (end to end) (NOTE 22) | 10 ms (i.e., 20ms between both UEs excluding metaverse server processing time) | [0.1-0.5] Mbit/s | 99.9 % | - | - | - | - | Pedestrians and users in vehicles | - |  |
| NOTE 1: The mobile metaverse server receives the data from various sensors, performs data processing, rendering and provide feedback to the vehicles and users. NOTE 2: Based on [8] and 3GPP TS 22.261 [7], typical data volume include 1) camera: 8 Mbit/s per sensor (unstructured), 2) LiDAR: 35 Mbit/s per sensor (unstructured), 3) radar: 160 Kbit/s per sensor (unstructured), and 4) real-time Status information including Telemetry data: [< 50 kbit/s] per sensor/vehicle/VRU (structured). Typical urban density [8] considers 120 vehicles and 220 users/smartphones (120 drivers + 100 VRU) present in an area of 40m*250m to actively enjoy immersive metaverse services for traffic simulation and traffic awareness.This further considers 2 cameras, 2 Radars, 2 LiDARs on road side per direction, which communicate with the same equipment at previous roadside infrastructure node, Estimated vehicle equipments are 7 cameras, 8 radar and 1 LiDAR for each vehicle. NOTE 3: The frequency considers different sensor types such as Radar/LiDAR (10Hz) and camera (10~50Hz). NOTE 4: The service area for traffic flow simulation and situational awareness depends on the actual deployment, for example, it can be deployed for a city or a district within a city or even countrywide. In some cases a local approach (e.g., the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. NOTE 5: The calculation is this table is done per one 5G network, in case of N 5G networks to be involved for such use case in the same area, this value can be divided by N. NOTE 6: User experienced data rate refers to the data rate needed for the vehicle or human, the value is observed from industrial practice. NOTE 7: The network based conference focus is assumed, which receives data from all the participants, performs rendering (image synthesis), and then distributes the results to all participants. As rendering and hardware introduce some delay, the communication delay for haptic feedback is typically less than 5ms. NOTE 8: To support at least 15 users present at the same location (e.g., in an area of 20m*20m) to actively enjoy immersive Metaverse service concurrently, the area traffic capacity is calculated considering per user consuming non-haptic XR media (e.g., for video per stream up to 40000 kbit/s) and concurrently 60 haptic sensors (per haptic sensor generates data up to 1024 kbit/s). NOTE 9: In practice, the service area depends on the actual deployment. In some cases a local approach (e.g., the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. NOTE 10: The arrival interval of compressed haptic data usually follow some statistical distributions, such as generalized Pareto distribution, and Exponential distribution [10]. NOTE 11: The end-to-end latency does not include sensor acquisition or actuator control on the vehicle side, processing, and rendering on the user side (estimated additional 100ms total). Target e2e user experienced max delay depends on reaction time of the remote driver (e.g., at 50km/h, 20ms means 27cm of remote vehicle movement). NOTE 12: UL data transfer interval around 20ms (video) to 100ms (sensor), DL data transfer interval (commands) around 20ms. NOTE 13: The service area for teleoperation depends on the actual deployment; for example, it can be deployed for a warehouse, a factory, a transportation hub (seaport, airport etc.), or even a city district or city. In some cases, a local approach (e.g., the application servers are hosted at the network edge) is preferred to satisfy low latency and high-reliability requirements. NOTE 14: The area traffic capacity is calculated for one 5G network, considering 4 cameras + sensors on each vehicle. Density is estimated to 10 vehicles/km2, each of the vehicles with one user controlling them. [8] NOTE 15: KPI comes from 3GPP TS 22.261 [7] clause 7.11 “remote control robot” use case. NOTE 16: Examples of typical data volume including 1) ~8Mbps video stream. Four cameras per vehicle (one for each side): 4*8=32Mbps. 2) sensor data (interpreted objects), assuming 1 kB/object/100 ms and 50 objects: 4 Mbps [8]. NOTE 17: These KPIs are only valid for cases where the viewport rendering is done in the tethered device and streamed down to the AR glasses. In the case of rendering capable AR glasses, these KPIs are not valid. NOTE 18: These values are aligned with the tactile and multi-modal communication KPI table in TS 22.261 [7], clause 7.11. NOTE 19: These values are aligned with “high-speed train” DL KPI from TS 22.261 [7] clause 7.1 NOTE 20: To leverage existing streaming assets and delivery ecosystem, it is assumed that the legacy streaming data are delivered to the rendering device, which incrusts this in the virtual screen prior to rendering. For a live streaming event, the user-experience end-to-end latency is expected to be competitive with traditional live TV services, typically [1-5] seconds. NOTE 21: For example, the glTF format [13] can be used to deliver avatar representation and animation metadata in a standardized manner. Based on this format, the required bitrate for transmitting such data is highly dependent on avatar’s complexity (e.g., basic model versus photorealistic). NOTE 22: These values are aligned with “immersive multi-modal VR” KPIs in TS 22.261 [7], clause 7.11. |  |  |  |  |  |  |  |  |  |  |

* * * End of Copy * * *

* * * Begin of annex * * *

Annex

This annex contains the original content of the use cases mirror with TR 22.870 v0.4.0 (S1-253370).

Copy from Immersive session:

9.2	Use case on immersive gaming

9.2.1	Description

With the growing adoption of extended reality in the 5G era, it is expected that 5G-Advanced and 6G will scale immersive experiences for the enterprise and consumer mass market. In 6G, users of immersive technologies are expected to work, play, and interact seamlessly in both the physical and virtual worlds. These immersive experiences would be enabled by leveraging advanced extended reality (XR) and multimedia features such as user-interaction via Avatar-Holographic conferencing, spatial collaboration on Hi-fidelity 3D objects, high resolution immersive 2D-3D Cloud Gaming, high resolution 360o 2D-3D content streaming and Multi-Modal AI-intermediated User Experiences.

User-interaction via Avatar or Holographic conferencing:

User-interaction in immersive experiences can be via Avatar or Holographic conferencing. This feature allows participants to engage in virtual meetings where they appear as lifelike avatars or holograms, creating a sense of physical presence despite being miles apart. By leveraging advanced motion capture and real-time rendering, users can express emotions, gestures, and body language, making interactions more natural and engaging. This form of conferencing not only enhances communication and collaboration but also breaks down geographical barriers, enabling a more inclusive and dynamic way of connecting with others.

Spatial collaboration on Hi-fidelity 3D objects:

Spatial collaboration on high-fidelity 3D objects enables multiple participants regardless of their location to work together in immersive environments. The feature enables multiple users to interact with detailed 3D models in a shared virtual space, allowing for real-time modifications and discussions. By providing a highly realistic and interactive representation of objects, users can explore every angle, scale, and detail as if they were physically present. This level of collaboration enriches immersive experiences, and is particularly beneficial in fields such as design, engineering, and architecture, where precision and detail are crucial. It enhances creativity, improves communication, and accelerates decision-making processes, making it an invaluable tool for modern collaborative efforts.

High resolution immersive 2D or 3D Cloud Gaming:

High-resolution immersive 2D or 3D cloud gaming with rich backgrounds is revolutionizing the gaming industry by delivering stunning visual experiences directly from the cloud. This technology allows gamers to access and play graphically intensive games on various devices (e.g. handheld gaming device, high-resolution television or monitor, head mounted displays) without the need for high-end hardware. By leveraging powerful cloud servers, games can render intricate details and complex environments in real-time, providing seamless transitions between 2D and 3D perspectives. The rich backgrounds enhance the immersive experience, making players feel as if they are truly part of the game world. This advancement not only democratizes access to high-quality gaming but also paves the way for more interactive and visually captivating game designs, pushing the boundaries of what is possible in digital entertainment.

High resolution 360o 2D or 3D content streaming:

High-resolution 360° 2D or 3D content streaming is transforming how digital media is experienced, by offering an unparalleled level of immersion and detail. This feature allows users to stream content that can be viewed from any angle, providing a fully immersive experience whether in 2D or 3D. By leveraging advanced compression algorithms, powerful cloud infrastructure, high-resolution 360° streaming ensures smooth playback and high-quality visuals, even on devices with varying capabilities. This innovation is particularly impactful in fields such as virtual tourism, education, and entertainment, where it enables users to explore environments and scenarios as if they were physically present.

Multi-Modal AI-intermediated User Experiences:

Multi-modal AI-intermediated user experiences are redefining how interactions with technology is performed, by integrating various forms of input and output to create seamless, intuitive interactions. This approach leverages AI to interpret and respond to multiple modes of user interaction, such as voice, text, gestures, and even facial expressions and video streams representing user context. By combining these inputs, AI can provide more accurate and context-aware responses, enhancing the overall user experience across all forms of 2D and 3D content consumption discussed above. Users can control actions through a combination of voice commands and gestures, with the AI understanding and executing tasks based on the context, presenting the results via hi-fidelity 2D or 3D assets.

These extended reality and multimedia features and 6G capabilities such as enhanced communication, sensing, compute and AI would enable significant improvement in immersive experiences across many fields including education, healthcare, entertainment, and enterprise collaboration, bringing us closely to vision of seamless merging together of the physical and virtual worlds.

Enhanced Immersive gaming enables single or multiple participants in various location(s) to interact in real time with digital representation of object(s) and their surrounding environments, which are generated with hi-fidelity 3D assets and digital representation of human using avatars and/or holograms. These digital representations of object(s), environment and humans are created leveraging inputs such as visual, audio or tactile features and/or location information generated from sensors (such as cameras, RF sensors and haptic sensors) embedded in the vicinity/environment or on the participants. In addition, the interactions between entities in the immersive gaming experiences can be made more accurate, intuitive and seamless using cloud gaming and multi-modal AI-intermediated techniques.

Sustainability impact analysis:

Economic Growth:

The demand for immersive experiences drives technological innovation in many areas including communication, XR, compute and AI, with applications beyond gaming. Overall, immersive gaming contributes to industry growth, creating jobs and fostering economic activity while providing a rich and multifaceted experience.

Inclusion:

Immersive gaming offers numerous benefits, especially, social benefits such as fostering teamwork, emotional connection and a sense of community and camaraderie, irrespective of the physical distance of separation. The use case discussed below aims by no means to exclude people but instead, the goal is to include as many participants in this gaming experience as possible irrespective of their separation distance. In the case, all the participants (i.e. players, cheerleaders and spectators) are localized, further consideration is required on whether this technology is needed. Also, significant consideration needs to be made to ensure that the technologies involved simulate seamless immersive gaming as much as possible to ensure that the experience of the participants is not negatively impacted or burdensome.

9.2.2	Pre-conditions

The University of Nee's basketball team is playing the last game of the tournament against a visiting team from the University of Zee. This is a home game for the University of Nee so its players would be participating in person at the Nee city basketball gymnasium. However, the team from the University of Zee are flying in from another country, but due to bad weather, two players from the University of Zee, the team's MVP, David and another player Steve flights were delayed so they would have to join the game virtually from location A.

As a result, the basketball players involved in this game are in two different locations, Nee city basketball gymnasium and location A. The immersive services at these locations are provided by Network Operators NeCom and ZeCom, respectively. Each location is outfitted with equipment (e.g. camera, sensors and edge computing resources) to support the immersive experience. Each player wears immersive gears including HMD/AR glasses and motion capturing devices such as haptics gloves, smart watches and compute puck/phones.

There are also dance routines performed by twelve cheerleaders physically present at the Nee city basketball gymnasium and three virtual cheerleaders at location B with the immersive experience in that location provided by Network Operator TeCom. Each cheerleader would also wear HMD/AR glasses and haptics gloves.

Also viewing the game are spectators (approximately one hundred people), some of whom are in-person at the Nee city basketball gymnasium and many more watching remotely. Leveraging high resolution 360o 2D/3D content streaming techniques, the in-person spectators view the game using HMD/AR glasses while the remote spectators can watch the game using the HMD, tablet, or even their phones.

9.2.3	Service Flows

1. When the game starts, all the immersive participants at the Nee City basketball gymnasium can view their surroundings, including in-person players, the cheerleaders and spectators through their immersive devices. In addition, virtual players and cheerleaders represented as avatars/holograms with accompanying movements, gestures and even facial expressions are rendered into their appropriate display devices. From the perspective of the virtual players and cheerleaders, they see themselves as participating in activities (i.e. playing basketball or dancing) in the Nee City basketball gymnasium and with background features showing highly realistic 3D digital representation of the objects (such as the court and baskets), players, cheerleaders and spectators that are present at the location.

2. For a truly immersive experience involving between local and virtual players, the ball is virtually created and interacted with by both in-person and remote players using gloves outfitted with haptic sensors. The movements and impacts on the ball from the players are communicated to all viewers in "real-time". During halftime, dance routines are performed by cheerleaders twelve of whom are local, while the remaining three are participating from location B. The movements, locations and gestures of the cheerleaders are combined virtually and displayed to the all the viewers.

3. In addition, the spectators can customize their viewing perspectives, such as the desirable viewing angle, location or resolution. Spectators interact with their immersive applications using voice commands and gestures, with the AI understanding and executing tasks based on the context, presenting the results via hi-fidelity 2D or 3D assets.

4. The immersive experience of players, cheerleaders and spectators are enabled by cellular traffic flows that can be classified into the following categories:

a)	Compute flows: Traffic flows that are related to immersion and AI related compute that might be split between the compute processing available across cellular connections.

b)	Conversational and game state flows: Traffic flows that are related to audio, video, avatar/hologram-based interaction between users. Also, traffic flows that are related to information representing the global immersion state across all users and objects.

c)	Streaming flows: Traffic flows that are related to slower time scale streaming of information to certain users, e.g. spectators.

9.2.4	Post-conditions

With the advancements in immersive gaming technologies and 6G capabilities, local and remote players, cheerleaders and spectators can seamlessly participate and watch an immersive basketball game.

9.2.5	Existing features partly or fully covering the use case functionality

Existing immersive features from the 3GPP SA1 Mobile Metaverse Services Study and Work document in TS 22.856 [49] and TS 22.156 [28] can be used to enable some of the features in this use case.

But for XR display devices, pixel density below 60 PPD (Pixels Per Degree) triggers noticeable "Screen Door Effect", users can perceive individual display pixels, creating a mesh-like visual obstruction akin to viewing through a screen door [157]. This optical distortion phenomenon severely degrades immersive gaming experiences. For 2D/3D video, to provide users with an ultimate experience, a horizontal field of view (FOV) of 210 degrees and a vertical FOV of 100 degrees are required [158].

With the rapid development of video coding, both the video compression ratio and pixel quantization methods will become more efficient. For example, H.265 supports a video compression ratio of 400:1. Compared with the individual quantization of different color components in the RGB format, the YUV 4:1:1 chroma subsampling format can be adopted for pixel quantization, where only 12 bits are required to quantize one pixel [160].

In addition, many current display devices support a refresh rate of 90 frames per second (fps) with 8K resolution. Therefore, a refresh rate of 90 fps also needs to be considered as a typical value.

9.2.6	Potential New Requirements needed to support the use case

Table 9.2.6-1: Key Performance Indicator (KPI) for Immersive Gaming

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max allowed end-to-end latency | Service bit rate: user-experienced data rate (note 6) | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed | Service Area |  |
| Compute flows: [5 ms - 20 ms] Conversational and game state flows: [50 - 100 ms] Streaming flows: [200 - 300 ms] | Player/Cheerleader DL: (Note 1) [640 Mbps] (8K, 120fps, compression ratio of 300 and 8 bits per color) [240 Mbps] (8K, 120fps, compression ratio of 400 and 12 bits per pixel) [200 Mbps] (8K, 90fps, compression ratio of 400 and 12 bits per pixel) UL: [100 Mbps] (note 2) Spectator: DL: (note 3) [320 Mbps] (2D 8K, 120fps, compression ratio of 300 and 8 bits per color) [120 Mbps] (2D 8K, 120fps, compression ratio of 400 and 12 bits per pixel) [100 Mbps] (2D 8K, 90fps, compression ratio of 400 and 12 bits per pixel) UL: [10 Mbps] (note 4) | [99.9 – 99.99 %] | N/A | N/A | N/A | [≤10 cm] | Stationary, pedestrian | [38 m x 15 m] (note 5) |  |
| NOTE 1: It is important to note that the data rates may change under different assumptions. Data Rate = Video resolution * (Bits per color * 3) * Refresh rate * # of eye buffer / (compression ratio), with the following assumptions: 2 eye buffers; 8K video resolution; refresh rate of 120 fps or 90 fps. The frame rate of 120 fps is assumed as it has been shown that such high frame rate help reduce the probability of simulator sickness [15]. 90 fps is the typical frame rate in current display device. compression ratio of 300 and 8 bits per color; or compression ratio of 400 and 12 bits per pixel [160]. A resolution of 8K (8192 x 4320) per eye can help to remove graphics pixelation and provide good XR user experience [159]. High refresh rates (e.g. 120 fps) are very correlated and inter prediction between frames increase compression. Some codecs (e.g. MV-HEVC) may further drop the bitrate requirement. NOTE 2: Significantly higher compared to 5G to enable sensor sharing for split computation. NOTE 3: DL data rate for spectator is assumed to be 2D 8K video NOTE 4: UL tracking for spectator not as intensive as the player/cheerleader NOTE 5: Average basketball court is 28 m x 15 m (adding 5 m on both sides for the spectator seating) to make a 38 m x 15 m basketball gymnasium. NOTE 6: The provided values are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |

9.5	Use case on seamless immersive reality in education

9.5.1	Description

Immersive reality combines immersive telepresence and immersive collaboration. Immersive telepresence allows remote participants to appear as physically present in the very same environment as co-located participants. The remote participants perceive the environment and other participants as if they were there in person, thanks to stimulation of their visual, aural, and haptic senses. Immersive collaboration provides a new way of interaction with other people and objects.

Immersive reality technology opens up new opportunities in the education sector, where students are able to collaborate, discuss, and learn in an immersive setting with remote and co-located participants. Participants interact with each other in a more natural way in an immersive reality environment, and participants are also able to interact more naturally with virtual or digital representations of real objects.

The immersive classroom may be local (where all students are physically co-located and learn with virtual objects), hybrid (with both physically co-located as well as remote participants), or fully immersive (where both students and instructors are virtually present).

Figure 9.5.1-1: Hybrid Immersive Classroom Scenario

For example, consider a hybrid immersive chemistry classroom, exploring and assembling 3D models of complex molecules, shown in Figure 9.5.1-1. The "orange-handed" student and the teacher represent participants remote to the physical classroom while the "red-handed" and the "blue-handed" students represent participants physically present in the classroom. The equipment local to the physical classroom, e.g. cameras, and wearables worn by the students, senses the real environment, communicates the actions and the outcome of the collaborative molecule assembling, and communicates to establish immersive co-presence of the remote and physically present participants.

Further justification and background for the KPIs for this use case is found in the following table [31]:

Table 9.5.1-1: Justification and clarification of KPIs

| KPI | Target Range | Justification |
| --- | --- | --- |
| User-experienced data rate [Mb/s] | < 250 | For DL, but also for UL |
| Area traffic capacity [Mb/s/m2] | < 250 < 20 | Indoor: per floor in a multi-story building Wide area: for immersive experience "on the go" |
| Mobility | seamless HO | Pedestrian for participants in the classroom |
| End-to-end latency [ms] | < 10 < 50 < 150 | Split rendering Voice Collaboration |
| Reliability [%] | 99.9 – 99.999 | Depending on data stream |
| Positioning accuracy [cm] | ≤ 10 horizontal ≤ 10 vertical | Positioning of AR glasses and sensors. Network-assisted positioning can be used for e.g. sensors, but high positioning accuracy for AR typically requires device-based sensors and sensor fusion. |

Sustainability impact analysis:

Material resources: Increased electronic waste from the disposal of devices and network equipment. Increased material consumption from producing the hardware components and expanding network infrastructure including raw material extraction, manufacturing processes, and transportation

Emissions: Reduction of emissions by a reduction in physical travel for work or education.

Inclusion and Equality: Enhances educational opportunities of the population regardless of their location, and improves the efficacy of the education process, both for the students and for the teachers. However, there is a potential digital inequality depending on connectivity access, information technologies (IT) literacy and economic status.

Trustworthiness: Preserved/uncompromised privacy is key for the enablement of this service. However, there are still potential risks for privacy intrusion associated to localisation and positioning data.

9.5.2	Pre-conditions

The students in the physical classroom are each using an AR device and have joined the classroom application.

Both AR devices are very simple and light so that the students can use them for long periods of time, which means that the AR devices have limited battery and limited computing capabilities.

The physical classroom has a local server with available computing resources.

9.5.3	Service Flows

1. The teacher, who is located remotely, starts the classroom lesson and pulls up a 3D model of a complex molecule. The AR devices of the students in the physical classroom uses the local server to perform the necessary computation tasks to enable their AR devices to display the same 3D model of the complex molecule (e.g. shadows and lighting aspects of the molecule).

2. The teacher asks the students in the classroom and the remote student to work together to disassemble the molecule and use the components to construct a new one.

3. A combination of the information from the wearables of the students and the cameras present in the physical classroom sense the environment, e.g. the students' hand gestures and movements.

4. The AR devices of the students in the physical classroom uses the local server to perform the necessary computation tasks (e.g. via an AI/ML model) to anticipate their hand gesture and movements and translate them into specific actions on the 3D molecule model.

9.5.4	Post-conditions

None.

9.5.5	Existing features partly or fully covering the use case functionality

-	Sensing: immersive experience requires the human sensory system to receive realistic stimuli from a mixed or virtual reality. Some scenarios may use ISAC or may apply sensor fusion of network and sensor data of connected sensors. Sensing has been covered in Stage-1 in TS 22.137 [6].

-	Positioning: this use case requires accurate positioning for a seamless immersive experience. Positioning requirements are defined in TS 22.261 [14]. However, the KPIs from this use case need to be enhanced with greater accuracy compared with the 5G KPIs.

-	Media synchronisation: low E2E latency, in combination with the synchronisation of different media for each participant, as well as synchronisation of inter-participant media are needed to ensure a coherent and realistic user experience. This has been covered by TS 22.156 [28] (e.g. [R-5.1.1-002] and [R-5.1.1-003]).

-	Digital immersive mapping as a service: to assist seamless immersive reality, digital immersive mapping is provided as a service by the network. This has been covered by TS 22.156 [28] (i.e. clause 5.2.1. Localized mobile metaverse service).

-	Service continuity: service at a minimum level to provide a sufficient and for the end user comprehensible and satisfactory QoE across diverse locations ranging from local wireless networks to the wide area network. This has been covered by TS 22.156 [28].

-	Distributed Federated Learning: distributed federated learning involving multiple UEs is discussed in TS 22.261 [14]. What is not discussed in federated inference, where an AM/ML model includes information from multiple cameras/wearables/sensors for inference.

9.5.6	Potential New Requirements needed to support the use case

[PR 9.5.6-1]	Subject to operator policy, the 6G network (e.g. core network) shall enable the support of computing tasks in the Service Hosting Environment to render 6DoF video and spatial audio.

[PR 9.5.6-2]	Subject to operator policy, the 6G network (e.g. core network) shall enable the offloading of computing tasks to the Service Hosting Environment to render 6DoF video and spatial audio to a 3rd party.

[PR 9.5.6-3]	Subject to operator policy, the 6G network (e.g. core network) shall support hosting of e.g. an AI/ML model in the Service Hosting Environment based on latency, transport load or data privacy requirements.

[PR 9.5.6-4]	Subject to operator policy and privacy considerations, the 6G system shall support means to coordinate data flows from UEs (e.g. cameras/wearables/sensors), associated with a specific place (e.g. a classroom, meeting room, office), to enable e.g. an AI/ML model to improve the user's perception of and interaction with an immersive scene, e.g. to more accurately predict scene changes or object movements within that specific place.

Editor's Note: terminology and definition of 'place' is FFS

Editor's Note: clarifications on privacy considerations are FFS

Table 9.5.6-1: KPIs for Seamless Immersive Reality in Education

| Scenario | User-experienced data rate [Mb/s] | Area traffic capacity [Mb/s/m2] |  | End-to-end latency [ms] |  |  | Positioning accuracy [cm] |  | Mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Indoor | Outdoor (Wide Area) | Split rendering | Voice | Collaboration | Horizontal | Vertical |  |
| Seamless Immersive Reality in Education | [< 250] | [< 250] | [< 20] | [< 10] | [< 50] | [< 150] | [≤ 10] | [≤ 10] | Pedestrian |

9.6	Use case on collaborative service in multi-site involved immersive communication

9.6.1	Description

Compared to AR/VR services under existing 5G networks, 6G immersive XR services aim to provide an ultimate user experience through capabilities such as ultra-high resolution, high frame rate, wide color gamut, high dynamic range, wide field of view, and advanced encoding/compression techniques. It supports more natural interaction methods (e.g. voice interaction, gesture interaction, head interaction, eye tracking) to achieve complex perception of users and environments. The 6G system and service platform need to collaborate to complete functions such as rendering, synchronization, encoding, distribution, storage and management of immersive XR services, and support real-time transmission and processing of panoramic video data, field of view video data, etc.

6G immersive XR services present new challenges for platforms, networks and terminals. Based on 3GPP R17 XR project research, current XR service data rates are 30/45Mbps downlink and 10 Mbps uplink for AR scenarios; future requirements with increased video resolution and frame rate can reach Gbps levels. While data volumes increase, immersive XR services have data characteristics of variable packet size, non-integer period, jitter and multiple streams. Concurrent traditional data flows (signaling flows, audio streams, video streams) also exist in the network. In addition to stronger computing capabilities, ensuring real-time transmission of relevant data within the 6G system between various processing nodes is also a key aspect. The timeliness of data transmission will impact the success rate and quality of service, so 6G networks must ensure efficient transmission of large volumes of data to maintain service level agreements

This case demonstrates the 6G network’s capability to provide computing service for enabling a multi-site involved immersive communication scenario.

9.6.2	Pre-conditions

- Alice, Bob and Lisa, Tom are customers of Network Operator A, a 6G service provider.

- On New Year's Eve, a public event will jointly held across two cities - City A and City B. Participants had the option to attend in-person at the event venues for massive gatherings, or experience an immersive viewing through different presentation methods (e.g. outdoor large screens, mobile phones/pads, VR/AR glasses)  to feel as if present.

- Alice attended on-site at the City A venue. Bob attended on-site at the City B venue while Tom watched the event in the bus from City B to City A. Lisa experienced the event remotely from her home in City C through a wireless VR device.

- To meet the needs of multi-dimensional perspectives and long-distance follow-up shooting, the event organizer chooses wireless cameras to capture video streams. For saving the time of encoding at camera side and keeping as much as details of video stream, low-compression and even uncompressing ([161], [64], [67]) are selected for cameras on-site. There are also multiple sensors on site collecting signals. Network Operator A’s network provides wireless transmission for video streaming and sensors data.

- Besides providing communication service to enable this use case, Network Operator A's 6G network also provides computing service to the organizer’s platform for stream processing to fulfill demands on immersive presentation. Rendering, encoder/decoder, and other capabilities are also pre-deployed in the network on the event venues in City A and City B.

9.6.3	Service Flows

Sub-scenario 1 (Immersive Program Viewing):

For example, two singers perform the same song on stages located in City A and City B respectively. Through the immersive viewing service, Alice, Bob and Lisa, Tom - attending from different physical locations using different means - perceive the experience as if the two singers were performing on a single stage together.

The following Figure 9.6.3-1 illustrates stream flows between City A and City B venues during the live interaction show.

NOTE:	Not all entities and control message are shown in above figure for the sake of simplicity.

Figure 9.6.3-1: Stream flows between two venues during the live interaction show

- 1, 1`: Local performances are presented to on-site audiences in City A and City B event venues. Meanwhile, multi-dimensional data (such as audio, video, stage view, audience seating view, 3D models, motion, etc.) from the both venues is being acquired. And sending to on-site core network.

- 2, 2`: The core network selects corresponding processing nodes (e.g. core network nodes with computing resources, edge server) to process data (such as data cleaning, compression, encoding, decoding or rendering) and routing data traffics between processing nodes based on the work load of the nodes and computing service requirement received from organizer’s platform.

- 3, 4: The rendered data from City B is being transmitted to City A through the 6G network and is holographically presented to the on-site audience.

- 3, 4`: In the opposite direction, the rendered data from City A is being transmitted to City B through the 6G network and is holographically presented to the on-site audience.

- The 6G network also needs to collaborate with remote audience service VR application servers to render and fuse the data processed from Cities A and B into a global stage scene data and then distribute it to wireless VR devices located in different locations for remote audiences.

- All these data are used to reconstruct through immersive services and presented via Lisa’s wireless VR device.

- During Tom’s bus moves from City B to City A, Tom can keep receiving the render data from 6G network with the requested quality of immersive presentation on VR devices, and enjoy the live show as he is in the event venue.

Sub-scenario 2 (regular Communication Services Remain Unaffected):

Throughout the duration of the event, audiences' regular communication services such as making phone calls, sending text messages, and IMS multimedia services are unaffected.

- Alice shares her experience at the event venue with friends through a video call, and the video interaction remains smooth without issues like audio-video desynchronization or stuttering.

- Bob successfully receives congratulatory New Year greeting messages sent by his friends from all the country, without delays.

- A phone call from Lisa's friend reaches her successfully, and the voice calling service quality does not degrade.

During this event, there are substantial concurrent up-streams and down-streams of multiple data types (user interaction sensing data, AI algorithm data, AR/VR interaction data) which poses challenges for 6G networks. 6G networks must not only function as a data conduit but also enable required real-time data processing operations to ensure user experience.

9.6.4	Post-conditions

- Alice, Bob, Tom and Lisa have an equivalent viewing and interactive experience during the event.

- Alice, Bob, Tom and Lisa's regular communication services are unaffected during their participation in the event.

9.6.5	Existing features partly or fully covering the use case functionality

Currently, communication requirements as described in TS 22.261 [14] clauses 7.1, 7.6.1 and 7.10 are provided. And it is assumed that there are always sufficient computing resources at the selected one node for running rendering server. 5G network lack mechanisms for intermediary data for collaborative tasks in large-scale rendering.

An "Audio and Video Production" use case which uses HD and UHD resolutions deployed over 5G system is introduced in TS 22.104 [64]. Some normative requirements related to support of media service are in TS 22.263 [67].

9.6.6	Potential New Requirements needed to support the use case

[PR 9.6.6-1] Subject to operator’s policy and regulation, the 6G network shall be able to provide 6G Computing Service controlled by the core network of 6G system to an authorized third-party for supporting rendering.

[PR 9.6.6-2] Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment used for the 6G Computing Service for XR rendering e.g. during UE mobility.

[PR 9.6.6-3]: 6G system shall be able to support immersive media content production with the KPI requirements summarized below in Table 9.6.6-1.

Table 9.6.6-1: Performance requirements for immersive media content production via wireless link

| Use Cases | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate (UL) | Service bit rate: user-experienced data rate (DL) | Reliability | # of UEs | UE Speed | Service Area |
| Immersive media content production via the wireless link | [100ms] (note 3) | (Note 1) [2.65 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 4) [1.1 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 10) [7.96 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 6) [4.8 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 10) | N/A | [99,99 %] | 4 (Note 2) | Stationary, Pedestrian | 30 m x 30 m |
| NOTE 1: This bit rate is assuming to use the JPEG-XS [352] pre-compression encoding method to ensure both high-quality images and high encoding efficiency, while also realizing lower encoding delays [161]. Which is intended as a targeted value per UE and not a strict requirement. NOTE 2: 4 cameras are used in simple stage [353], more cameras may be needed for condition of larger number of performers, with complex stage and lighting conditions. It depends on the deployment. NOTE 3: One-way delay that is from camera to holographic player. |  |  |  |  |  |  |  |

## 9.8	Use case on holographic telepresence in healthcare

### 9.8.1	Description

Holographic telepresence will enable users located in multiple remote locations to interact with each other in real-time as if they were together in a single shared space– for example, one projected as holographic presence appearing in the office to present the document in front of other colleagues while actually being in the car. The 6G system is expected to facilitate such seamless, real-time immersive interaction, allowing participants to engage in activities ranging from virtual meetings to collaborative work and remote medical consultations. Utilising immersive media such as hologram can enhance communication efficiency and user experience, bridging geographical distances and creating a sense of co-presence among users.

Although audio and video calls can realise interactions between the participants in different locations, those interactions are still very different from those which can be achieved when all participants are in the same physical location. The reasons of this difference can be the limitation of capturing devices such as cameras and microphones to capture the atmosphere of one location as well as the limitations of displays to render that atmosphere in another location. With the real-time telepresence based on holography and avatar technology, the participants in multiple locations are possible to enjoy truly immersive experience as if all of them would interact in the same physical location. However, to realize holographic communication, there are multiple challenges that 6G network need to address.

Bandwidth, latency and Computing of Holographic communication

AR/VR has been identified as one of the important technologies to realise immersive communications in 5G. The immersive experience can be further enhanced with more immersive media including hologram, avatar and multi-sensorial media. Hologram displays require a large volume of data about the user’s motion and sensory inputs captured by cameras, sensors and actuators as well as volumetric data from multiple viewpoints to accommodate 6DOF movements of the observer [104]. Thus, the synchronized multi-modal data streams for holograms may require extremely high bandwidth, potentially network performance in the range of 100 Gbps or beyond  [105], [106]. To reduce the challenge to the transmission bandwidth, various compression and decompression technologies are developed such as point-cloud techniques and even powered by AI. Also holographic avatars or computer-generated holograms to replace photo-realistic hologram are considered and may reduce the demand on transmission rate to 1Gbps or lower. However, the additional processing latency may consume the delay budget reserved for transmission while the increased computation workload must be optimized or offloaded to support portable display devices that the users can access ubiquitously.

Security of holographic communication

Even with such ultra-high data rate communications, confidentiality must be guaranteed as it is today. Widely used encryption algorithms can only support data rates up to 38.70 Gbps on mobile environments [107], [108]. Existing algorithms cannot cope with the required data rates for communications utilizing immersive media so that shall be enhanced in 6G systems. The 6G system shall support encryption and integrity protection algorithms that can handle data rates in the 100 Gbps range. As the computational performance of the chipset on UE improves over the years, and the exploration of distributed computing technologies, the possibilities of the existing algorithms achieving the required performance also improves. Considering the amount of data which needs to be encrypted and integrity protected, having a faster algorithm will improve overall efficiency.

On the other hand, the captured and transmitted data for the holograms contains a significant amount of personal biometric information such as facial features, voiceprints, which need proper privacy protection to ensure the compliance to regulatory requirements.

Sustainability impact analysis:

Economic Growth:  Holographic communication cannot be achieved without technological advancements in many areas including capturing devices, displays and communications. Once holographic communication is achieved, we can expect a new type of collaborations in many industries, which can lead to further economic growth.

Emissions: Reduction of emissions will be expected by reducing physical travels.

Inclusion: Holographic communication can realise closer interaction among parties more frequently than ever without traveling. It also brings an opportunity for those who are unable to travel to explorer a remote location as if they are actually in that location.

### 9.8.2	Pre-conditions

A patient Alice is in the house, while the practitioner Bob is at the clinic A and the practitioner Tom is at the hospital B.

The house, clinic and hospital are equipped with dedicated sensory system (including multiple cameras, microphones and other sensors), holographic display device and a mobile communication device, while the practitioners additionally have XR devices to capture the pose and gesture, and haptic information.

The operator managed edge/cloud-based application servers can execute avatar generation, hologram computation and rendering.

The mobile network and communication devices supports 6G.

### 9.8.3	Service Flows

Scenario#1 One-to-One diagnostic

- Alice and Bob start the holographic communication application on the mobile communication devices.

- They enter a session ID to establish a connection between two locations.

- They connect their displays to mobile devices to start receiving/sending the holographic data.

- The sensory system in Alice’s house captures her image and status and then send them to the cloud-based application server via the mobile device for processing and rendering.

- The application server generates Alice’s hologram data based on the received multiple data streams with individual modal media such as audio, video, haptic information, etc. and sends it to the mobile device in the clinic.

- Upon receiving the hologram data, the display can project Alice’s 3D image in the clinic.

- Bob with XR device can examine Alice’s status as if Alice is in the clinic.

- Similarly, Bob’s hologram data including associated gesture and pose is sent to the communication device in Alice’s home via the application server and displayed in naked eyes.

- Alice can receive feedback from Bob as if Bob is in the house.

- All data transmitted over wireless links is encrypted as well as integrity protected to ensure the security of the communication.

Scenario#2 Joint diagnostic(avatar-based)

- Alice, Bob and Tom setup a holographic conference via the mobile communication device. For the purpose of privacy, Tom chooses to use holographic avatar which is generated in the network.

- When Tom makes an examination and talks, the multi-modal data streams captured in the hospital and his holographic avatar data are transmitted to the selected edge servers of Alice and Bob for rendering, which may include the modal data of Tom’s pose, gesture, motion, or facial expression, the conversational voice, and the interactive information. As the examination goes on, the ty of modal data may change accordingly.

- Alice’s and Bob’s mobile devices send captured hologram related data to the edge servers chosen for other participants respectively.

- Tom’s animated holographic avatar is simultaneously generated in Alice’s and Bob’s edge server and then is converged with hologram of Bob or Alice individually to render the whole scenario.

- The rendered virtual conference is transmitted to Alice’s and Bob’s mobile device for display.

### 9.8.4	Post-conditions

Alice and Bob can engage in real-time, bidirectional holographic communication as if they are in the same room, despite the geographical distance. This will be beneficial especially when they need close interaction during the consultation, for example rehabilitation.

Bob and Tom can make the join diagnostics for Alice’s health as if they are in the same room. And Tom’s avatar can reflect his gesture, motion and facial expression correctly and synchronized.

### 9.8.5	Existing features partly or fully covering the use case functionality

The service requirements for avatar-based real-time communication have been specified in clause 5.2.2 of TS 22.156 [28]. The high data rate requirements for 5G use cases are specified in clause 7.1 of TS 22.261 [14]. However, these are not requirements for 6G systems and are insufficient for realising 6G immersive media communication that utilises realistic 3D image.

### 9.8.6	Potential new requirements needed to support the use case

[PR 9.8.6-1] The 6G system shall support holographic communication with the following KPIs.

Table 9.8.6-1: Performance Requirements for Holographic Telepresence

| Use Case | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |
| Holographic telepresence in Healthcare | Presence: [<100ms] Movement: [<20ms] (note 1） | 4K video resolution: [~ 400Mbps] (30fps, 8 parallax, 15 bit/pixel, H.265/H.266 with compression ratio 100) 8K video resolution: [~ 900Mbps] (30fps,8 parallax, RGB, compression ratio 200) Hologram/Point cloud [500Mbps-1Gbps] (30fps,8 parallax, RGB, AI-based) (note 2) | [99.9%-99.999%] | 1 | Stationary, Pedestrian | [5 m x 5 m] (note 3) |
| NOTE 1: For real-time presence of hologram, refer to [169]; for more interaction, the motion-to-photon delay will consider the effect of cyber sickness [170] NOTE 2: For volumetric-based holography, the bandwidth is impacted by the effective pixel count, which is related to the resolution, colour quality and bit-depth [106] the data rate is calculated based on [105] for uncompressed raw data, and optimized by different compression algorithms with different compression rate stated in [171]. With the help of AI technologies such as Neural Holographic Video Compression (NHVC), the bandwidth for transmitting hologram will be optimized a lot [173] NOTE 3: The size of a small room/office. |  |  |  |  |  |  |

Editor’s Note: The KPI value for holographic communication is FFS.

9.9	Use case on mixed reality gaming

9.9.1	Description

Immersive communication use cases will continue to drive requirements on mobile networks. The need for slim light-weight Mixed Reality (MR) / Augmented Reality (AR) glasses has been emphasized announcement of device manufacturers. Multiplayer MR gaming is considered a target use case for 2030 timeframe.

We here consider a multiplayer game with users having a shared spatial map of the environment. The virtual scene is updated and placed in the same way and location for all players simultaneously. A 6G network should provide differentiated connectivity and beyond-connectivity services with consistency in performance.

For a shared gaming experience, the technical requirements are determined by the upstream of the environment from different players, the downstream of virtual objects, and a game logic that defines the connection between reality and virtual objects for multiple users.

The following gaming scenario is considered:

Users A and B are simultaneously acquiring and upstreaming a spatial map of the local environment to a cloud server. In the cloud, a shared virtual scene is generated and transmitted to Users A and B. The users are moving outdoors and indoors in a Dense Urban, Urban and Rural scenario.

Sustainability impact analysis:

Energy resources: The increase usage of energy connected to the new MR equipment should be considered by focusing on energy performance and consumption.

Material resources: The material resources connected to new MR equipment should be considered by focusing on total material use.

Emissions: Emissions to air, water and soil connected to the entire lifecycle of new MR equipment can be addressed with the aim to be minimized but not avoided entirely.

Education: MR gaming can be used in education such as teaching languages.

Health: On the positive side, MR gaming likely increases physical activities compared to other computer gaming. Furthermore, MR gaming can be fun and lead to well-being. On the negative side is the risk of addiction to gaming. Specific scenarios, e.g. MR gaming used in healthcare like might also be used for reintegration of difficult cases in society (e.g. post-traumatic stress syndrome).

Inclusion & equality: There is a possibility for improvement of the social / (gender) equality by e.g. helping chronically ill people to live a better life and thereby participate in society. From a inclusion point of view, affordability is important aspect to consider as those without a headset otherwise are excluded.

Trustworthiness: Privacy and integrity issues related to the is risk that sensitive data is collected; that personal data get into the wrong hands; or intrusion of bystander privacy.

9.9.2	Pre-conditions

The following pre-conditions are considered:

- Two or more users are co-located and collaborating on a game in a mixed digital and physical environment.

- The users are equipped with light-weight AR/MR devices and connected to a 6G network.

- Low latency sensor hardware is required on the devices for per-pixel accurate alignment.

9.9.3	Service Flows

The following service flow is considered:

- Immersive video (RGB and depth) and audio feed is captured on user device.

- The stream is partially compressed on device and offloaded over a 6G network to the cloud. The tradeoff depends on the level of details, latency, and accuracy.

- In the cloud, a shared virtual scene is generated and pre-rendered.

- The virtual scene potentially including coordinates, audio, video, and depth, is transmitted to user device.

9.9.4	Post-conditions

Users have an enjoyable gaming experience.

9.9.5	Existing features partly or fully covering the use case functionality

Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, is already a well-known concept.

Media Codecs, such as versatile video coding, are commonly available and considered for the compression of uplink and downlink images.

In TS 22.156 [28] Mobile Metaverse services requirement for association and coordination for objects related to a session.

Table 7.1.1 in TS 22.261 [14] has requirements for interactive audio and video service defined for indoor/hotspot scenario in note 3 “For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4ms while the corresponding experienced data rate needs to be up to 8K 3D video [300 Mbit/s] in uplink and downlink”.

9.9.6	Potential New Requirements needed to support the use case

The following requirements needs to be fulfilled for the Mixed Reality gaming scenario:

[PR 9.9.6-1] The 6G system shall support service continuity for mixed reality gaming between indoor and outdoor, and between Dense Urban and Rural deployments.

[PR 9.9.6-2] The KPIs in Table 9.9.6-1 should be supported in Dense Urban, Urban and Rural indoor/outdoor scenario:

Table 9.9.6-1: Performance of Mixed Reality Gaming

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application latency | Communication service availability | Overall user density | Activity factor | UE speed |
| Mixed Reality gaming | [50] Mbit/s (note 1) | [50-100] Mbit/s (note 3) | [20] ms (note 2) | Dense Urban [99]% Urban [99]% Rural [98]% | Dense Urban [25,000] /km2 Urban [1000-10,000]/km2 Rural [100]/km2 | 2% (note 4) | 5 km/h |
| NOTE 1: Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, NOTE 2: The local encoding and processing on the glasses is assumed NOTE 3: envisioned 1080P resolution 2 grayscale cameras 1920 x 1080 x 8 bit, a depth sensor 1920 x 1080 x 8 bit with 60 FPS means 1920 * 1080 * 24 * 60 bps = 3Gbps uncompressed. Depending on compression ratios that would lead to 50 Mbps to 100 Mbps NOTE 4: Activity factor 2% is based on 20% uptake of XR service and 10% activity factor for the XR application. |  |  |  |  |  |  |  |

## 9.12	Use case on personalized interactive immersive guided tour

### 9.12.1	Description

Localized (i.e. location-aware) mobile Mixed Reality (MR) services provide users with information and services that are of local relevance. This use case focuses on a personalized interactive immersive guided tour service of a city (Figure 9.12.1-1) including outdoor/indoor configurations. The following technical aspects are supported:

Personalization:

Each visitor of the group can select one or multiple 6G connected device(s) he’s willing to wear during the guided tour based on his desired degree of immersion. He may also add, remove or change a 6G device during the guided tour. Therefore, heterogeneous 6G devices such as lightweight Augmented Reality (AR) glasses, tablets/phones with potential tethered devices, eXtended Reality (XR) video or optical see-through headsets, haptic gloves and/or wristbands, wearables, and immersive audio headsets are connected for a group of visitors during the same period.

Each visitor of the group can select a different personalized setting such as avatar or language (and/or personalize a virtual representation) of the remote touristic guide application, possibly with AI techniques for an intuitive and fast avatar and translation personalization.

The touristic information content is adapted simultaneously to the preferences of each visitor of the group (e.g. adapted overlaid text, picture, 3D virtual asset, video, audio).

Interactive immersion:

Each visitor of the group can choose a personalized degree of immersion based on the 6G device(s) he’s wearing. The rendering of different media (e.g. haptics, video, audio) needs to be spatially and timely consistent along the guided tour to maintain a good level of immersion.

An AR Spatial Computing Service is required to ensure a good level of immersion by integrating virtual content into the visitor’s real environment in a seamless manner (e.g. by managing occlusion, maintaining consistent lighting). AI techniques for real-time environment analysis are necessary, especially in fast-evolving environments (e.g. crowding). Such techniques may be used for the proper placement of virtual content by ensuring that each group member has a good point of view of the virtual content.

One major requirement for the virtual content placement is the safety of the group of visitors.

The group of visitors can interact in real-time with the remote touristic guide represented by a personalized avatar for each visitor. Interactions may be conversational (i.e. asking/answering a question), gesture-based (e.g. hand shaking with a haptic glove).

AI-based services may be used to analyze the facial expression/gaze of the visitors to trigger content adaptations (e.g. when some visitors are looking to particular objects, additional touristic information related to this object is displayed).

Configurated and maintained Quality-of-Experience (QoE):

Data flow metrics supporting the QoE for each visitor of the group need to be configured and maintained along the guided tour which may be composed of outdoor and indoor configurations, sparse and dense, crowded areas potentially leading to various fluctuations of 6G access type connectivity and connectivity conditions (e.g. capacity/congestion).

XR data flow metrics supporting QoE (e.g. XR pose-to-render-to-photon, roundtrip interaction delay) need to be measured and monitored periodically and the XR content and/or the network parameters need to adapt accordingly to changes in 6G connectivity conditions.

Figure 9.12.1-1: Interactive immersive guided tour

### 9.12.2	Pre-conditions

A family including adults, a teen and young child are visiting Paris. Hank is the remote activity guide and teacher for the group. Each member of the family has configurated a dedicated avatar of Hank for the guide tour.

They all wear different 6G connected devices. The mother wears a 6G smartphone and sometimes lightweight tethered AR glasses, the father wears an immersive audio headset, the children wear XR headsets and sometimes haptic gloves.

The family decides to participate in a day-long special art immersion event. The event includes participation in immersive performances (e.g. opera, ballet, theatre), immersive visual art creation (e.g. sculpture, crafts), visits to indoor landmarks (e.g. museum), etc.

### 9.12.3	Service Flows

The service flows are described for an interactive immersive experience in dedicated locations of the overall guided tour of Paris.

- The family is provided with instructions on activities and services. Hank is configured to interact with all family members as a group virtual teacher and guide with personalization (e.g. avatar, language, age related content, etc.) for each person. The behavior of the participants (e.g. haptic feedback, captured facial expressions, gestures are analyzed by AI) triggers a personalized immersive experience.

- The avatar positioning is using AR Spatial Computing Service to identify the best position within the real environment for visualization, participation and safety. Changes in the scene (such as another visitor walking through the scene) may result in changing the positioning of the avatar(s).

- The AR Spatial Computing Service has identified the Palais Garnier, and the guided tour application server provides XR content composed of a dance performance with volumetric video, immersive audio, haptics and virtual objects. The XR content is placed and rendered at an optimal position relative to the participants and the sensed environment.

- The application provides each family member immersive experiences, including 6 Degrees-of-Freedom (6DoF) experiences (i.e. each participant is free to change position as forward/backward, up/down, left/right translation, combined with changes in orientation).

- The father asks a question to the remote guide through his avatar. The XR content animation is paused for all the family members and the answer to the question is provided to all the family members.

- Once the dance performance is done, the family and the guide avatar enter the Palais Garnier. Each family member receives personalized touristic information (e.g. overlaid text, picture, 3D virtual asset, video, audio, haptics) linked to the real surrounding objects they discover during the walk.

- At another location, the young kid is guided by Hank to participate in an activity creating a virtual clay and pebbles sculpture. The XR content animation is synchronized for all the family members to watch the activity and interactions.

- Suddenly the 6G connection conditions fluctuate (e.g. changes in capacity and/or congestion) during a significant period of the presentation. The 6G network may adapt and/or trigger adaptations of the XR content to preserve an acceptable QoE for each family member during that period.

- When exiting the Palais Garnier, the teen replies to a question and shakes hands with the guide, and everyone sees Hank’s avatar doing it with the teenager. The teen kid wears a haptic glove to sense the handshake.

### 9.12.4	Post Conditions

With the advancements in interactive immersive XR technologies and 6G capabilities, visitors can acquire knowledge, touristic, and historical information of a city in a comprehensive and personalized way.

### 9.12.5	Existing features partly or fully covering the use case functionality

The following existing features can partly cover some functionalities of this use case.

Features on localized metaverse service for receiving locally relevant XR content are provided in the 3GPP SA1 Mobile Metaverse Services Study (TR 22.856 [49]) and Work (TS 22.156 [28]) documents. This includes exposure and coordination requirements in TS 22.156 [28] clause 5.1.1.2 and “spatial anchors” requirements n TS 22.156 [28] clause 5.2.1. The placement of the XR content in the real environment to ensure an optimal viewing, interaction and safety for each group member may need additional requirements.

Features on avatar-based real-time (including IMS) communication are provided in the 3GPP SA1 Mobile Metaverse Services Study (TR 22.856 [49]) and Work (TS 22.156 [28]) documents. The time and spatial synchronizations (e.g. coherent gestures, facial expressions, displacements) of the personalized avatars for the group of participants may need additional requirements.

Features on QoE/QoS management of multi-modal flows considering low round-trip latency, and the synchronisation of different media (e.g. haptic, video, avatar, audio) for each group member, is provided in TS 22.156 [28] (e.g. [R-5.1.1-002]). QoS management features using the Network Exposure Function (NEF) API are provided in TS 23.501 [140], TS 23.502 [30] and TS 23.503 [141]. A fast dynamic QoE/QoS adaptation to the fluctuations of the 6G connection conditions during a real-time communication may need additional requirements.

Features on Tactile and multi-modal communication service are provided in TR 22.847 [164], including use cases related to closed loop haptics (robotic, surgery etc.), with corresponding requirements in TS 22.261 [14] clause 6.43. Other haptics-based use-cases, such as those including open loop haptics (haptics enhanced media streaming, distribution and communication, avatar etc.) were studied in TR 26.854 [165].

To support the heterogeneous 6G devices of this use case, tethered AR glasses (TR 26.998 [166], TR 26.806 [167]) and split rendering architecture (TR 26.928 [50], TS 26.565 [51]) have been studied with QoE/QoS support. The synchronization of heterogeneous data flows from/to various UEs within a group of participants may need additional requirements.

Performance KPIs and parameters partially covering the functionality include:

- Synchronization threshold between audio and video for VR (TS 22.261 [14] clause 7.6.1), i.e. in the range of [125 ms to 5 ms] for audio delayed and in the range of [45 ms to 5 ms] for audio advanced.

- Synchronization threshold to ensure acceptable QoE for support of haptic media in addition to other audio and video media (TR 26.854 [165] clause 10) as follows:

  - in the range of [25 ms to 50 ms] for audio delayed and in the range of [25 ms to 12 ms] for audio advanced.

  - in the range of [15 ms to 20 ms] for video delayed and in the range of [50 ms to 30 ms] for video advanced

Additional performance specifications necessary to address this use case include:

- New synchronization threshold KPI ranges, dependent on scenario (e.g. max allowed end-to-end latency, user experienced data rate) are to be specified.

NOTE:	The synchronization threshold definition TS 22.261 [14] is assumed. Synchronization threshold can be calculated based on (but not limited to) one or more of the following: network delays, transmission & synchronization delays between data flows for a user, rendering delays, sampling frequencies of the media, buffer and jitter pacer, processing delays, mismatches between estimated and real positioning; variabilities between UEs for any given user, etc. Higher synchronization thresholds values correspond to less stringent user experiences.

### 9.12.6	Potential New Requirements needed to support the use case

[PR 9.12.6-1]	Subject to operator policy and privacy considerations, the 6G system shall provide means (e.g.  association of XR content with location and relevant positioning information) to ensure an optimal placement in viewer relative to the real environment (e.g. to ensure safety or optimal interactions between participants, etc.).

[PR 9.12.6-2]	Subject to operator policy and privacy considerations, the 6G system shall provide a means to synchronize media/flow and data for rendering in time and in XR space (e.g. coherent gestures, facial expressions the personalized avatars of the group of participants.

NOTE:	For “XR space” clarifications, see TS 26.119 [304].

[PR 9.12.6-3]	Subject to operator policy and privacy considerations, the 6G system shall support a mechanism, including enabling one or more authorized third party(ies) to adapt XR session QoS dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the fluctuations of the 6G connection conditions and QoE policies.

[PR 9.12.6-4]	Subject to operator policy and privacy considerations, the 6G system shall provide a means to synchronize heterogeneous data flows from/to a set of UEs (e.g. phone, glasses, tethered ring) associated with a single user and between UEs associated with multiple users in a multimodal communication (e.g. each user having glasses and/or phones).

[PR 9.12.6-5]	Subject to operator policy and privacy considerations, the 6G system shall provide a means to keep heterogeneous media flows synchronized within an acceptable threshold whether rendered on a single or multiple UEs, for each user in a multimodal communication.

The potential new KPIs detailed in Table 9.12.6-1 are needed to support the use case:

Table 9.12.6-1: KPIs for personalized interactive immersive use-cases

| Characteristic parameter (KPI) |  |  | Influence quantity |
| --- | --- | --- | --- |
| Scenario: Max allowed end-to-end latency (note 1) | Service bit rate: user-experienced data rate | Synchronization threshold (note 2) |  |
| Audio (UL/DL): [10] ms | Audio (UL/DL) [5-512] kbit/s | Audio-to-haptics lag: [25] ms; Haptics-to-audio lag: [12] ms | Delay (ms), Packet loss (%), Update rate (Hz), Packet size (bytes), Throughput (kbit/s) for each media type based on TR 26.854 [165] Table 10.3-1. |
| Immersive video (DL): [200-300] ms | Immersive video (DL) [10-20] Mbit/s | Visual-to-haptics lag: [20] ms; Haptics-to-visual lag: [30] ms |  |
| Avatar between remote guide and UEs: [20] ms | Avatar: [0.1-30] Mbit/s (depending on the format) Avatar animation: 2 Mbit/s uncompressed. 1 Mbit/s compressed | Audio-to-avatar lag: [25] ms; Avatar-to-audio lag: [12] ms |  |
| Pose & action data (UL): [5] ms | Pose & action data (UL) [100 – 400] kbit/s | Avatar-to-haptics lag: [20] ms; Haptics-to-avatar lag: [30] ms |  |
| Environment sensing data (UL): [5] ms | Environment sensing data (UL) [10 – 50] Mbit/s | Pose-to-visual lag: [50] ms (pose UL, visual DL) Visual-to-pose lag: [20] ms (visual DL, pose UL) |  |
| Haptic (DL): [5] ms | Haptic (DL): [0.25 – 160] kbit/s for parametric compressed format [up to 6400] kbit/s for time sample format. See. TR 26.854 165] Table 5.4-1. | Audio-to-pose lag: [50] ms Pose-to-audio lag: [20] ms |  |
| NOTE 1: Synchronization threshold values vary for active versus passive engagement scenarios. Scenarios other than the ones listed may require other synchronization thresholds for the same media combinations. NOTE 2: “media X to media Y lag” refers to the positive time difference between the reference media X component and the specified media Y component. For example, an “audio-to-haptics lag” of 25ms means that haptics media arriving within 25 ms after the audio is acceptable. |  |  |  |

## 9.7	Use case on multiple application media synchronization

### 9.7.1	Description

In critical immersive communications such as 3D remotely controlled repairs or surgery, users may be equipped with multiple devices for multiple media components (e.g. haptic device for pressure, VR glasses for video, wireless headphones for audio) and each device receives traffic for the corresponding media component from the networks.

Unless the user receives synchronized information from all application media flows involved in the remotely controlled activity – in this example, presented to the user through haptic gloves and headset – the user can easily make errors, e.g. destroying the equipment that is being remotely repaired, or harming the patient by remote surgical procedures.

Remote Repair Room

Remote Technician

App for remote control/haptics

UE

RAN

Core Network

App for visual

App for sensor data

App for audio

Figure 9.7.1-1: An example of remotely controlled repair

Then, the tighter synchronization between different media components is required. For example, a group of data carrying multiple media components is defined as a chunk of haptic data and chunk of video data and the group of data need to be delivered collectively within a required time window.

Also, the group of data including more than one media components can be delivered via more than one traffic flows and/or UEs when each device is receiving corresponding media component via specific flow.

In a case when the network condition is poor or varying, it would be hard to support this level of synchronization without the network to understand the inter-dependent packets across multiple flows.

The corresponding 5G requirements from TS 22.261 [14] below is defined for the multi-modality but it does not cover the above mentioned scenario.

-The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

-The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

In this use case there are different distinct application flows that must be synchronized, whereas in existing requirements, the media for a single application is synchronized.

### 9.7.2	Potential New Requirements needed to support the use case

[PR 9.7.2-1]	Subject to operator policy, the 6G system including IMS shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device (i.e. UE or tethered devices).

NOTE:	The applications whose traffic flows are synchronized are associated. It is assumed that the association is known to the 6G system.

Table 9.7.2-1:  KPIs for media synchronization for multiple applications

| Use case | Audio-Haptic synchronization thresholds | Video- Haptic synchronization thresholds | Audio-video synchronisation thresholds |
| --- | --- | --- | --- |
| Remotely controlled repair | - In the range of [50 ms to 0 ms] for audio delayed (NOTE 1) - In the range of [25 ms to 0 ms] for audio advanced (NOTE 1) | - In the range of [15 ms to 0 ms] for video delayed (NOTE 1) - In the range of [50 ms to 0 ms] for video advanced (NOTE 1) | - In the range of [125 ms to 5 ms] for audio delayed (NOTE 2) - In the range of [45 ms to 5 ms] for audio advanced] (NOTE 2) |
| NOTE 1: as defined in TS 22.261 [14] clause 6.43.1. NOTE 2: as defined in TS 22.261 [14] clause 7.6.1. |  |  |  |

## 9.18	Use Case on Immersive Audio Production in Live Events

### 9.18.1	Description

Definitions

transfer interval: time difference between two consecutive transfers of application data from an application via the service interface to 3GPP system. As stated in TR 22.827, transfer interval refers to the periodicity of the packet transfers. It has to be constant during the whole operation time.

end-to-end latency: (see TS 22.261 [14]) the time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

audio processing time: the time that it takes at application to perform all kinds of audio data processing such as, filtering, gain, encoding, decoding.

latency experienced by the user: the total one-way delay between capturing an analog audio signal and replaying it. It is the sum of all internal time delays introduced by all involved entities of the wireless system to transfer an analog signal from a source, e.g. a microphone, to a sink where the original signal is reproduced in an analog manner, e.g., at a loudspeaker. This latency does not refer to a single data packet but to all packets of a consecutive data stream with a constant periodicity, often called as real-time communication.

mouth-to-ear latency: (see TS 22.263 [67]) end-to-end maximum latency between the analogue input at the audio source (e.g. wireless microphone) and the analogue output at the audio sink (e.g. IEM). It includes audio application, application interfacing and the time delay introduced by the wireless transmission path.

real-time latency budget: the overall real-time latency budget is composed of different parts depending to which kind of latency it refers. Figure 9.18.1-1 shows the components that build up the latency experienced by the user. During the time period of the transfer interval the analog audio signal is converted from analog to digital and split into frames of a constant size in samples. Then the audio data are further processed, e.g. audio encoding, and delivered to the wireless network. The network adds its specific end-to-end latency and after receiving the data, the application decodes the audio data and converted it from digital to analog to generate an analog signal that can be replayed. In case of unsynchronized operation between application and 3GPP network, additional time for buffering at the interfaces between the two must be included.

Figure 9.18.1-1: Latency experienced by the user

In a live event or concert, immersive audio production aims to fully engage the audience by delivering three-dimensional soundscape experience. The three-dimensional listening experience can be enjoyed either on-site or remotely via a live audio stream. This is achieved by using audio devices that capture the sound coming from all directions and combine their outputs accordingly to create a three-dimensional audio stream. The production of immersive audio streams can be supported by two different types of devices:

•	Type A: microphone array

o	contains at least four audio transducers,

o	delivers either an immersive audio output stream or the raw data of each transducer, and

o	has capabilities for localization.

•	Type B: single microphone

o	has capabilities for localization,

o	includes a mechanism for tight synchronization, and

o	outputs a raw mono audio stream.

The type of capturing device used depends on the respective usage scenario.

In addition to the capturing devices, monitor devices are used in audio productions that replay the audio signals on loudspeakers, commonly known as stage monitor wedge, or individual in-ear monitor systems (IEM).

IEMs typically receives a binaural, stereo, or double-mono mix, while the wedge is fed by a mono mix. Stage monitor wedges are usually not wireless, as they generally have a built-in amplifier or are connected to an external close-by amplifier. IEMs are always based on wireless transmission.

Synchronicity requirements

The devices deployed in live audio production scenarios may require support for time synchronization, in particular to:

•	support alignment of user data from different audio sources (wireless microphones),

•	minimize buffering when processing multiple isochronous data streams, and

•	reduce latency by optimizing interfacing and audio data flow through all network entities, i.e., alignment of the processing of the audio data with the scheduled 3GPP network traffic.

Synchronicity is defined as the maximum allowed time offset between the synchronization leader and any network device synchronized with it specified in seconds.

The requirements for the alignment of the phase locked mono audio streams depend on the targeted output stream (see 5G-MAG report: “Time synchronization services for media production over 5G networks”):

Editor’s Note:  reference for this report is needed.

•	mono audio signal: 50 - 100µs,

•	stereo audio signal: 10 - 50µs, or

•	immersive audio signal: 1 - 10µs.

For immersive audio capturing with independent audio devices generating a mono audio stream such as wireless microphones, the phase difference between the different audio streams must be known to compensate for it later when generating the immersive audio stream, or it must be so small and constant that it has no impact on the immersive audio output stream.

If the phase difference is in any order and varies, it might be very difficult or nearly impossible to produce an immersive audio image of the previous captured scenario.

Latency requirements

In general, latency of a data network depends on the data service, i.e., packet size and transfer interval / periodicity. Typical network performance tests use Ping measurements with 32-byte packets in random sequence without fixed transfer intervals. This might be valid to explore average network performance but does not reflect live audio applications where a streaming latency should be used to specify the overall system latency.

The overall system latency is made up of various components, including transfer interval, end-to-end latency, audio processing time, and sound propagation time in case of not using IEMs.

Latency in live monitoring scenarios can degrade the perceived quality of the audio and even affect the artist’s / user’s ability to perform. Different factors influence the effect of latency, such as the musical abilities of the musician, his critical listening skills, and the type of instrument played. In addition, the monitoring environment also affects the audibility of latency. Therefore, it does not exist a hard limit for the latency requirements but different limit ranges for different live monitoring scenarios and applications. The total tolerated latency varies from 4ms in case of live monitoring with IEM up to 40ms in case of live monitoring with wedge monitor.

### 9.18.2	Pre-conditions

-	The local operator or the person responsible for the production network provides a network that supports all needed requirements regarding latency, synchronicity, and reliability.

-	All wireless microphones and IEMs on-site are switched on, registered, and connected to the 6G system.

-	Audio processing units for e.g. creating IEM streams or immersive audio streams are connected to the 6G system.

### 9.18.3	Service Flows

A typical immersive audio service flow in a live event is composed of several performers using immersive audio capturing devices (wireless microphones) and an audience experiencing the immersive audio content through loudspeakers installed at the venue or remotely through streaming platforms.

1.	The event location is equipped with sound capturing devices according to the producer’s instructions.

2.	Each sound capturing device supplies audio and position information to the 6G system.

3.	Audio processing units connected to the 6G system combine the incoming audio streams from the different capturing devices and generate required audio output streams, e.g. individual IEM streams, immersive streams for on-site sound reinforcement, or immersive streams for remote consumption.

4.	The generated audio streams are sent back to the respective sink.

5.	The audio stream is played back through the respective audio device.

### 9.18.4	Post-conditions

The immersive audio production at a live event should operate as needed without any perceivable interruptions/audible impairments during the entire duration of the event.

### 9.18.5	Existing features partly or fully covering the use case functionality

In TS 22.263 [67] the AV production use case is described in detail and its associated requirements are explained and defined. Here in clause 4.3, it is pointed out that “the 5G system should enable non-public networks that can be deployed in an agile ad-hoc way”. The related requirements of NPNs are described in TS 22.261 [14], clause 6.25.

Periodic deterministic communication required for audio streaming applications in live scenarios is already described in TS 22.104 [64], clause 5.2, and examples for the required transfer interval can also be found there as well as in TS 22.263 [67]. The QoS model to support low latency real-time communication is defined in TS 23.501 [140], clause 5.7. Standardized 5QI values are also specified that support a packet delay budget of 5ms with a guaranteed flow bit rate.

Synchronicity is a key parameter to combine individual mono audio streams and convert them into an immersive audio stream. In TS 22.263 [67] clock synchronization is already described but it refers to align video and audio streams only. In addition, TS 23.501 [140] describes features that can be used independently or in combination to enable time-sensitive communication, time synchronization and deterministic networking. To interact with the 5G system two types of translators are described: the Device-Side TSN Translator (DS-TT) on the UE side and the Network-Side TSN Translator (NW-TT) on the 5G network side.

### 9.18.6	Potential New Requirements needed to support the use case

[PR 9.18.6-1]	The 6G system shall be able to provide deterministic low-latency with the KPI requirements summarized below:

Table 9.18.6-1: Performance requirements for immersive audio production in live events.

| Scenario | # of active UEs (Note 1) | Max. UE speed [km/h] | Max. service area [m²] | Synchro- nicity [µs] (Note 2) | Max. E2E latency [ms] (Note 3) | Positioning accuracy [m] (Note 4) | Min. packet error rate | User data rate UL [Mbit/s] (Note 5) | User data rate DL [Mbit/s] (Note 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE device type A: |  |  |  |  |  |  |  |  |  |
| large-scale event | 15 - 80 | 50 | 500 x 500 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 5 - 20 | 0.5 - 2 |
| small-scale event | 1 - 15 | 10 | 50 x 50 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 5 - 20 | 0.5 - 2 |
| UE device type B: |  |  |  |  |  |  |  |  |  |
| large-scale event | 50 - 300 | 50 | 500 x 500 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 1.2 - 2.5 | 0.5 - 2 |
| small-scale event | 4 - 50 | 10 | 50 x 50 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 1.2 - 2.5 | 0.5 - 2 |
| Note 1: the figures are estimated assuming a steady increase in the use of wireless mics based on the development of the last decades Note 2: according to 5G-MAG report on “Time synchronization services for media production over 5G networks” Note 3: more stringent values compared to TS 22.263 [67], Table 6.2.1-1 as immersive audio requires higher processing time for encoding and decoding Note 4: estimated range based on experience with current immersive audio productions Note 5: range from uncompressed audio with 24bit / 48kHz up to 24bit / 96kHz as for immersive audio uncompressed audio is preferred and there is no compression mechanism for immersive audio available at this point in time Note 6: range from compressed audio up to uncompressed audio with 16bit / 48kHz for the use in IEMs |  |  |  |  |  |  |  |  |  |

NOTE:	This table is derived from Table 6.2.1-1 of TS 22.263 [67] but specifies more stringent values for E2E latency and user data rates and specifies the additional KPIs synchronicity and position accuracy as this is required for immersive audio production.

Editor’s NOTE: Values in Table 9.18.6-1 are FFS.

## 9.3	Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

### 9.3.1	Description

Usually, the limited image rendering capability of mobile devices poses challenges for services that require high-quality image processing capability like high order super-resolution and de-noising algorithm, as well as high-quality rendering capability like ray tracing effect for gaming. As shown in Figure 9.3.1-1, the GPU computing power for image rendering increasing in last ten years and the predication up to 2030 for mobile phone platform as well as the computing power required for advanced services has been summarized as below based on [16], [17], and [18]:

Figure 9.3.1-1: increasing computing power: offered by mobile GPU vs. required by services

It can be observed that with the failure of Moore's law, the growth of mobile platform computing power gradually slows down. Offloading rendering task to cloud (including operator managed data network, edge or central cloud), where sufficient computing power is hosted, is a promising trend to support advanced multi-media services in the future while lowering the weight and the power consumption of the XR glasses and mobile phones.  UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile devices to dynamically offload part or all of rendering task to the cloud to improve the user experience. As shown in Figure 9.3.1-2 the offloading decision can be based on communication link status and computing resource availability in the cloud (operator managed data network, edge or central cloud) in order to maintains deterministic rendering task processing latency. There's an expectation for 6G systems to consider such dynamic rendering offload framework to adapt to the synergized multi-media service processing trend.

Figure 9.3.1-2: UE-Network-Cloud synergized multi-media operation framework

The rendering task split function is a function provided by the operating system (OS) of the terminals to map the rendering task request from the application (APP) to local or remote rendering resources. When a rendering task is requested to be executed, the task split function can select the local and/or the remote rendering resources based on the relevant factors, such as required processing quality, radio link status, estimated latency, available computing resource in operator managed data network, edge or central cloud, to ensure the deterministic user experiences for the requested multi-media service. The typical services which can be benefited from UE-Network-Cloud synergized multi-media operation include photo enhancement and ray tracing for gaming and so on.

The UE-Network-Cloud synergized operation raises several new issues, as follows:

-	Uplink data rate: The rendering task offloading operation usually requires a high uplink data rate to achieve higher service availability, considering the metadata to be uploaded for the rendering task is usually large with tight latency, e.g. 5 Mbits/50 ms, 150 Mbits/1.5 s (see detail in service flow part). However, the current network typically cannot guarantee sufficient uplink data rate especially at cell edge.

-	awareness of real time communication link capability and cloud computing resource availability: The rendering task split function might not know the exact communication link capability such as data rate, latency, and UE power consumption for data transmission; the computing resource availability in operator managed data network, edge or central cloud is also unknown, which make it hard for the UE to make the splitting/offloading decision. Moreover, the latency for offloading rendering task to cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage, so the UE needs to be aware of the real time network and computing resource status to switch between local and remote rendering.

-	Computing task level Packet Delay Budget (PDB) requirement guarantee: The uplink metadata burst for a rendering task offloading arrives in a random way, and the latency requirement is intended for the whole data burst (i.e. the metadata of the task) rather than an individual packet. As shown in Figure 9.3.1-3, the current Guaranteed Bit Rate (GBR) QoS flow framework guarantees the data rate in a fixed averaging window and fulfils latency in a per packet basis which is suitable for the periodic traffic. For the burst data carrying the metadata for rendering tasks, the latency and data rate should be guaranteed toward the whole data set rather than a specific packet. Therefore, the fixed averaging window as for periodic traffic is not suitable for bursty traffic. There is no suitable QoS type to guarantee the deterministic transmission latency for unpredictable uplink bursty data traffic.

Figure 9.3.1-3: Burst latency requirement

Moreover, in comparison to the accidental single-packet loss whose effect can be mitigated by the packet-loss-resilient solution, the successive packet loss or burst packet loss will incur more frequent video and audio buffering and play stalling. To solve such issue, AI/ML based approach to concealing the lost packets draws a growing attention, which is capable of recovering the missing audio packets as long as 100 ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [101].

### 9.3.2	Pre-conditions

1. Rendering resources are deployed in the cloud (operator managed data network, edge or central cloud).

2. The UE operating system supports to map the rendering task from the APP to the local (in the mobile phone) and/or the remote (in the cloud) rendering resources.

### 9.3.3	Service Flows

Synergized photo enhancement case:

1.	User A is at a tourist attraction, taking photos with his mobile phone. Once he clicks the photo button, the phone senor produces raw data of the photo, including up to 6 frames (4K per frame) of raw files spanning 300 ms for HDR reconstruction. Raw files retain the original uncompressed image data from the camera sensor, which is usually larger than compressed image formats such as JPEG and PNG and has unique advantages for image processing in practice.

NOTE 1: 	2 to 10 frames are needed for HDR reconstruction based on [19]; 6 frames are assumed in this use case.

2.	Upon reception of the rendering task request from the camera APP for photo enhancement, the rendering task split function inside the UE OS determines whether or not to upload the raw data to the cloud for more powerful post-processing to enhance the photo quality, for which some factors would be considered such as potential delay and UE power consumption for data transmission.

The rendering task split function obtains necessary information from the network, including real time communication link capability and cloud computing resource availability, and coordinates with the core network about whether the rendering task can be fulfilled (e.g. having bandwidth and latency assurance, locating appropriate computing resources for processing the photos, etc.).

NOTE 2:	It is assumed that the core network is able to obtain the computing resource availability in operator managed data network, edge or central cloud.

3.	If the rendering task split function decides to upload the photo to the cloud, the raw data for the photo is compressed to around 150 Mbits (based on the assumption of 6 frames × 4K raw data and compressed ratio assumption in [20], [21]) and uploaded to the cloud within around 1.5 seconds and downloaded the processed photo from the cloud. The UE can reduce the number of frames for uploading based on communication link capability. Therefore, the required uplink data rate in radio layer would be 150 Mbits/1.5 seconds = 100 Mbps.

NOTE 3: 	E2E latency of 3 seconds is assumed based on users' patience statistics as shown in [22], where 1.5 seconds is allocated for uploading to the cloud while 1.5 seconds for processing in the cloud and downloading to the UE.

4.	If the rendering task split function decides not to upload the photo to the cloud, the local post-processing will be performed.

5.	When User A views the photos, the downloaded photo or the local enhanced photo can be shown.

Synergized gaming enhancement case:

1.	On the train back home to celebrate Chinese New Year, User A starts a delay-sensitive first-person shooter game displayed on XR glasses with friends online, where the gaming APP at the connected mobile phone produces the scene metadata, including 3D objects, lighting data, and materials, user actions,  and then encodes the scene before transmits it to the XR glasses. The typical data size after compression can be 5-20 Mbits (assuming middle to high complexity 3D model including 0.35 to 2 million vertexes (200 bits per vertex) 3D models and compression ratio assumption for 3D model in [23]).

To win the battle, User A and friends must frequently exchange the status of their game roles by voice call supported by the gaming APP, all the time during the game even when network connection is poor (e.g. when the train going through the tunnel).

2.	The gaming APP at the mobile phone submits the metadata to rendering task split function inside the OS for rendering, the rendering task split function can seek to utilize the more powerful computing in the cloud for higher quality such as ray tracing effect. The rendering task split function determines how to split the metadata to the cloud by considering some factors such as potential delay and mobile phone power consumption for data transmission. For example, the mobile phone selects to offload the slow changed scene background rendering to the cloud while performing rendering for fast moving objects using local rendering resources.

The rendering task split function further coordinates with the core network to check whether the rendering requirements can be fulfilled and sends an offloading request to the core network if all conditions are met.

3.	The mobile phone uploads the metadata allocated for the cloud rendering within 50 ms [24] and downloads the rendering result.

4.	The mobile phone combines the local rendered result and cloud rendered result to User A.

5.	Once the radio condition worsens or the rendering resource is not available, the full local rendering can be activated to maintain the user experience.

### 9.3.4	Post-conditions

User A continues enjoying high quality photo taking and high-quality gaming experience.

### 9.3.5	Existing features partly or fully covering the use case functionality

The split rendering architectures defined in TR 26.928 [50] and TS 26.565 [51], and the corresponding QoS requirements in TS 22.261 [14], allow a UE to offload rendering requirements to the cloud for XR rendering. These architectures assume that the metadata including 3D models of the gaming and APP logic is pre-installed in the cloud, therefore only user pose information is needed to be transferred in the uplink and there is no dynamic offload decision on whether to render a specific scene in the cloud or with UE local resources (i.e. fully rely on the rendering result in the cloud). Therefore, the uplink requirement and dynamic offload decision requirement are not covered by the existing split rendering use cases and part of the downlink requirements on rendering result delivery are covered by the existing split rendering use cases.

It is worth noticing that Edge Enabler Client (EEC) is defined in TS 23.558 [52], which provides supporting functions needed for Application Clients. There is some similarity between EEC and the "rendering task split function" described in this use case. In this use case, instead of communicating with the Edge Enabler Server (EES) in the DN that only manages the edge applications, the rendering task split function coordinates with the core network for splitting/offloading rendering tasks.

In TS 22.261 [14] clause 7.6.1, there is requirement for 5G system to support interactive task completion during voice conversation: the 5G system shall support low-delay speech coding for interactive conversational services (100ms, one-way mouth-to-ear).

### 9.3.6	Potential New Requirements needed to support the use case

[PR 9.3.6-1] Subject to user consent, the 6G system shall support burst type QoS with the KPI requirements summarized below:

Table 9.3.6-1:  KPIs for multi-media services with deterministic experience via collaborative processing among UE-network-cloud

| Use cases | Burst size (note 1) | Max Allowed latency for a burst (note 2) | Service bit rate: user-experienced data rate | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement | UL: [150 Mbits] | UL: [1500 ms] | UL: [100 Mbit/s] | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement | UL: [5-20 Mbits] | UL: [50 ms] | UL: [100 – 400 Mbit/s] | Stationary or Pedestrian | Countrywide |
| NOTE 1: Assuming 6 x 4K raw pictures and compression ratio in [21] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [23] for gaming enhancement. NOTE 2: 1500 ms is derived from the E2E latency of 3 s (based on users' patience statistics as shown in [22]) and 1.5 s for processing in the cloud and downloading. 50 ms uplink latency is derived from [24]. |  |  |  |  |  |

[PR 9.3.6-2] Subject to user consent, the core network of the 6G system shall provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real-time network status and the computing resource availability in the Service Hosting Environment, edge or central cloud.

NOTE 1:	In the case of tethered XR headset connected to a UE, the latency, computing resource consumption and power consumption (incurred by application processing and communication processing) of both the UE and the XR Glasses need to be also considered.

NOTE 2:	It is assumed that the core network is able to obtain the information of computing resource availability in the Service Hosting Environment, edge or central cloud, which enables the core network to locate appropriate computing resources for the rendering tasks.

[PR 9.3.6-3] Subject to operator policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed throughput (current and/or predicted, within the boundary of the 6G system) of the associated ongoing session.

[PR 9.3.6-4]: The 6G system shall be able to provide deterministic user experience for multi-party call with the KPI requirements summarized below:

Table 9.3.6-2:  KPIs for deterministic user experience for multi-party call

| Use case | Max. mouth-to-ear delay | Audio-video synchronisation thresholds | Max. duration of consecutive packet losses | Throughput (UL and DL) | Availability | UE speed |
| --- | --- | --- | --- | --- | --- | --- |
| multi-party call | [100ms] (note 1) | [- in the range of [125 ms to 5 ms] for audio delayed - in the range of [45 ms to 5 ms] for audio advanced] (note 2) | [100 ms] (note 3) | [>=30Mbps] (note 4) | [99%] (note 5) | up to [500km/h] (note 6) |
| NOTE 1: one-way delay [102]. NOTE 2: as defined in TS 22.261 [14] clause 7.6.1. NOTE 3: it is referring to the capable of recovering the missing audio packets as long as 100ms, based on the assumption of 20ms voice samples encapsulated into one audio packet [101]. NOTE 4: it is derived based on 4K 60 fps video encoded with HEVC [103]. NOTE 5: it means the probability to provide the above KPIs during the time that a user intends to use the above services. NOTE 6: it is to consider the high-speed train scenario as in TS 22.261 [14] clause 7.1, which is intended as a targeted value and not a strict requirement. |  |  |  |  |  |  |

Copy from AI session:

## 6.10	Use case on home robots

### 6.10.1	Description

In 6G era home robots will engage in household chores based on preconfigured models, such as sweeping the floor, vacuuming, folding clothes, washing dishes, and organizing rooms. They will also take care of family members by monitoring health data, reminding medication, and dialling emergency calls. Additionally, they will socialize and entertain with humans while interacting with other smart devices to create a more intelligent ecosystem. As shown in Figure 6.10.1-1, all of this aims to bring us a more convenient, comfortable, and safe family life. In this use case, not all of the AI inference actions are performed solely by home robots.

Figure 6.10.1-1: Home robot performs specific tasks based on pre-trained models

As shown in Figure 6.10.1-2, the working principle of the robot includes two main components: perceive and action. "Perceive" refers to the process of sensor data from, such as, cameras or radar, for the input to a large model and obtaining perception result. "Action" refers to the process of taking perception result and task as input to a large model and obtaining action parameters to control the joints of the robot to complete the corresponding action.

task

Sensing data

Perception result

Perceive

Action parameters

Action

Figure 6.10.1-2: The perception-action loop of a home robot

In the case of specific tasks in specific scenarios, the perception model can be deployed locally on the robot as its size is not large. However, for general tasks in open environments, the perception model needs to achieve the perception ability similar to an adult human brain towards the world. Based on psychophysical and neurophysiological studies [281], [282], humans can perceive the gist (e.g. overall meaning) of complex visual scenes within around 150ms after stimulus onset. This rapid initial perception enables us to grasp the general context of a scene, even if the stimulus is briefly presented (around 10ms). In the meantime, it takes longer for a human being to identify individual objects, e.g. finer object identification requires larger than 200ms [283] and around 300ms [284] In the target scenarios, robots are expected to have similar perception time as an average human. The number of neurons in an adult human brain is approximately 100 billion, and the current model size for complex logical reasoning and task orchestration in laboratory or limited environments, the model size is also generally on the order of 100B. Due to cost and size limitations, robots are unable to locally implement perception functions for free and open environments.

Also, considering that it may exist the situation that to distribute the model to several robots for local inference. When offering services enabled by AI inference, the mobile network also meets the challenge of providing enough computing power within reasonable cost and ensure its end-to-end performance. Therefore, the mobile network system could consider to support some of these distributed inference techniques, and allows the subscribers to choose and utilise them to improve the service quality.

For example, in this use case, a robot and the network could negotiate and perform the layer split distributed inference together, i.e., some layers of the model could be performed on the UE, while the others would be offloaded to the network. By using this technique, the UE could run a large model whose computing requirements may be far beyond the capabilities of a typical mobile phone, while protecting the data privacy by making some local data processing.

The 6G network will be equipped with perception models, enabling it to provide perception services  (i.e. 6G network performing AI inference of these models on behalf of robots, to generate the perception)to robots in open environments whenever necessary.

This use case aims to provide timely perception and action guidance via the 6G network, assisting robots cope with complex real-world environments and provide users with better intelligent services.

### 6.10.2	Pre-conditions

The Operator X offers comprehensive 6G services for robots, including communication and AI services. Relevant resources have been deployed in the network to facilitate services like large model inference (e.g. using the AI/ML models are provided by 3rd Party), assisting robots in delivering enhanced performance in complex environments.

Lily purchases a robot to assist with various house chores. The robot exhibits a certain degree of local intelligence. By equipping it with one or more advanced models, the robot can better understand the owner’s basic needs by controlling its body joints. For example, it can help with household chores, take care of family members’ well-being, engage in social interactions and entertainment with humans, as well as interact seamlessly with other smart devices.

Lily has subscribed to specific 6G services (communication and AI services) for her robot. Therefore, the robot is able to access to 6G services, such as perception service.

The robot has two perception modes as shown in Figure.6.10.2-1:

- Perception mode a) for specific task locally in robot; and

- perception mode b) for general task in open environments via 6G network.

(a) perception for specific task locally in robot

(b) perception for general task in open environments via 6G network

Figure 6.10.2-1: Two perception modes of a home robot: (a) perform local perception for specific task, and (b) perform perception for general task in open environments via 6G network

When a robot is instructed by its owner to start general tasks in an open environment, it will request the perception service for the open environment from the 6G network (e.g. 6G network performing AI inference using the AI/ML models provided by 3rd Party), and, the robot will interact more frequently with the 6G network, which helps ensure safer behaviour through perception services.

### 6.10.3	Service Flows

Step 1: One day, while on a shopping trip with Lily and her child, the home robot carried a newly purchased wine glass that Lily had carefully selected. At this time, the robot follows Lily, moves when Lily moves and stops when Lily stops, holding the newly purchased wine glasses. Therefore, it is Perception mode a), and only the local model on the robot is needed to work, as shown in Figure 6.10.3-1(a). However, when an urgent phone call interrupted their outing, and Lily tells the robot that she is going to take a call and asks the robot to watch the child. The robot finds that taking care of the child is a general task in an open environment, so it switches to perception mode b), which the robot cannot support locally, as shown in Figure 6.10.3-1(b).

(b) perception for general task in open environments

(a) perception for specific task locally in robot

I'm going to take a call, please watch the kids for me.

Follow me and help me carry the wine glasses.

Figure 6.10.3-1: The robot switches perception mode from perception mode a) to perception mode b) after lily’s instruction

Step 2: Considering the AI service capability supported by network (e.g. inference for perception service) and AI model(s) supported by network, the home robot requests the perception service for general task in open environments from the 6G network and frequently sends locally collected important information to the 6G network. The request message includes the perception requirements/demand (e.g. latency, inference accuracy) from the home robot. Then the 6G network determines whether the perception requirements can be satisfied taking UE demand in the request message into account and then sends the response (e.g. acknowledgement including promised requirements) to the home robot.

Step 3: (Perception) The 6G network utilizes the sensing data transmitted by the robot, optionally incorporating environmental information from 6G network itself, as input for the model inference (e.g. using the 3rd Party provided AI/ML models). When the 6G network perceives a normal situation, and it will not interrupt robot’s current task, as shown in Figure 6.10.3-2 (a); when the 6G network perceives a dangerous situation quickly enough comparable to how an average adult does, such as, the ball suddenly falls from the child's hand and rolls to the side of the road while the child runs to pick up the ball just before a car approaches, it generates corresponding behaviour guidance to send to the robot, as shown in Figure 6.10.3-2 (b).

task

Figure 6.10.3-2: The perception-action loop when a normal situation or dangerous situation is detected by the 6G network, respectively

Step 4: The 6G network delivers ACK or the perception result to the home robot within the latency included in step 2.

Step 5: (Action) The robot uses local perception result or perception result from the 6G network, along with the task, as input to the local large model to output action parameters for action. The actions corresponding to a dangerous situation detected by the 6G network may include, for example:

Action 1: Drop the box containing the cup;

Action 2: Move towards the child while emitting an alarm sound;

Action 3: Pick up the child and move to a safe area;

Action 4: Explain to Lily what happened and why the box containing the cup was dropped.

Step 6: After executing one or more actions suggested in step 5, the robot assesses the action results. It then reports the service experience (e.g. bad, fair, excellent) along with locally collected environmental data to the network. In this way, the 6G network monitors the performance of the perception service and, if necessary, may upgrade AI model (e.g. by retraining, switching) to guarantee the inference accuracy.

Step 7: When at home, Lily plays with her child to draw the picture. Lily starts the robot and requests the robot to execute a GenAI application to generate a picture of scenery of mountains and rivers. But the robot cannot load and execute the full large model due to memory limitation, it sends a request to network and ask for the distributed inference service.

Step 8: If the distributed inference service is available, the robot could further negotiate with the network about the inference mode, QoS requirement, etc.

For example, the robot could ask for layer split inference, and provide information including the required inference framework (e.g., pytorch) and model profile (e.g., model structure, number of layers, suggested layers to be offloaded to the network, etc.). In addition, the robot may request for certain QoS requirement, e.g., end-to-end latency, which combines transmission and computing processing latency.

Based on the computing and network resource status, the network may accept the request as it is, or may propose to update some configurations, or even provide its own suggested parameters for the robot to check. For example, in the case of layer split inference, the network may negotiate with the robot or decide by itself about the number of layers to run on the network.

Step 9: After finalizing the configurations of the distributed inference service, the network will choose appropriate computing resource in the Service Hosting Environment for running the model segments and fulfil the service requirements, e.g., end-to-end latency.

Step 10: The robot could either upload the relevant portion of the split model to the network, or provide necessary information for the network to get and load the model. Once both the robot and the network load the model segments, they could conduct the distributed inference service collaboratively.

Step 11: Finally, Lily receives the final picture.

### 6.10.4	Post-conditions

If the robot relies solely on local model inference, it may encounter system crashes in overly complex situations. Alternatively, if it only considers executing the previous command from the female owner, it may pose a danger to the child. However, by providing timely perception and action guidance through the 6G network, the requirements for the robot's local intelligence level can be reduced. Users can purchase a robot at a lower price and obtain high-intelligence robot services that can more easily adapt to complex real-world environments.

### 6.10.5	Existing features partly or fully covering the use case functionality

Currently, 3GPP systems only provide communication services, and there is no functionality to provide AI as a service.

### 6.10.6	Potential New Requirements needed to support the use case

[PR 6.10.6-1] Subject to operator’s policy, the 6G network shall be able to provide AI service (e.g. AI model inference) to a UE.

[PR 6.10.6-2] Subject to operator’s policy, the 6G system shall be able to support negotiation of the service performance (e.g. latency, inference accuracy), between UE and 6G network, when providing AI service (e.g. AI model inference).

[PR 6.10.6-3] Subject to operator’s policy, the 6G network shall be able to support mechanism to guarantee the service performance (e.g. latency, inference accuracy) when providing AI service (e.g. AI model inference).

[PR 6.10.6-4] Subject to operator policy and user consent, the 6G network shall support mechanisms for scheduling and provisioning network resources and computing resources in Service Host Environment for distributed AI inference service.

[PR 6.10.6-5] Subject to operator policy, the 6G network shall support mechanisms to ensure a guaranteed service performance of the distributed AI inference service.

NOTE: The UE is not involved in distributed AI inference service.

[PR 6.10.6-6] The 6G system shall be able to provide the communication and AI service for user with the KPI requirements in the Table 6.10.6-1 below.

Table 6.10.6-1: KPIs for home robot perform perception for general task in open environments via 6G network

| Use case | Traffic type | Average packet size (Byte) | Transfer interval (ms) | Data Rate (Mbps) | Joint E2E latency (ms) (NOTE 6) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| home robot perceiving overall context of the scene (NOTE 5) | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [150] (NOTE 3) | [99.9 %] |
| for identifying individual objects in the scene (NOTE 5) | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [200-300] (NOTE 4) | [99.9 %] |
| NOTE 1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE 2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE 3: Based on psychophysical and neurophysiological studies [281], [282], human beings can perceive the gist (e.g. overall meaning) of complex visual scenes within around 150ms after stimulus onset. This rapid initial perception enables us to grasp the general context of a scene, even if the stimulus is briefly presented (around 10ms). NOTE 4: For human beings, it takes longer for a human being to identify individual objects, e.g. finer object identification requires larger than 200ms [283] and around 300ms [284]. NOTE 5: In the target scenarios, robots are expected to have similar perception time as an average human being. NOTE 6: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment), and UE is only considered to contribute to the communication service latency. |  |  |  |  |  |  |

## 6.49	Use case on 6GS providing low-latency AI inference service

### 6.49.1	Description

Robots are expected to be widely used in future daily life, such as in casual interactive sports, education, family life (incl. household chores). A mobile AI-embodied robot can be an example to describe how the device can execute the specific task. The robot can be equipped with a number of sensors incl. cameras to collect the environment information around the robot, e.g., images taken at a certain frequency. To represent an advanced approach to replicating human-like intelligence and motor control, mobile AI-embodied robot design usually mimics both the human brain aiming at broad generalization and situational awareness similar to human cognitive functions (“Brain” illustrated in Figure 6.49.1-1) and cerebellum focusing on motor control, precision, coordination, and timing of movements (“Cerebellum” illustrated in Figure 6.49.1-1). Depending on the applications, the visual data. incl. images captured by robot sensors are fed into Brain, where a large vision-language model (VLM) can be used. The Brain is expected to understand the input information (e.g. images, any additional multi-model information) and conduct corresponding analysis for task orchestration, generate instructions, action planning and afterwards sending the actions/instructions to a robot cerebellum, e.g., implemented with a vision-action model (VLA). The Cerebellum further takes the input from Brain, and further determines the discrete actions to, e.g., pick up the badminton in the court, open the refrigerator.

Figure 6.49.1-1 Functional units used for Home robot

For mobile AI-embodied robots, usually a specific/complex task requires a large amount of AI reasoning. The traditional equipment with the limited capabilities (such as limited size of LLM), computing resource and battery power may not be able to run these complex tasks locally for an ideal experience. Power consumption, heat dissipation, memory size and computing capability are the four main physical constraints to limit the LLM size supported by a terminal (e.g. phone, robots, AR/VR etc). Power consumption, for example, limits the application of AI inference in mobile devices. Assume we have a chip which supports 1~5Hz multimodal inference for one 7B LLM [275], and the battery capacity is around 1kWh, the robot can only operate for 2~4 hours under full load. The above issues can be well-resolved by offloading complex computational tasks (AI inference) from a device with limited capabilities and resources to a more capable remote processing node, which is also a developing trend in the market and can improve the battery charging cycle.

The Brain and Cerebellum can be deployed together (e.g. both in SHE) or separately (e.g. Cerebellum locally on robot, Brain in SHE as it is designed for action planning which requires a large amount of sophisticated AI reasoning/inference with VLM). The Cerebellum controlling action execution may involve invoking the software development kit in the robot. Considering the UE limited capabilities mentioned above, it is suggested not to deploy the Brain and Cerebellum together in the robot. The complex computing task can be offloaded from the robot to the processing node.

The robot can perform different tasks based on the given specific scenarios.   For some interaction tasks, the robot receives the user prompts and sends the corresponding response. The service flow is described in clause 6.49.3. Corresponding to human reaction time, the required response time is 200ms~300ms for better user experience [276].

Many research articles show that the average human reaction time is 200ms~300ms. The article [276] from the International Computer Science Institute reveals that the differences across the languages within a range of 250ms ~300ms for response in conversation. The response time plays a critical factor to make conversations/interaction with a human being natural. The fastest a human can react to some stimulus with a vocal response under maximally favourable conditions is just below 200ms from [277]. One source document proposes an average reaction time of 228ms for auditory stimuli and somewhat large 247ms for visual stimuli [278]. Based on the mentioned values, it is suggested that the response time (from the moment the robot is aware of certain changes: stimuli detected by sensory receptors and transmitted to the brain, through central processing via brain interpreting and deciding on a response, to motor execution time coordinated by cerebellum) is below 200ms for better user experience.

The response time mentioned above can refer to the E2E latency in the context of this use case. Considering the Cerebellum is deployed on robots and the Brain on the network side (e.g. 6G CN, SHE), the E2E latency also includes the round-trip transmission latency between the robot and the Brain, the computing time in the Brain and the Cerebellum. When the Cerebellum and Brain are deployed together on the network side (e.g. 6G CN, SHE), the E2E latency calculation is similar.

The existing mechanism defined in the 3GPP is the 6G network only acts as a pipe responsible for traffic transmission and the application server performs data processing. Considering the low latency requirement for the computing service, it is difficult to guarantee the E2E latency performance requirements, e.g., the interaction task mentioned above which is about conversation responses to the users, which is normally around 200ms [276]. The communication resource and computing resource (i.e. 6G CN, or SHE) should be jointly managed and coordinated by 6G network, when providing computing service in order to support the E2E service performance (e.g., latency). 6G network is expected to control computing and communication resource to fulfil the E2E latency required by the computing task needed by the low-latency AI inference service.

### 6.49.2	Pre-conditions

Operator OZ has deployed in 6G network 6G/3GPP services for intelligent interactive robots focusing on supporting low E2E latency which partially includes the AI inference time of the 3rd party-provided VLM/LLMs (e.g. “Brain”) on the network side for the robot to handle complex tasks (incl. sophisticated reasoning, decision-making, activity-planning). The Cerebellum is deployed in the robot locally for local intelligence/quick reaction. By design, the Cerebellum in the robot can collaborate with the Brain in the network.

Bob purchased a robot designed for interactive casual sports. Bob also has a subscription to Operator OZ for comprehensive 6G/3GPP services for the robot. The robot is able to access to 6G service.

The Robot, as the user Bob’s playmate, can join in outdoor activities together.

### 6.49.3	Service Flows

Figure 6.49.3-1 an illustration of robot performing interactive outdoor activities, to show the E2E latency 6G system is expected to provide consists of the round-trip communication latency between Robot and the “Brain” (which can be a VLM hosted on the network side, e.g. 6G CN/SHE), as well as the latency of computing service.

NOTE: 	the interaction between user Bob and Robot is out of scope, what worth noting is Cerebellum may also further process information (e.g. instructions, actions) received from Brain, before performing e.g. precise motor actions.

- The robot activates its sensors (such as mic, camera) and is on standby for wake word. Bob speaks ‘Hey, let’s exercise!’ and the robot receives the voice.

- The robot sends the audio and the local environment information (e.g., playground) to the in-network Brain by the 3GPP network.

- The in-network Brain receives user prompt and the environment information, conducts the environment analysis based on the AI inference model and computing resources and obtain the response to Bob.

- The robot receives the response from the Brain and replies to Bob ‘Hello Bob! What activity shall we do today? Running, cycling, or stretching?’ The E2E latency in the service flow contains the steps from step 2 to step 4 which describes how the robot will respond to the user prompt.

- Bob selects one activity and speaks ‘Let’s go running’.

- The robot sends the user instruction and local environment information to the in-network Brain.

- The in-network Brain generates the initial task planning and it may contain the instruction: Guide the user to warm up for 5 minutes and the according the actions.

- The Cerebellum in the robot receives the initial task planning information and determines the discrete actions for the execution. The robot performs dynamic stretch and Bob follows the robot to imitate warm-up motions.

- The robot takes images in order to tracks Bob’s posture.

- The robot sends the images to the Brain to validate the posture. The Brain checks whether the motion is correct. If the motion is correct, the Brain may respond the audio ‘Perfect!  Keep going.’ If the motion is not correct, the Brain may respond the audio ‘Adjust your arms higher, Bob’ and provide the correct actions feedback.

- The Cerebellum determines the correction for the next execution based on the updated task planning information. The E2E latency in the service flow contains the steps from step 10 to step 11 which describes how the robot will react to the change in the step 10 based on interaction between Cerebellum and Brain. The interaction between the Cerebellum and Brain is to get the changed environment information so that the Brain can update the planning action and send to the Cerebellum for further discrete actions. Considering that the E2E latency refers to the human reaction time, step 10 describes the change of the environment and step 11 describes how the robot will react to the change. The duration between the steps can be the reaction time and it is included in the E2E latency. In order to fulfil the requirements of E2E latency, the network provides the communication resource and computing resource in order to guarantee computing service performance.

- The robot performs the demo correction to adjust Bob motion.

- The warm-up is completed. The robot and Bob start running.

### 6.49.4	Post-conditions

The robot activates mobility to match Bob’s speed and complete the running task.

### 6.49.5	Existing features partly or fully covering the use case functionality

5G system enhancements for edge computing, as defined in TS 23.548 [137].

5GC supports the following connectivity models to enable Edge Computing:

-	Distributed Anchor Point: For a PDU Session, the PSA UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed e.g. due to UE mobility and using SSC mode 2 or 3.

-	Session Breakout: A PDU Session has a PSA UPF in a central site (C-PSA UPF) and one or more PSA UPF in the local site (L-PSA UPF). The C-PSA UPF provides the IP Anchor Point when UL Classifier is used. The Edge Computing application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point mechanisms. The L-PSA UPF may be changed due to e.g. UE mobility.

-	Multiple PDU Sessions: Edge Computing applications use PDU Session(s) with a PSA UPF(s) in local site(s). The rest of applications use PDU Session(s) with PSA UPF(s) in the central site(s). Any PSA UPF may be changed due to e.g. UE mobility and using SSC mode 3 with multiple PDU Sessions.

5GC supports the Edge Application Server Discovery Function (EASDF), including the EASDF discovery and selection.

5GC supports Edge Computing in 5G System considering different connectivity models, including:

-	EAS discovery and re-discovery.

-	Edge relocation.

-	Network exposure to Edge Application Server.

-	Support of 3GPP application layer architecture defined in TS 23.558 [5].

-	Support of AF guidance to PCF determination of proper URSP rules.

The existing features defined in the 3GPP are about how to guarantee the communication service including the traffic routing path optimization. The computing service provided by the 6G network is not covered.

### 6.49.6	Potential New Requirements needed to support the use case

[PR 6.49.6-1] The 6G system shall be able to provide the communication and AI service (assuming AI inferencing in Service Hosting Environment) for user with the KPI requirements in the Table 6.49.6-1 below.

Table 6.49.6-1: KPIs for physical AI robot to perform cognitive functions for to interact with human user via 6G network

| Use case | Traffic type | Average packet size (Byte) | Transfer interval (ms) | Data Rate (Mbps) | Joint E2E latency (ms) (NOTE 4) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Support robot conscious awareness for interacting with human user | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [<200] (NOTE 3) (NOTE 5) | [99.9 %] |
| NOTE 1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE 2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE 3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making based on various stimuli. Such human brain reaction time ranges from mean auditory reaction time 140-160ms, touch 155ms, to visual reaction time 180-200ms [279]. NOTE 4: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment). and UE is only considered to contribute to the communication service latency. NOTE 5: The human to robot latency (vice versa) is not included. |  |  |  |  |  |  |

## 6.26	Use case on optimizing user experience for GenAI applications

### 6.26.1	Description

Applications based on generative AI (GenAI) capabilities are becoming popular recently, and as the research and development goes deeper, we see they are offering services from simpler text-based chatting to more complex multi-modal services like audio and video generation. Examples of these new applications and services include real-time translation, image or video recognition / visual assistant and real-life assistance (e.g. home doctor, cooking and game agent). Some of them are already commercialized and have the potential to be a main type of traffic in the future.

Therefore, it’s important to understand the traffic characteristics of these GenAI applications, so that the 6G network could optimize the service experience of their users, while at the same time improving the efficiency of the network when transmitting this traffic.

Figure 6.26.1-1: traffic characteristic of typical GenAI applications

The figure above shows three kinds of typical GenAI applications, visual assistant, text-to-image generation, and chatbot. Their characteristics are summarized as below:

Common characteristics:

- Burst traffic: high bandwidth requirement during a short time period can be noticed in both situations, while application #1 mainly for uplink and application #2 for downlink;

- Traffic encryption: not shown in the figure, but current GenAI applications usually support end-to-end encryption of the traffic, though it ensures the security and privacy of the user, it also makes the network harder to identify the traffic type and provide proper service quality assurance to these applications;

- Various connection design: applications are using different connection designs, for example, some applications have long-lived connection for each task/session, and some applications use different connections and even different source IPs for each request, and might support parallel transmission (e.g. in image generation case).

Application #1 Visual Assistant:

- Low transmission latency requirement: not shown explicitly in the figure, the real-time service requires low end-to-end latency. For example, according to these following analysis, good interaction experience requires a round-trip latency within 400 ms [176]. And because the current GenAI algorithm usually takes time to generate content (for example, GPT-4o is able to respond to audio inputs within at least 232 ms), the transmission latency boundary is quite limited (e.g. less than 70 ms) [177];

- Short-lived connection: also not shown explicitly, the image upload sets up a new connection each time.

Application #2 Text-to-image Generation:

- Unpredictable bandwidth requirement: the GenAI application supports multi-modal communication, and could transmit text, audio or video randomly based on user request, and therefore has no periodic traffic pattern;

- High reliability requirement: unlike audio or video processing which are tolerant to some packet loss, the “prompts” used to express user intent are required to be transmitted reliably for correct processing.

Application #3 Chatbot:

- Small packet length: Chatbot applications typically have very small packet length, including both data packets and ACK packets, which would lead to decrease of processing speed considering the current user plane design.

Table 6.26.1-1: KPIs comparison between GenAI application and short video application

| Application | Average packet size (Byte) | Uplink |  |  |  |  |  | Downlink |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average Data Rate (Mbps) | Packet Percentage | Traffic Percentage | Burst Data Rate (Mbps) | Burst Packet Percentage | Burst Traffic Percentage | Average Data Rate (Mbps) | Burst Data Rate (Mbps) |
| short video | 1144 | 0.007 | 17% | 1% | 0.23 | 17% | 1% | 0.91 | 24 |
| short video live streaming | 921 | 0.034 | 34.7% | 1.6% | 0.23 | 38% | 1.6% | 2 | 13.2 |
| visual assistant | 838 | 1 | 49% | 48% | 67.6 | 51% | 52% | 1.2 | 69.4 |
| text-to-image generation | 987 | 0.03 | 28% | 5.6% | 0.9 | 22% | 2.7% | 0.6 | 34.1 |
| chatbot | 87 | 0.008 | 48% | 22.7% | 0.08 | 48% | 31.8% | 0.11 | 0.14 |
| Note: meanings of KPIs are as follows average (uplink/downlink) data rate: average data rate of uplink/downlink traffic during the whole transmission (uplink) packet percentage: number of uplink packets divided by number of all packets during the whole transmission (uplink) traffic percentage: amount of uplink data traffic divided by amount of all data traffic during the whole transmission burst (uplink/downlink) data rate: average data rate of uplink/downlink traffic within a burst burst (uplink) packet percentage: average number of uplink packets within a burst divided by average number of packets within a burst burst (uplink) traffic percentage: average amount of uplink data traffic within a burst divided by average amount of data traffic within a burst |  |  |  |  |  |  |  |  |  |

Table 6.26.1-1 also shows a statistics example of KPIs of traditional video streaming applications and typical GenAI applications. It’s obvious that GenAI applications have quite different characteristics from traditional applications, for example, noticeably high burst traffic and uplink packet/traffic percentage (more interactive).

To better serve the GenAI applications and services, it’s proposed in this use case that several enhancements could be introduced into the 6G system, including:

- UE and network coordination: UE could help network to identify the application and traffic type more accurately and quickly, and the network could help UE make better flow control

- Better QoS assurance: to ensure the GenAI application’s performance and service reliability, the network need to be aware of the burst and provide assurance for the whole burst; to deal with short-lived connections, QoS configuration needs to be done in a shorter time period; provide end-to-end service assurance by choosing an optimized path to the application server; and make QoS adjustments according to payload type, real-time network status and user feedback

### 6.26.2	Pre-conditions

User Andy has subscribed to operator B’s services and uses B’s mobile network to access and utilize an AI image generation service provided by a GenAI service provider DrawYourPicture.

Operator B and GenAI service provider DrawYourPicture have a specific SLA in place that allows the Operator B to treat the traffic to/from DrawYourPicture in an optimised manner.

### 6.26.3	Services Flows

1.	Andy is writing a blog on his phone and would like to use DrawYourPicture’s mobile application service to generate an image as part of his blog.

2. 	DrawYourPicture’s service is e2e encrypted. To ensure the service quality, Andy’s phone coordinates the predicted service requirements to the network before sending the actual request, including for example application type (GenAI/Image generation), size and duration of the burst traffic, required latency, etc.

3.	After receiving these information, the network checks the SLA with DrawYourPicture and tries to prepare an optimized session for Andy. The network may choose an optimized path to DrawYourPicture’s application server and prepare and configure a proper QoS rule utilizing, for example, historical data, AI predictions.

4.	Andy sends some prompts to DrawYourPicture with the content of the blog.

5.	To ensure the service quality, the network needs to assure the whole packets related to the prompts are delivered through an optimized path for better performance and avoiding packet loss.

6.	DrawYourPicture’s application server receives the prompts and uses its GenAI algorithm to generate an image.

7.	The network prepares to send the generated image back to Andy, it coordinates with his phone about the workload status of the user plane and provides a recommended bandwidth to it.

8.	Andy checks the generated image and thinks it doesn’t reflect all his requirements. So he decides to send a sample image searched online to DrawYourPicture, to help it generate a better image.

9.	Before sending the image, Andy’s phone coordinates the new requirements and current QoE to the network, and the network updates the QoS rules and/or routing paths for this image upload accordingly. After that, the phone uploads the image to DrawYourPicture.

10.	Andy receives a newly generated image from DrawYourPicture and is satisfied with the result.

11. The network summarizes the whole service procedure and keeps the record for self-learning and self-optimization for providing connections to similar image generation applications in the future.

### 6.26.4	Post-conditions

Andy gets the ideal image for his blog post, and experiences an optimized service from operator B.

Operator B fulfils Andy’s service requirements associated with DrawYourPicture service and improves the network efficiency through coordination with Andy’s phone.

### 6.26.5	Existing features partly or fully covering the use case functionality

The paper [178] describes a feasible approach on how to identify encrypted traffic, which requires: 1) the initial packet’s SNI is not encrypted, and 2) need to have packets transmitted in X seconds’ time to help identification. This mechanism is not suitable for identifying GenAI applications which have burst type of traffic.

The current mobile network system supports QoS assurance mechanism. Although the current mechanism is implemented by exchanging signaling within control plane, and it usually takes a lot of signaling exchanges and introduces additional latency, which is not suitable for short-lived connections.

The current mobile network system supports measure QoE of the user, but the analysis is not too accurate because traffic statistics are used as the only approach.

### 6.26.6	Potential New Requirements needed to support the use case

[PR 6.26.6-1] 6G system shall provide efficient QoS assurance mechanisms for different types of application traffic, with the KPI summarized below:

Table 6.26.6-1: Required KPI for GenAI applications

| Traffic Type | Burst size | Max Allowed latency for a burst | Service bit rate: user-experienced data rate | Delay | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: max allowed latency for a burst: max latency for sending out the whole packets within a burst delay: transmission latency of a packet |  |  |  |  |  |

## 6.48	Use case on service robot for power grid

### 6.48.1	 Description

Service robot offers greater versatility by performing complex tasks that require dexterity, adaptability and interaction with human-centric environment. Thanks to major breakthroughs in artificial intelligence (AI), machine learning and advanced materials, service robots (e.g. humanoid robot) are poised for considerable growth over the next decade and present a massive investment opportunity.

According to a report released by Markets and Markets in January 2025, the global humanoid robot market is expected to grow from US$2.03 billion in 2024 to US$13.25 billion in 2029. Meanwhile, Goldman Sachs predicted in February 2024 that the total addressable market for humanoid robots could reach US$38 billion by 2035. Since service robots are more flexible and capable of adapting to complex terrain, they are particularly appealing for tasks that are “dangerous, dirty, and dull”, for example, mining, disaster rescue, grid inspection, nuclear reactor maintenance, and chemicals manufacturing, etc. Customers are willing to pay a higher price for robots that can do dangerous jobs that people are reluctant to do.

For example, the service robots can be deployed in power grids, enabling inspection, maintenance and emergency response operations in extreme environment. As shown in Figure 1, it is the high-altitude ultra-high voltage AC project in southwest China, which covers 1316 kilometers of 1000kV lines and four 1000kV substations. With the high altitude (e.g. 3450m) and mountainous terrain, it is hard for human technicians to go to the site. The service robots (e.g. humanoid robot) may mimic human movements and navigate the existing power grid infrastructures (e.g. narrow stairs, climb ladders), which are expected to play an important role in power grid inspection. The service robots can tolerate the hazards and continue working with the hand-operated tools, which enhances workers’ safety while ensuring service continuity.

Figure 6.48.1-1 The Ultra-high voltage line and 1000kV substation in southwest China

On the other hand, different levels of robot autonomy and intelligence [272] have been developed. For the robots with higher level of autonomy, they are growing fast with the utilization of large AI models (e.g. LLM, VLM, SLAM). With service robot as an example, the motion control such as walking, grasping, turning, balance, posture control, reflexes and corrective actions, can be supported via embedded controller and chips for fast response. However, the large AI models for advanced autonomy are too heavy to run efficiently on embedded hardware of service robot. Offloading the computing-intensive “brain” of service robot to nearby 6G network (e.g. service hosting environment) can enable lightweight service robots with limited onboard power. To support this, the service robot needs to send multi-modal data (e.g. sensor data, LiDAR, video, voice, etc) to the 6G network. These multi-modal data can then be processed via the large AI model to generate robot control commands and natural language responses appropriately. With the support of pervasive computing capability in 6G network, the service robot for power grid inspection can respond instantly to complex situations with real-time perception-action loops as shown in Figure 2.

Figure 6.48.1-2 Illustration of service robot for power grid

### 6.48.2	 Pre-conditions

PowerGrid Corp, a leading energy provider, manages over 300,000 km of transmission lines, 600 substations, and remote renewable energy plants across mountains, forests, and coastal zones. Due to climate change, PowerGrid Corp suffers from rising maintenance costs and dangerous inspection conditions. In order to improve operation efficiency, they purchase 500 service robots for grid inspection, maintenance and emergency response.

The 6G network is operated by operator A. Operator A is responsible for the communication service provision around the power grid area. Moreover, Operator A owns sufficient computing power and support edge intelligence. The PowerGrid Corp subscribe the communication service and AI service from Operator A which provide low latency communication service as well as the large AI model inference service via service hosting environment for service robots in the field.

One day, a massive high-voltage power grid runs across snowy peaks and rugged valleys. Ice storms, falling rocks, and wildlife make this terrain not only remote but dangerous for human technicians. To ensure uninterrupted power delivery to millions of people, an service robot named Voltus is deployed to inspect, maintain, and report anomalies in the grid infrastructure.

### 6.48.3	 Service Flows

- Voltus is airlifted by drone to a remote grid substation. Upon landing, it connects to the 6G terrestrial network for communication. Meanwhile, it activates its simultaneous localization and mapping system to map the area using LiDAR and vision.

- Voltus walks along the grid lines. A human technician in intelligent control center connects remotely and speaks to the service robot:“Voltus, inspect the substation.”

- Voltus uses thermal imaging to detect overheating in transformers. Meanwhile, it takes video of the substation. The multi-modal data (e.g. the thermal image, video stream, sensor data and LiDAR data) are sent to the service hosting environment within 6G network.

- Upon receiving the multi-modal data, the video recognition model in service hosting environment analyzes the input data and detects objects based on the visual and thermal patterns match.

- On top of that, the large AI models (e.g. LLM, VLM, SLAM) in service hosting environment perform reasoning and planning, and generates the robot control commands and natural language responses. For example, the LLM generate the analysis report: two insulators show thermal anomalies and one grounding wire is partially detached. The LLM in service hosting environment generates robot control commands: 1) tighten the loosened bolt using a multi-tool; 2) replace the two insulators.

- The analysis report and robot control command are sent to Voltus. Based on the command, Voltus’s embedded system performs motion control, which executes walking, balancing and climbing a small pylon. Then Voltus replaces the cracked ceramic insulators from its utility belt.

- When tightening the bolt, Voltus further queries the large AI model in service hosting environment:“What torque should be applied to a 500kV line bolt with rust mitigation coating?” The large AI model in service hosting environment replies:“Apply 45 Nm torque using non-conductive wrench. Confirm line de-energized.” Upon receiving the reply from large AI model in service hosting environment, Voltus tightens the loosened bolt using a hand-operated tool accordingly.

- After a while, a human technician in intelligent control center speaks to Voltus:“Voltus, confirm insulator replacement, send close-up.”

- Voltus uses speech recognition and sends the annotated images to the human technician. Moreover, Voltus responses to the human technician using speech :“Insulator integrity verified. ”

- Upon receiving the responses from Voltus, the human technician checks the images and speaks to the Voltus robot:“Repair successful. Recommend re-inspection in 2 weeks.”

- Upon completion of the substation inspection, Voltus can be airlifted by drone to another grid substation for inspection.

### 6.48.4	 Post-conditions

Due to the service robots based grid inspection and maintenance, long term cost saving and higher operational efficiency can be achieved for the power grid.

### 6.48.5		Existing features partly or fully covering the use case functionality

| Specifications and clause | Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TS 22.104 [64] clause A.2.2.3 | A mobile robot essentially is a programmable machine able to execute multiple operations, following programmed paths to fulfil a large variety of tasks. This means, a mobile robot can perform activities like assistance in work steps, collaboration with other robots, e.g. for car assembly, and transport of goods, materials and other objects. Mobile robot systems are characterised by a maximum flexibility in mobility relative to the environment, with a certain level of autonomy and perception ability, i.e., they can sense and react with their environment. Table A.2.2.3-1: Service performance requirements for mobile robots | Compared with the mobile robot in 5G system, the service robot in 6G system is able to perform complex tasks with the LLM assistance in service hosting environment. |

### 6.48.6	Potential New Requirements needed to support the use case

[PR 6.48.6-1]	The 6G system shall be able to provide the communication and AI service for user with the KPI requirements summarized below:

Table 6.48.6-1: Proposed KPIs for use case on service robot

| Use case | Traffic type | Message/Frame size(Byte) | Transfer interval(ms) | Data rate(Mbps) | E2E latency(ms) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Service robot | UL sensor data (NOTE1) | 1250-12500 | 10 | 1-10 | 100-150ms (NOTE4) | 99.99% |
|  | UL LiDAR | 28800 | 100 | 27.6 (NOTE3) |  |  |
|  | DL Control command (NOTE2) | 625-12500 | 50 | 0.1-2 |  |  |
| NOTE1: refers to the kinematic state, environment perception, manipulation status info except LiDAR to be sent from the service robot to the network to enable effective motion planning, object interaction and navigation. NOTE2: refers to the control command towards service robot, e.g. high-level task, action plans, motion strategy, gripper command, etc. NOTE3: the data rate of LiDAR is based on frame rate 10Hz, 28800 points/frame, 12byte for one point cloud, i.e 28800*16*8*10=27.6Mbps. NOTE4: E2E latency includes two parts: the round-trip latency for communication service and the latency for AI inference within the service hosting environment. The typical robot control loops requires 100-150ms latency [273] for AI inference, communication and control. For example, the communication may take about 40ms while the AI inference may take about 100ms [274] for the service robot. |  |  |  |  |  |  |

## 6.38	Use case on AI for disability support

### 6.38.1	Description

As AI evolves, it will further improve human communication, enabling proactive communication with people, devices, and surroundings. This includes hearing aids with complete noise cancellation and the ability to tune in on specific sounds, visual aids enabling recognition beyond normal vision, and communication aids using eye movements or brain interfaces. This can have a revolutionary enabling effect for people with various disabilities and people in challenging situations where typical human capabilities are insufficient, allowing them to recuperate some of the impaired capabilities through the aid of AI-powered tools.

AI enhanced communication can be about person-to-person communication, for example a phone/video call, or person-to-AI function communication, where the AI function/agent assists the person in orientation, understanding, and information processing.

AI enhanced communication can also be applicable to IMS emergency communications when a user with disabilities communicates with a PSAP.

Sustainability impact analysis

Use of AI for disability support is valuable and has benefits within health & human wellbeing area. However, challenges regarding trustworthiness and energy needs to be considered

Energy resources

From the environmental perspective, energy-efficient AI models and increased reliance on renewable energy sources could help minimize the risks. Sources of electricity for AI devices and operations is an important factor as it could impact emissions to air, water and soils.

Trustworthiness

Ethical challenges include determining responsibility - for instance, assessing the impact and entity accountable in the event of technology malfunctions or if developers or providers decide to discontinue support. Determining the appropriate level of control over current and future technology functionality, given the technology’s intimate nature. Ensuring the technology’s value alignment, particularly concerning privacy and integrity as the technology is likely to collect, analyze, and store highly sensitive user data. Other issues in terms of value alignment are mitigating bias, ensuring that the technology works for everyone, and identifying the beneficiaries of the technology (e.g. those with the greatest need versus those with the greatest ability to pay). In evaluating trustworthiness, several aspects must be considered. These include the functionality, reliability, and benevolence (friendliness) of the technology itself, as well as external factors such as the trustworthiness of developers and providers, business models, data management practices and adherence to policy and regulations

### 6.38.2	Pre-conditions

The following pre-conditions are considered:

- A user has impaired vision, hearing, mobility, or orientation capacity, due to either disabilities or environmental factors that affect bodily functions.

- The user is equipped with a device capable of sending video and audio streams, such as smart glasses with camera and microphone. The device is also capable of receiving audio streams and optionally video streams.

### 6.38.3	Service Flows

The following service flow is considered, involving an application that can be part of the network (e.g. as an IMS application) or external:

- Immersive video (RGB and depth) and audio stream is captured on user device.

- In an application, the video and audio streams are analysed for user input (cues, intents)

- User gesturing, audio cues, eye tracking, and field-of-view tracking (what the user is looking at).

- In an application, the video stream is further analyzed, and situational awareness is established.

- Based on audio, data input, or visual cues from the user, the application adds contextual information to the visual and/or audio stream or modifies the incoming audio stream. This may include:

  - Scene understanding: answering questions “What am I looking at?”, “Explain what is happening”. Adding digital tags in field-of-view.

  - Orientation: answering questions “Where am I?”, “How do I go to X?”. Adding digital arrows in field of view.

  - Visual aid: text in field-of-view read out in audio.

  - Audio aid: amplification, enhancing contrast, simplification or translation of audio stream.

  - Communication aid: assistance in communication with others, e.g. through translating user intents (gestures, eye movements, field-of-view tracking, etc.) to audio stream.

  - Reminders: assistance in remembering appointments etc., e.g. through audio or digital messages in field-of-view.

- A virtual overlay of contextual and modified information, in the form of video and/or audio streams, is transmitted to user device.

The above service flow can also be applicable to IMS emergency communications service with a PSAP.

### 6.38.4	Post-conditions

Users get orientation, scene understanding, and adaptive communication support from the application through their devices and can thereby become more independent and capable.

### 6.38.5	Existing features partly or fully covering the use case functionality

Some forms of assistance can be implemented as an over-the-top service, on top of the existing mobile broadband connections. However, in the next subsection this contribution analyses the requirements for arrangements where the network involvement provides an enhanced service.

### 6.38.6     Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.38.6-1] Based on operator policy and user consent, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support.

NOTE 1:	The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e., enhancements, clarifications, translations, speech to text, text to speech, etc), visual aid (modification of the visual stream).

[PR 6.38.6-2] Based on operator policy, user consent and regulatory requirements, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support.

[PR 6.38.6-3] The following KPIs in Table 6.38.6-1 should be supported, in Dense Urban, Urban, and Rural indoor/outdoor scenario:

Table 6.38.6-1: Performance requirements

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application round trip latency | Communication service availability | Overall user density | Activity factor | UE speed |
| AI for Disability support | [1-2] Mbit/s | [50-100] Mbit/s (note 1) | [300-1000] ms (note 2) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (note 3) | [50]% | 5 km/h |
| Note 1: envisioned 4K resolution Note 2: Input stream (video, audio, data) to output stream (video, audio, data) 300-1000 ms dependent on task including the disability application Note 3: It is assumed that 10% of all users are using AI for Disability support |  |  |  |  |  |  |  |

Editor’s Note The value of Application round trip latency are FFS

Editor’s Note The value of Bit rate up link are FFS

* * * End of annex * * *