3GPP TSG-SA WG1 Meeting #112                           S1-254163

17-21 November 2025, Dallas, USA               (revision of S1-25xxxx)

Title:	proposal for CPRs of Ubiquitous connectivity (Y.1.11)

Agenda Item:	8.1.10

Source:	ZTE, CSCN

Contact:	XuLing xu.ling@zte.com.cn

Abstract: it proposes the ubiquitous connectivity related CPRs for Clause Y.1.11 based on the Rapporteurs draft (S1-254020).

Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):s upport for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

NEW Editor’s Note: PR 8.2.6-1 and PR 8.17.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.11-1 at this time.

NEW Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.11-1 – NTN /satellite accesscommunication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite. | PR 8.3.6-1 PR 8.3.6-3 | LEO Coverage Flex configs |
|  | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity [ZTE] merged into PR8.4.6-1 |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity with minimum interruption for UEs within NTN, between TN and NTN, and within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios, during the transition. | PR 8.4.6-1 PR 8.8.6-1 PR 8.3.6-2 | Service continuity TN-NTN,NTN,LEO |
|  | The 6G system with satellite access, shall support SMS delivery to a density of UEs i.e. up to [1000] UE per km2. | PR 8.4.6-2 | KPI (Y.2) high density # of UEs SMS [ZTE]It is suggested to merged into UC KPI table 8.x.2-1 |
| Y.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS [62] for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 | PWS Text Messaging Adverse propagation conditions |
| Y.1.11-1-4 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes |
| Y.1.11-1-5 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-UE direct communications |
| Y.1.11-1-6 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching |
| Y.1.11-1-7 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-1-8 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info |
| Y.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) to minimise the necessary bandwidth of the inter-satellite and feeder links, and provide a computing service to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing [ZTE] It is suggested to seperate which are targeting different aspects. |
| Y.1.11-1-10 | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) , provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 |  |
| Y.1.11-1-11 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 |  |
| Y.1.11-1-12 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions. | PR 8.18.6-1 | QoS Improvement [ZTE] remove the examples which are solution oriented. |
| Y.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service to UE mounted on SWAP(Size, Weight and Power) constrained moving platforms (e.g. flying drone, small vessels mounted) with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB |
|  | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP(Size, Weight and Power) constrained moving platforms (e.g. flying drone, small vessels mounted) with the following KPIs in Table 8.6.6-1 in TR22.870]. | PR 8.6.6-1 | Embb [ZTE]It is suggested to merge it with KPI table 8.6.6-1 |
| Y.1.11-1-15 | The KPIs in Table 8.8.6-1: KPIs for global mobile video should be met for global mobile video service. | PR 8.8.6-2 | KPI Global mobile video service |

Table Y.1.11-2 – NTN-based Positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | Subject to regulatory requirements, operator policy, and user consent, the 6G system with satellite access shall be able to determine and provide the UE location meeting the regulatory requirements for both sparse and dense satellite constellations. | PR 8.3.6-4 PR 8.5.6-2 | UE Location Regulatory Support |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained devices to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of NTN and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| Y.1.11-2-4 | The 6G system with terrestrial and satellite access shall be able to provide integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity |
| Y.1.11-2-5 | The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the KPIs in Table 8.5.6-1: Performance requirements for satellite-based positioning services | PR 8.5.6-1 | KPI |
| Y.1.11-2-6 | The 6G system with satellite access shall be able to provide positioning service following the KPIs in Table 8.7.6-1: Performance requirements for satellite-based positioning services for energy constrained devices from [338] | PR 8.7.6-2 | KPI Energy constrained devices |
| Y.1.11-2-7 | The 6G system with terrestrial and satellite access shall be able to provide positioning service for UE type e.g. Airplane Mounted and UAV Mounted with high availability, by using 3GPP positioning technologies and fulfilling the KPIs in Table 8.10.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies. | PR 8.10.6-1 | KPI |
| Y.1.11-2-8 | The 6G system with satellite access shall be able to provide positioning service with the KPIs from Table 8.11.6-1: Performance requirements for positioning services with satellite access | PR 8.11.6-2 | KPI |