3GPP TSG SA WG 1 Meeting #112 	S1-254175

17-21 November 2025, Dallas, Texas, USA

Source:	SoftBank Corp.

pCR Title:	Resubmission Explanation of High Altitude Platform Station (HAPS)

Draft Spec:	3GPP TR 22.870 V0.4.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Shiro Fukumoto (shiro.fukumoto01@g.softbank.co.jp)

Abstract: Following a suggestion from the SA1#110 meeting, a proposal was submitted at SA1#111 to add an explanation of High Altitude Platform Stations (HAPS) to an Annex in TR 22.870 for facilitating the study of 6G use cases and potential requirements. During the meeting, the content for this proposal was stabilized in document S1-253571. However, the document was noted due to a request to also consolidate other NTN solutions into the Annex. This contribution resubmits the identical content from S1-253571.

1. Introduction

Summary of the fundamentals of HAPS to facilitate the study of their use cases and potential requirements for 6G

2. Reason for Change

Adding the text proposals in an Annex part.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.4.1.

* * * First Change * * * *

Annex X (Informative): 
High Altitude Platform Station (HAPS)

X.1	HAPS Fundamentals

High Altitude Platform Station (HAPS) is defined by the International Telecommunications Union (ITU) as "a station on an object at an altitude of 20 to 50 km and at a specified, nominal, fixed point relative to the Earth." This operational altitude, within the stratosphere, allows a single platform to provide wide-area coverage, making it an ideal solution to complement traditional terrestrial networks, especially in remote and rural areas where people still lack reliable internet.

HAPS platforms are not a single technology but a system of systems. These systems are broadly divided into (a) Aviation Systems, which cover the flight vehicle, energy, and navigation, and (b) Service Systems, which include the communications payload and its connection to the core network.  The most relevant aspects lie within the service systems, where familiar architectural principles are adapted for a stratospheric environment.

Overcoming gravity is a primary challenge, unlike for ground-based or satellite networks. Stratospheric flight is achieved through different types of flight vehicle systems, which can be broadly categorized into Heavier-than-Air (HTA) and Lighter-than-Air (LTA) platforms.

For HTA, these platforms function like conventional aircraft, using lateral velocity to generate lift. They offer excellent manoeuvrability and station-keeping capabilities. However, they are constrained by the high energy demand required to maintain flight and by limitations on payload weight.

In LTA, balloons are buoyant platforms that use lift gases, such as helium, to stay aloft. This design provides significant advantages in energy efficiency and payload capacity. Their main trade-offs are reduced manoeuvrability and station-keeping ability, although modern stratospheric balloons can achieve considerable navigation control by leveraging wind patterns at different altitudes.  Airships combine the advantages of buoyancy from being lighter than air with the navigational benefits of lateral propulsion.

X.2	HAPS in the communication ecosystem: Comparison with satellites

HAPS will be operating in the stratosphere at approximately 20 km altitude, that is much lower than satellites such as Low Earth Orbit (LEO) at 500+ km and Geostationary (GEO) at over 35,000 km. The fundamental difference is the propagation distance. A HAPS service link of 20 km is comparable to long-distance links in many terrestrial mobile networks. This proximity to Earth, combined with the favourable propagation conditions of an earth-sky link (free from ground clutter), is the key to its performance.

This translates mobile network performance compared to satellite-based direct-to-device (D2D) solutions as summarized in Table A.2-1:

Table A.2-1: Relative characteristics of GEO, LEO and HAPS

|  | GEO Satellite | LEO Satellite | HAPS |
| --- | --- | --- | --- |
| Altitude (km) | 35 786 | 300–1 200 | ≈ 20 |
| Typical RTT (ms) | ≥ 600 | 30–50 | < 10 |
| Coverage radius (km) | ≈ 300 | 20–50 | 50–100 |
| User terminal | Dedicated handset | NTN capable handset Standard Cellular handset (some satellite systems) | Standard cellular handset |

* * * End of Change * * * *