3GPP TSG-SA WG1 Meeting #112	S1-254224

17-21 November 2025, Dallas, Texas, USA            	(revision of S1-253605)

Source:	     Huawei, HiSilicon, China Mobile, KPN, China Telecom, Turkcell, OPPO, NEC, Turk Telekom

New use case title:	New use case on 6G provide companion robot on-demand customized services

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.3.2

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

Abstract: This contribution proposes a new use case to 3GPP TR 22.870 clause 6, on 6G providing companion AI robot on-demand customized services with quality assurance.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

<Skip>

[257]	3GPP TS 28.538: “Management and orchestration; Edge Computing Management”.

[6.x1]	SWS-240002, "GSMA Proposal on 6G", 3GPP Stage 1 Workshop on IMT2030 Use Cases

[6.x2]	The Growth of One-Person Households, https://www.globalissues.org/news/2025

* * * Second Change (new text) * * * *

## 6.x	Use case on providing on-demand customized services with quality assurance

### 6.x.1	Description

After years of planning, 5G became a reality in 2019, and as of January 2024, globally 261 operators had launched commercial 5G mobile services in 101 countries [6.x1]. 6G network is required to support various capabilities such as integrated sensing and communication (ISAC, e.g., range/ velocity/ angle estimation, localization, imaging), AI (e.g., AI model training and inferencing, distributed data processing), communication, etc. This requires the 6G system to be able to provide customized services on-demand with quality assurance to satisfy diversified service requirements. During the runtime of these customized service networks, 6G network is required to adapt to dynamic changes of multiple factors (e.g. the connection state of a user device, the computing load of MEC, the traffic load of network nodes change, etc) and feedback on the service from the user (provided either at the time service is running or afterwards) and promptly to guarantee user experience.

The use case is provided in the context of interactive companionship by AI robots (incl. daily leisure or outdoor activities). By 2018, the share of one-person households worldwide was estimated at 28%. Looking ahead, this figure is projected to reach 35% by the mid-21st century. The global share of one-person households is projected to reach around 35% by the middle of the 21st century [6.x2]. The field of companion AI robots is experiencing a noticeable growth due to several factors such as ongoing advancements in robotics and artificial intelligence, and the rising demand for social companionship. Such AI robots significantly enhance users’ physical activity adherence, engagement, and exercise duration, and they especially motivate users to be more active in their outdoor routines. The social interaction also helps improve users’ quality of life (e.g. reducing loneliness, stress, improving mood), and when desired provide conveniences by performing simple forms of daily-life assistance.

### 6.x.2	Pre-conditions

1. The user Tanguy, who is an adult living alone in Paris, purchased an intelligent AI-embedded companion robot to enhance engagement, emotional well-being, and physical activity. The robot accompanies Tanguy in both leisure and playful activities that foster outdoor engagement, such like stroll in the city/park, and games such as toy retrieval.

2. Tanguy has a subscription with Operator “Naranja” for obtaining customized 3GPP services. The UE in the AI robot can be uniquely identified by Naranja’s 6G network (e.g., with 3GPP identifiers), and can access 3GPP/6G services.

3. Tanguy pays a monthly minimum subscription fee, which grants the access to the “Naranja” 6G network, maintaining account status, and providing access to the customized 3GPP services offered. An additional fee applies for actual exact consumption of the customized 3GPP services (usually consisting of flexible and dynamic combination of multiple 3GPP/6G services, to satisfy Tanguy’s exact customized needs). This enables the Operator to benefit from predictable revenue from monthly fees alongside potential incremental income from usage fees – this is particularly interesting because due to the nature of the versatility of AI robot and its companionship scenarios, the actual 3GPP/6G services consumption of Tanguy is hard to be predicted. In the meantime, this also allows user Tanguy to enjoy flexibility by paying only a minimum fee with freedom to control costs through managing the actual usage on a monthly basis.

### 6.x.3	Service Flows

- The AI robot is switched on, registered to the 6G network, and can access 6G network’s services via default connection.

- Tanguy has been working remotely, and wants to get some air in a peri-urban park located on the edge or periphery of Paris, taking the AI robot for providing entertainment such as storytelling, music, or games, and for helping keep Tanguy socially engaged and entertained. Depending on the situation, the robot might assist with health monitoring, or guide Tanguy during the walk (e.g., in case the user is in a wheelchair).

- Tanguy requests customized service from Naranja’s 6G network, via an intent expressed in natural language, sent over the UE in the AI robot. The intent includes two main objectives so that his AI robot can navigate them to the park and back home. Additionally, Tanguy is expecting a parcel delivered to his home. The delivery requires Tanguy’s signature. Tanguy is worried that the delivery person may arrive while he is outside. Therefore, he wants to keep the stroll well within the available time before the delivery appointment (P.S. This could also be a scheduled Zoom meeting with his business partners). He also needs to know the updated expected time of return (“ETR”) to his home, so that he can call the delivery person and let him know of the ETR, if needed. The intent includes the main objectives:

  - the need to navigate around uneven terrain such as dense forest floors with roots, rocks, and thick vegetation (this could be more important for users in wheelchairs or any other preferences for the planned routes),

  - the need to timely compute the ETR for Tanguy, and the need to inform the delivery person of any update of ETR (The ETR is bound to change due to varying conditions in the area, and the dynamically updated routes).

The intent sent by the customer to the 3GPP network may look like the following: “Plan a leisure stroll path from <Address or the GPS coordinates of home> to <name or the GPS coordinates of the park> that avoids obstacles and difficult terrains. Send the planned path to my robot. I plan to spend around 45mins in the park. Send my robot updated ETR in real-time.”

- The 6G network receives the intent via the UE on the AI robot. The 6G network parses the request (e.g., using an AI Agent), identifying the distinct needs for navigation and communication services. The network maps these needs into specific 3GPP services such as environment sensing, AI processing, dynamic path planning, and alert events. In this process, the 6GC reasons and self-reflect to decide and plan the optimal services to use and their interconnectivity.

- The 6G network with AI mechanisms (e.g., AI Agents in the 6G network) arranges and interconnects multiple 3GPP/6G services for providing the requested customized service. For example, the 6GS may configure the following four capabilities and interconnects them in that order.

  - The environment sensing capability (e.g., ISAC) for generating a 3D map of the general area between the customer’s current location and the destination (Parisian peri-urban park).

  - Combined with ISAC, the 6G network’s AI capability for detecting potential obstacles in the city, e.g., places with steps, broken or blocked sidewalks, areas with heavy foot traffic. When inside the peri-urban park, ISAC with 6G network’s AI capability can detect uneven terrains, Unpaved trails, or dirt paths, rocks, and thick vegetation.

  - The path planning capability for planning the optimal path that avoids all identified obstacles from customer’s location to the park, inside the large varying terrains of the peri-urban park, and back home.

  - Depending on Tanguy’s location and currently planned route (incl. inside the park and back home), ETR is calculated and updated. This ensures Tangy can truly relax and engage with the robot without worrying about missing the appointment with the delivery person.

The 6GS configures the four capabilities to operate continuously until the service is terminated. For example, any changes in the initial sensing data detected by the environmental sensing capability are promptly relayed to the AI capabilities in the 6G network, and then to path planning capability, enabling real-time (updates to) navigation of Tanguy’s stroll, enabled by 6G network’s customized on-demand service.

- The 6G network autonomously provisions the customized combination of 3GPP services. This includes instantiating and configuring network function instances, setting up inter-connections (e.g., communication tunnels), deciding the QoS requirements, particular sensing resolution, etc. All of this is performed on the fly, autonomously by the AI capabilities in the intelligent 6G network.

- For each service, the 6G network may check any previously retained context, using the past findings (e.g., data looked up and memorized, knowledge built from previous interaction with the customer, or decisions made in similar situations).

- Now 6G network is ready to provide the customized 3GPP services to the UE for this afternoon leisure stroll. Via the communication service, 6G network provides the planned path and any subsequent updates directly to the robot, in real time. During the leisure stroll, the 6G network continuously controls the execution of the customized 3GPP service, and manages the coordination of various capabilities and related resource optimization in the runtime to ensure the intended QoS of the services are assured. This includes updated path fulfils Tanguy’s expressed preference, and updated path always meets the expected time of return.

- When the customer returns home after completing the journey, the 6G network verifies with the customer and releases the 3GPP services and the resources.

- The customer provides feedback on the various aspects of the service provided by the 6GS (e.g., different aspects of the QoS).

### 6.x.4	Post-conditions

The customer returns home after visiting the peri-urban park at a time close to the ETR provided by the 6GS, meets with the delivery person for the parcel (or to join his scheduled conference call on time).

### 6.x.5	Existing features partly or fully covering the use case functionality

1. For 6G, the inter-dependency of resources related to providing various constituent 3GPP services (used synergistically and simultaneously) for a particular type of usage scenario/applications has not been covered, considering the resource efficiency and optimization at runtime adapting to changing network environment and unforeseen circumstances.

2. To provide communication services, traditional method based on predefined templates and fixed service flows have limitations regarding inflexibility, high threshold to use (e.g., requirements for expertized knowledge of detailed 3GPP network functions and corresponding APIs), etc. Besides, during the runtime when various constituent 3GPP services (communication, sensing, AI-related services such as UE-network collaborative inferencing) are collaboratively needed for customer’s certain set of objectives, 6G network needs to guarantee the overall SLA by autonomously adjust and optimize all aspects of the 3GPP services and capabilities in use. Existing optimization algorithms are designed on a single case-basis (e.g. for a particular type of communication service: eMBB or URLLC or mMTC).

3. 3GPP TS 22.261 Clause 6.40 and Clause 7.10 specify the communication services with performance requirements for AIML model transfer in 5G System. It does not cover the case that 3GPP network is involved in performing an AIML inferencing, which would need the 3GPP system to provide joint latency (considering communication latency and AI inference latency, which are both under the control of 3GPP network) for the intended application.

4. Rel-18, 3GPP TR 28.912 “Study on enhanced intent-driven management services for mobile networks” and 3GPP TS 28.312 “Intent driven Management Service for Mobile Network phase 2” define features of Intent Management Network (IMN) which allows subscribers to express their network-oriented intents (typically human-readable) about, e.g., delivery of a specific communication service or desired properties of the network. TS 28.312 has the following definition:

intent: expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them.

TS 28.312 clause 4.1.3 (“intent expectations for different types of management needs”) clarifies intent expectation is for “delivering network and service related object (e.g. network, service, slice)” and for “network and service related object (e.g. network, service, slice) performance”. Clause 4.5.1 (“intent expectation”) further clarifies the following:

The object may be a 3GPP managed object like a network slice, subnetwork (e.g. radio network) or other objects like a service.

5. Built on Rel-17 SEAL (Service Enabler Architecture Layer, Network Slice Capability Enablement (NSCE) in TS 23.435 (WID: NSCALE) provides some slice APIs for third-party application providers or verticals to use the network slices. The NSCE client can interact with the NSCE server to trigger network slice-related operations (e.g. S-NSSAI, DNN adaptation) in response to changes of application requirements. The NSCE server can provide guidance to the 5G network to update and modify the S-NSSAIs and DNNs of the route selection descriptors of the URSP rules for related application traffic, which adjusts QoS. In inter-PLMN scenarios, NSCE server checks with the OAM and/or 5G network whether the serving slice is available to offer the same performance at the VPLMN and make slice modification decisions when possible. This may allow a third-party service provider to request and manage network slices tailored to specific application needs. But for mass consumers, it seems a burden to require them to request/use network slices via the API framework (assuming their knowledge in 3GPP and slice template/attributes or implementation details of network slice), just because their applications (for a specific user, a given application on his/her UE) need customized 3GPP services to be efficiently provided by the 3GPP network. 3GPP should enable operators to monetize their 6G network investment by enabling 3GPP network to intelligently understand user intent and autonomously provide what is needed to fulfil the intent-based 3GPP service request. As this use case focuses on what the 3GPP network should to do when the intent-based 3GPP service request is received by the network, the service requirements on 6G network are captured (details in the proposed potential requirements).

NOTE: 	This applies to various scenarios not limited to this use case. E.g. 3GPP subscribers using immersive technologies (e.g. MR glasses) may need simultaneously various 3GPP services (communication, 3GPP UE-network collaborative rendering/AI inferencing, 3GPP sensing) for a specific immersive application. The relevant resources of these constituent 3GPP services which are inter-dependent need to be properly managed in runtime for guarantee QoS/SLA (quality assurance).

6. For network slice optimization, in TS 23.435, a vertical application pre-configures the policy for the NSCE, and the NSCE subscribes to the trigger events from the 5G network and management system. When an event/instance occurs while the user is using services via a network slice, the NSCE coordinates with the 5G network and management system to modify the network slice intending to adapt to the event.

7. For the customized services, traditional method based on predefined templates and fixed service flows has some limitations such as the inflexibility, high threshold to use (e.g., requirements for expertized knowledge of detailed 3GPP network functions and corresponding APIs), etc. In addition, traditional optimization algorithms are designed case by case, it is difficult to use a unified algorithm to optimize various kinds of objectives, which are also hard to optimize multiple factors simultaneously due to the complexity. AI Agent has powerful capabilities of environment perception, taking actions autonomously for specific tasks, self-optimization and exploring optimal solutions based on large-scale model. Therefore, it is necessary to study issues and identify requirements regarding AI Agent in 6G network to perform customized service network design and runtime control. The Benefits of using AI Agent based customized service network provision will be:

- Allowing creation of customized service network without expertized knowledge. Users can request for the customized service network through intent while AI Agent in 6G network will interpret the intent, translate the intent to the service and capability requirements for the network and generate the customized network.

- Enabling autonomous control/coordination of customized service network adapting to dynamic changes of multiple factors of network environments, and finding good adjust schemes for them to satisfy the various optimization objectives.

### 6.x.6	Potential New Requirements needed to support the use case

NOTE 1: 	These potential requirements can also be considered for further enhancing network slicing

NOTE x: The mention of AI capabilities such as AI agent doesn’t imply or preclude any architecture assumption or solutions.

[PR 6.x.6-1] The 6G system shall support mechanisms (e.g. AI mechanisms such as AI Agents) in the 6G network to provide on-demand customized services, which includes coordination of multiple 6G services (e.g. communication, sensing, AI service).

NOTE 1a:  6G network provides services based on capabilities of AI Agents: to reason based on the received service request (e.g., in natural language expressing intended objectives) through self-reflection, utilize tools (e.g. accessing URLs or documents to obtain contextual information, application context), access memories (e.g. historical task planning in similar service request context), etc.

[PR 6.x.6-2] Based on operator policy, the 6G network shall support AI mechanisms (e.g. AI Agents) to enable users (e.g. subscribers, a trusted 3rd party) to request the customized services (to meet user needs), which comprise dynamically and flexibly combined various 3GPP/6G services (e.g. communication related services, beyond communication service, and capabilities), based on Intent.

NOTE 2: The AI Agent in the 6G network can use LLMs, and can be fine-tuned to better support a particular type of applications.

[PR 6.x.6-3] The 6G system shall support mechanisms (e.g. AI mechanisms such as AI Agents) in 6G network to efficiently assure the performance of the customized service (comprising various 6G services) provided to a user according to network conditions/dynamics and user feedback.

* * * End of Second Change * * * *