3GPP TSG-SA WG1 Meeting #112 	S1-254279r1

Dallas, USA, 17-21 November 2025	(revision of S1-254260, 3220r3)

Source:	Apple, CableLabs, MediaTek, AT&T, vivo, OPPO

pCR Title:	New Use Case on Personal Data Usage

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.2.2 System and Operation Aspects

Document for:	Approval

Contact:	Mona Mustapha at apple dot com

Abstract: This contribution proposes a use case that highlights the need for consent when Personal Data is used.

1. Introduction

This use case intends to highlight the need for consent when Personal Data is used by the network.

Summary of changes compared with S1-254279 (changes in green):

- all definitions removed

- removed text in the description on GDPR

Summary of changes compared with S1-254260 (changes in yellow):

- terminology – clarified that Processing definition is only for Personal Data, so renamed "Personal Data Processing", applied elsewhere as needed

- PR2 Note on propagating Consent updates in the system re-phrased to be more generic

Summary of changes compared with S1-253220r3 (changes in cyan):

- Changed "Personal Data Usage Permission" to "Consent"

- Updated definition of "Consent" and "Personal Data", with notes to clarify the terms are based on GDPR, and a note to clarify that data subject is End-User, and Consent is expected to be updated by the End-User

- Added definition of "Processing (of Personal Data)", with note to clarify the term is based on GDPR

- In PR2, changed "dynamically" to "on demand" and added a Note to clarify that updates "on demand" would only need to be propagated in the system on a timescale exceeding one minute

- Removed PR3

- Additional clarifications in the description and service flow

2. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

---------- Proposed changes ----------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

(indicating only newly introduced references)

[x1]	Regulation (EU) 2016/679 on General Data Protection Regulation (https://eur-lex.europa.eu/eli/reg/2016/679/oj/eng)

[x2]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)"

[x3] 	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"

[x6]	3GPP TS 32.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services"

[x7]	3GPP TS 02.071: " Location Services (LCS); Stage 1"

---------- Proposed use case ----------

### 5.5.11	Use Case on Personal Data Usage

#### 5.5.11.1	Description

The 6G system will continue to use Personal Data in various ways, e.g. to enable services or to optimise service performance, in a similar fashion to the 5G system. As was done in 5G specifications, for 6G requirements where Consent is required to use Personal Data, it is intended verify whether Consent as supported in 5G is sufficient, or whether a more consistent treatment is needed.

In past generations, services provided by the operator are typically provided to individual subscribers in a "one-to-one" fashion, where Personal Data from a specific subscriber is needed to enable the same subscriber to receive the service. This Personal Data sharing is covered by the contractual agreement between the operator and the subscriber, and Consent as defined in 3GPP, is not applicable.

In 6G systems, it is envisioned that there will be services that an operator can provide to one entity (e.g. to a third party or multiple subscribers), where the service is generated by processing Personal Data from one or more other subscribers, i.e. the subscriber(s) whose Personal Data is used is not necessarily the only subscriber(s) that receives the service.

#### 5.5.11.2	Pre-conditions

Bob purchased a new subscription with Operator Vanilla, which includes access to a 6G XR gaming service, mobile data, voice services and text messaging.

As part of the agreement Bob signed with Operator Vanilla, there are privacy terms and conditions which indicate that e.g. the UE location, the duration of voice / data sessions, the amount of data consumed, are required to provide Bob with the services he signed up for.

Bob's subscription is created on Operator Vanilla's database which adds the services Bob has subscribed to, and the related privacy terms and conditions.

Bob has not subscribed to any sensing service from Operator Vanilla.

#### 5.5.11.3	Service Flows

1.	Operator Vanilla agrees to provide a sensing service to company Chocolate that provides V2X-related services in the town where Bob works.

2.	Operator Vanilla wishes to collect and process 3GPP sensing data that can be obtained via Bob's UE (and also 3GPP sensing data that can be obtained from a number of other subscribers who live or work in the same town) to generate the sensing service to company Chocolate.

3.	Operator Vanilla informs Bob about this request and purpose to use Bob's sensing Personal Data, i.e. 3GPP sensing data obtained via Bob's UE and other related information e.g. location information (how this is done is out of scope)

4.	Bob agrees to share his sensing Personal Data with Operator Vanilla for this purpose and gives his permission to do so (how this is done is out of scope). Bob still does not subscribe to any sensing service, but allows his data to be used.

5.	It is expected that Bob's permission is retained somewhere, e.g. in the network database, and that Bob does not have to be consulted each time Operator Vanilla wishes to use Bob's sensing Personal Data, but instead, Operator Vanilla will check Bob's permission in the network database.

6.	Operator Vanilla collects and processes Bob's sensing Personal Data along with other subscribers' sensing Personal Data (who have also consented to share their Personal Data) to generate a sensing service that is provided to company Chocolate.

7.	After one week, Bob notices that his UE's battery drains much faster than usual and decides to withdraw his permission for Operator Vanilla to use his sensing Personal Data (how this is done is out of scope).

8.	Operator Vanilla stops using Bob's sensing Personal Data when generating sensing service provided to company chocolate.

9.	One month later, Operator Vanilla wishes to once again use Bob's sensing Personal Data to provide sensing service to company Chocolate, but upon consulting the network database, Bob's permission was not changed, so Operator Vanilla continues not to use Bob's sensing Personal Data.

#### 5.5.11.4	Post-conditions

Bob's permission on whether or not his sensing Personal Data can be collected and processed by Operator Vanilla to generate sensing services provided to a different entity, i.e. the sensing service is provided to company Chocolate and not to Bob himself, can be taken into account by Operator Vanilla.

#### 5.5.11.5	Existing features partly or fully covering the use case functionality

|  | Consent is an opt-in | Verify consent before network uses Personal Data | UE can change / revoke consent | Indicates if consent was granted | Indicates purpose for Personal Data usage | Indicates whether Personal Data is used by MNO or third party | Upon consent revocation, stop related Personal Data usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LCS | Supported (TS 23.271 [x2] clause 9.5.3.3) | Supported (TS 23.273 [240] clause 6.1.2) | Supported (TS 23.273 [240] clause 5.4, UE can update LCS privacy profile / LPI) | Supported (TS 23.273 [240] via LPI + LCS privacy profile) | Not supported | Supported: (TS 23.273 [240], LCS privacy profile indicates permitted LCS clients / AFs) | Not applicable (one-off request) |
| MDT | Up to implementation (setting is out of 3GPP scope) | Supported (TS 32.422 [239]) | Up to implementation (setting is out of 3GPP scope) | Supported (Management / Signalling Based MDT PLMN List) | Not supported | Up to implementation (setting is out of 3GPP scope) | Up to implementation (TS 32.422 [239] clause 4.9.1) |
| NWDAF Analytics | Up to implementation (setting is out of 3GPP scope) | Supported (TS 23.288 [x6] clause 6.2.9) | Up to implementation (setting is out of 3GPP scope) | Supported (TS 23.288 [x6] clause 6.2.9) | Supported (TS 23.288 [x6] clause 6.2.9) | Unclear if supported, no consent checked for NEF exposure to AFs (TS 23.288 [x6] clause 6.2.2.3) | Supported (TS 23.288 [x6] clause 6.2.9) |
| CAPIF RNAA | Supported (TS 23.222 [x3] clause 4.17) | Supported (TS 23.222 [x3] clause 8.31) | Supported (TS 23.222 [x3] clause 8.23) | Supported (TS 23.222 [x3] clause 8.31) | Supported (TS 23.222 [x3] clause 8.11) | Supported (TS 23.222 [x3] clause 8.31) | Supported (TS 23.222 [x3] clause 8.23) |

Table 5.5.11.5-1: Gap analysis of consent for selected 5G features

From the table above, it can be seen that various aspects of Consent are not consistently supported across different features, and that for certain features like MDT and NWDAF analytics, a number of the Consent aspects are left up to implementation.

Gap analysis to configure consent in 5G

For LCS, the Stage 1 requirements defined in TS 22.071 [61] (inherited from TS 02.71 [x7] since Rel-98) state:

Means shall be provided for the UE subscriber to control privacy for value added services.

The user shall be able to change the setting of the Privacy exception list at any time.

Unless required by local regulatory requirements, or overridden by the target UE User, the target UE may be positioned only if allowed in the UE subscription profile. In general, for valued added location services, the target UE being positioned should be afforded the maximum possible privacy, and should not be positioned unless the positioning attempt is explicitly authorized. In the absence of specific permission to position the target UE, the target UE should not be positioned.

A requirement for user consent for Personal Data intended to be shared with a third party is defined in TS 22.261 [14] clause 6.10:

The 5G system shall be able to: […]

-	allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party.

This requirement is related to the CAPIF Resource owner-aware Northbound API Access (RNAA) in TS 23.222 [x3], where authorisation is required from the resource owner for usage of their data. According to TS 23.222 [x3], the resource owner is a UE user or MNO subscriber.

#### 5.5.11.6	Potential New Requirements needed to support the use case

[PR 5.5.9.5-001] Subject to regulatory requirements and operator policy, the 6G system shall apply End-User consent when the 6G network requests the UE to make sensing measurements on behalf of the 6G network, for sensing services NOT provided to that UE.

[PR 5.5.9.5-001] Subject to regulatory requirements and operator policy, the 6G system shall provide mechanisms to enable the End-User to make updates on demand to her consent with control over specific data types and purposes.

NOTE 1:	TS 33.501 [250] clause V.1.2 describes the relationship between End-User and Subscriber

NOTE 2: "On demand" request to update consent is expected to be propagated through the system on a reasonable timescale

NOTE 3:	Personal Data sharing between the End-User and 6G system for the reasons below do not require consent:

-	when used to meet contractual agreement(s) to provide service(s) from the network operator to the subscriber (includes Personal Data such as identifiers in the network and other Personal Data needed to support the service delivery to the subscriber e.g. location information),

-	when used to meet legal obligations (e.g. requested by authorities), or

-	when used to protect someone's life (e.g. sharing location for emergency calls).