3GPP TSG SA WG 1 Meeting #112 	S1-254327

17-21 November 2025, Dallas, USA	(revision of 4266r1, merge of 4122)

Source:	Nokia (Moderator), CATT

pCR Title:	Pseudo-CR on consolidated performance requirements for Ubiqiutous Connectivity

Draft Spec:	3GPP 22.870

Agenda item:	8.1.10

Document for:	Approval

Contact:	feifei.lou@nokia.com

Abstract: this contribution provides the consolidation of performance requirements in clause 8 ubiquitous connectivity.

1. Introduction

The consolidated performance requirements for clause 8 are categorised into two parts.

The consolidated performance requirements for satellite-based positioning services covers the following use cases and PRs. The scenarios in the KPI table are sorted from less demanding to more demanding cases.

- [PR 8.5.6-1] from use case on resilient positioning in satellite networks

- [PR 8.7.6-2] from use case on low-energy positioning in satellite networks

- [PR 8.10.6-1] from use case on hybrid TN and NTN positioning

- [PR 8.11.6-2] from use case on hybrid NTN and GNSS positioning

The consolidated performance requirements for satellite access covers the following use cases and PRs.

- [PR 8.4.6-2] from use case on service continuity for wearable mobile devices

- [PR 8.6.6-2] from use case on disaster relief

- [PR 8.8.6-2] from use case on global mobile video

- [PR 8.9.6-2] from use case on low-altitude logistics supported by NTN

The following PR is not consolidated due to FFS.

- [PR 8.2.6-1] from use case on ubiquitous and resilient network

2. Reason for Change

To provide the consolidation of performance requirements in clause 8 ubiquitous connectivity.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.0.

* * * First Change * * * *

All new texts

## 8.x	Consolidated performance requirements for Ubiquitous Connectivity

### 8.x.1	Consolidated performance requirements for satellite-based positioning services

Table 8.x.1-1: Consolidated performance requirements for positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | MAX UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |

Editor’s Note: maritime mounted UE speed is FFS.

### 8.x.2	Consolidated performance requirements for satellite access

Table 8.x.2-1: Consolidated performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Peak data rate (DL) | Peak data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Reliability | Service availability | End-to-end latency | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE 1: Area capacity is averaged over a satellite beam. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

[CPR 8.x.2-1]: The 6G system with satellite access, shall support SMS delivery to UEs with up to [1000] devices/km2 density.

[CPR 8.x.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *