3GPP TSG-SA WG1 Meeting #112	S1-254407

17-21 November 2025, Dallas, USA 	(revision of S1-254076, S1-253659)

Source:	Nokia, AT&T, ESA, Thales, Novamint, Airbus

pCR title:	Updated use case 8.7 on low-power positioning

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	orlett.pearson@nokia.com, laurent-walter.goix@nokia.com

Abstract: This contribution originates from a new use case on extended battery life for massive communications in the context of livestock and asset tracking in the agriculture domain. It has been revised as an update of the low-power positioning use case to consider these scenarios.

1. Introduction

Low-power positioning is key for tracking in the agricultural domain, eg for livestock and passive assets, where GNSS positioning dependency has a strong impact on battery-powered devices used in these scenarios.

2. Reason for Change

This contribution aims at providing new KPIs to cover tracking in the agricultural domain, in particular for livestock and passive assets (e.g. non-powered harvesting crates, trailers, wheelbarrows).

It also enhances the use case to accommodate cases when no initial position is known from GNSS, in future case where some of these devices may not onobard any GNSS capability anymore, thus resulting in enhanced battery lifetime and lower costs of operation.

A “sustainability impact analysis” of this use case is further provided.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1

R4407

- Adding reference [Huawei]

Rev SA1#112

- Converted to an update to the low-energy positioning use case

- Added KPI for livestock/equipment tracking

- Clarified frequency of messages

- Added sustainability impact analysis

---------- Use Case template ----------

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[…]

[x]	Biquan Zhao, Rebecca L McDermott, Galen E Erickson, Yijie Xiong, Technical note: assessing GPS sensor accuracy using real-time kinematic device for livestock tracking, Journal of Animal Science, Volume 102, 2024, skae253, https://doi.org/10.1093/jas/skae253

* * * Second Change * * * *

## 8.7	Use case on low-energy positioning in satellite networks

### 8.7.1	Description

This use case corresponds to a capability for 6G with satellite access to provide low-energy positioning for low-power and low-complexity devices (e.g. IoT and low-power devices) when served by a satellite network. Integrating GNSS into IoT devices operating over satellite networks significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning updates. The extent of this impact varies based on GNSS start modes (hot, warm, or cold), power consumption of GNSS modules, reporting intervals, and device mobility. According to the analysis carried out in [26], GNSS position fixes can reduce battery life from approximately 5% up to 45% in cases of frequent position fixes. Mitigating the power consumption due to GNSS would therefore be beneficial. To minimize energy consumption, NTN signals can be used for position estimation, reducing dependency on GNSS for position information updates.

Integrating NTN-based positioning could be particularly advantageous for applications such as asset tracking in logistics, where devices like containers, wagons and pallets typically operate over extended periods with limited power resources, as outlined in TS 22.261 [14]. While specific performance targets for battery life, message size, and reporting frequency are defined (See Table G.2-1 of “Annex G” in TS 22.261 [14]), positioning accuracy is not currently addressed. The horizontal positioning accuracy target and positioning service availability is expected to be aligned with [338].

Sustainability impact analysis

- Energy resources, material resources, emissions: Improved energy efficiency of tracker devices due to low energy positioning techniques saves overall energy resources over long periods of time, and further allows smaller batteries, which reduce water & soil pollution, or less battery & device replacement thus less e-waste and material extraction, and reduced emissions along the device lifecycle.

- Health: Less battery & device replacements due to lower battery drain related to positioning fixes means less maintenance actions on each device thus reducing the burden of on-site interventions and associated health risks.

- Work & income: Less cost of replacement of battery/device (including product & manpower cost). Workers’ activity usually assigned to replace batteries in remote areas can be optimized.

- Infrastructure: Improving Housing, Transport, Connectivity, Energy, Water, Waste management infrastructures by enabling easier deployments and more convenient business cases to deploy low-power asset tracking devices with less operational/maintenance costs in the long run (e.g. less device/battery replacement cycles, smaller form factor, less lost/stolen assets etc).

### 8.7.2	Pre-conditions

A battery-powered IoT or low-power device has been deployed in an environment where it can obtain its initial position and synchronization from non-3GPP methods (e.g. GNSS) and / or terrestrial networks. A combination of non-3GPP technologies (e.g. GNSS) and 3GPP technologies, terrestrial and non-terrestrial, can be configured if compatible with energy consumption requirements.

### 8.7.3	Service Flows

1. At some point, the IoT or low-power device may lose its original positioning sources due to one of the following reasons:

-	GNSS-originated position: GNSS signal may be lost due to interference, or obstruction, or intentionally disabled to augment its autonomy.

-	Terrestrial network-originated position: the IoT or low-power device may have moved beyond terrestrial network coverage, such as in the case of asset tracking sensor installed on a container to be transported by sea, rail, or road.

- In this context, it requires the IoT or low-power device to refresh a more accurate position. The IoT or low-power device, served by the satellite access, may know an approximate position from its original positioning.

- Therefore, the IoT or low-power UE initiates the 3GPP positioning method over satellite to estimate its current location.

- The 6G system broadcasts to the UE via its satellite access at least the following necessary information, including but not limited to:

-	Network assistance data: Satellite ephemeris and additional assistance data to improve accuracy (e.g. ionospheric models to correct the atmospheric delay errors, etc.)

-	Reference signal for time of arrival measurements.

- The IoT or low-power UE can use the 3GPP communication service to send this position into a message to a third party application server

- Steps 1-5 may be repeated several times a day

### 8.7.4	Post-conditions

The IoT or low-power device successfully estimates/updates its position based on information provided by the 6G system via the satellite access.

### 8.7.5	Existing features partly or fully covering the use case functionality

Ranging signals and positioning methods for IoT and low-power applications are defined in TS 36.211 [45] and TS 38.211 [46] and TS 36.305 [47] and TS 38.305 [48], respectively, but only for terrestrial networks. They need to be adapted to satellite access context.

The functional and performance requirements for asset tracking use cases reported in the below “Table G.2-1: Battery life expectancy and message size to support example use cases for asset tracking” of “Annex G (informative): Asset Tracking use cases” in TS 22.261 [14] do not include positioning requirements.

Table 8.7.5-1: Battery life expectancy and message size to support example use cases for asset tracking (Table G.2-1 in [14])

|  | Scenario | Battery Life Expectancy (note 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device Density |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Containers (note 2) | 12 years | 200 bytes | 2500 bytes | 24 | 21,6 Wh | 1,4 devices / m3 |
| 2 | Wagons | 20 years | 200 bytes | 2500 bytes | 24 | 36 Wh | 0,3 devices / m2 |
| 3 | Pallets | 7 years | 300 bytes | 300 bytes | 24 | 12 Wh | 4 devices/ m2 |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: A large containership with a mix of 20 ft and 40 ft containers is assumed. NOTE 3: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |

### 8.7.6	Potential New Requirements needed to support the use case

[PR 8.7.6-1]: The 6G system with satellite access shall be able to provide a mechanism for energy constrained devices to determine its position using 3GPP positioning technology.

[PR 8.7.6-2]: The 6G system with satellite access shall be able to provide positioning service with the following KPIs:

Table 8.7.6-1: Performance requirements for satellite-based positioning services for energy constrained devices from [338]

| Scenario | Battery Life Expectancy (note) | Horizontal Accuracy (95 % confidence level) | Positioning service availability | Positioning service area / Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- |
| Containers | 12 years | [100] m | [95 %] | Outdoor | Up to [37] km/h | Container mounted |
| Wagons | 20 years | [100] m | [95 %] | Outdoor | Up to [350] km/h | Train mounted |
| Pallets | 7 years | [100] m | [95 %] | Outdoor | Up to [100] km/h | Vehicle mounted |
| Agriculture & farming (e.g. livestock, equipment tracking) | 10 years | [5-10] m [x] | [95 %] | Outdoor (Rural/remote areas) | Up to [30] km/h | Animal mounted (e.g. collar) or passive asset/equipment mounted (e.g. non-powered harvesting crates, trailers, wheelbarrows). |
| NOTE: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency. The frequency assumed here is the number of times per day a device is involved in a positioning operation using terrestrial/6G Satellite Positioning, with the required accuracy anywhere within the required coverage area, which could be followed by a communication request from the UE to send this position into a message to a third party application server, unless this information can be obtained from the 6G network itself. In this table, frequency is assumed to be 1h (that is approximately 24 operations per day) and typical message size around 200 bytes. |  |  |  |  |  |  |

* * * *End of Changes* * *