3GPP TSG-SA WG1 Meeting #111 	S1-253050r2

25-29 August 2025, Goteborg, Sweden	(revision of S1-253050r1)

Source:	LG Uplus, SK Telecom

New use case title:	New use case on Event-based Inter-Operator Reselection Coordination

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Taeyeoun Kim (tkim@lguplus.co.kr), Jaehyun Ahn (jaehyun80@lguplus.co.kr)

Abstract: This use case primarily focuses on scenarios of specific events that require inter-operator reselection coordination. More specifically, it is based on an NTN scenario and cell reselection between MNOs and SNOs. Through this use case, MNOs can ask SNOs to provide the satellite-based communication only when to be needed such as during disasters, enhancing energy saving, and fulfilling other critical objectives.

---------- Use Case template ----------

x.1	Use case on Event-based Inter-Operator Reselection Coordination

x.1.1	Description

The concept of ubiquitous network aims to enhance connectivity and improve user experience by ensuring uninterrupted access to communication services across all times and locations. Within this framework, Non-Terrestrial Networks (NTNs) are recognized as a pivotal enabler of ubiquitous connectivity, as they expand and sustain network coverage in areas where traditional terrestrial infrastructure is limited or unavailable. Among NTNs, satellites are anticipated to play a critical role in realizing ubiquitous connectivity, particularly through the deployment of Low-Earth Orbit (LEO) satellite constellations or Geostationary Orbit (GEO) satellite systems.

For the integrated network, maintaining seamless connectivity and keeping connection to an appropriate and efficient network between TN and NTN is essential. Thus, to realize this integrated network, effective collaboration must be achieved between Mobile Network Operators (MNOs) and Satellite Network Operators (SNOs), with clear coordination and standards defined.

The use case of “Event-based Inter-Operator Reselection Coordination” is a practical example of a Satellite Terrestrial Integrated Network (STIN), where the satellite network acts as a complementary system to the terrestrial network to ensure ubiquitous connectivity to user devices. This use case assumes that, when specific events occur, the satellite network ensures seamless connectivity while others do not get affected to their connections. The specific events could include scenarios for network resiliency such as situations where the terrestrial network (TN) is out of service due to disasters and scenarios that offload connection to the NTN for operational efficiency of the TN such as resolution of overloaded TN, energy-saving of TN during low-usage periods, and communication of aerial devices far from TN.

The common theme in this use case is that events cause changes in the coverage of the TN, and the satellite network must compensate for these changes. Given the distinct characteristics of terrestrial and satellite systems, such as differences in signal power attenuation and cell size, the inter-operator coordination is required to ensure seamless connectivity. More specifically, this use case focuses on the user equipments (UEs) in idle mode. For the event area, the NTN operator broadcasts updated cell reselection priority to the idle mode UEs in its satellite beam coverage.

Figure 1: Use case of event case and cell coverage comparison

Fundamentally, the inter-operator reselection coordination use case is to minimize unnecessary power consumption in user equipment (UE) by preventing unintended network reselections.

x.1.2	Pre-conditions

Operator A is a Mobile Network Operator that is serving a region named PLACE1, and Operator B is a Satellite Network Operator that can also serve the region PLACE1 as a complementary. Including the region of the PLACE1, the cell reselection priority is set for TN to idle mode devices for their connection stability and power efficiency. For example, in the areas where TN cells are actively serving customers, the neighbour cell list of TN cells in Operator A is configured to give high priority to other TN cells while assigning lower priority to NTN cells. the neighbour cell list of NTN cells in Operator B is configured to give lower priority to NTN cells and higher priority to TN cells.

Someday, the “event” occurred. For example, the infrastructure of Operator A is destroyed in PLACE1 due to the natural disaster. There exist user devices in idle mode that are served by Operator A: UE1 in PLACE1 and UE2 not in PLACE1, but nearby the PLACE1.

x.1.3	Service Flows

1. Usually, Operator A and Operator B set priority of NTN to be low in both TN and NTN, in order to prevent undesired network reselection of customers.

2. If an event such as disaster or traffic offload occurs over a region, i.e. PLACE1, Operator A sets priority of NTN higher within TN and NTN. Also, Operator A will request Operator B to change the priority of NTN for PLACE1. Accordingly, UE1 and UE2 are initially expected to camp on the Operator B.

3. The user devices (UE1 and UE2) in Operator B’s gNB coverage which include PLACE1 receive the SIB information and performs reselection, wherein Operatros B’s gNB coverage would be bigger than PLACE1.

- UE1 (in PLACE1): Since UE1 is in the regional information of PLACE1, perform the cell reselection to be connected to Opertator B.

- UE2 (out of PLACE1, but in the Operator B’s gNB coverage): Due to the large size of the satellite beams, UE2 is in the overlaied network with Operator A and Operator B. Thus, UE2 compares the regional information in SIB signal of Operator B and its GPS information, and perform cell reselection to Operator A.

x.1.4	Post-conditions

UE1 that is in the event area PLACE1 maintains the network connection during the event by reselecting to the NTN cell in idle mode.

UE2 that is not in the PLACE1 but in the coverage of Operator B during the event still connect to Operator A. Thus, UE2 prevents unnecessary power consumption when reselecting NTN. (Note that if the event area is small, limit in satellite beam size may cause unnecessary requests for priority adjustments for NTN even in surrounding areas outside the intended region.)

x.1.5	Existing features partly or fully covering the use case functionality

Clause 5.2.4.5 in TS 38.304 defines the cell reselection criteria for NR inter-frequency and inter-RAT scenarios. The UE can be provided with priorities for different frequencies or RATs, and the specification outlines the conditions under which the UE shall reselect to a higher or lower priority based on the received signal quality and level.

Clause 5.2.4.6 in TS 38.304 specifies the cell reselection criteria applicable to intra-frequency and equal priority scenarios. In these cases, the reselection decision is based on a ranking criterion that incorporates measurement offsets to determine the most suitable cell.

x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-1]: The 6G system shall support a standardized inter-operator management/exposure interface between an MNO and an SNO to report event-related priority directives and support measurements and coordinate reselection policies by the request of an MNO.

[PR x.1.6-2]: The 6G system shall support inclusion of event-scope information including regional identifiers and validity time in broadcast system information to guide UE priority application and cell reselection.

[PR x.1.6-3]: The 6G system shall support inter-operator coordination procedures between TN and NTN operators for event-based reselection, including initiation, activation, update, termination and rollback, with acknowledgement and validity control.

[PR x.1.6-4]: The 6G system shall enable a UE, when idle or inactive, to conditionally reselect to NTN based on operator-provided priority information and the UE’s location and context to minimize power consumption.