3GPP TSG SA WG 1 Meeting #111 	S1-253218r1

25-29 August 2025, Goteborg, Sweden	(Revision of S1-253218)

Source:	Xiaomi

pCR Title:	Update on child health management assistant

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Ruby Gong (gongyubing@xiaomi.com)

Abstract: This pCR updates the use case child health management assistant in clause 6.22. Two new potential requirements are added: data transmission between the AI agent application on UEs, and AI agent application selection.

* * * First change * * * *

6.22.2	Pre-conditions

Mother Emma and Father David bought a smart watch phone for their daughter Lily. Lily's smart watch phone has an AI agent aboard and health application.

Lily's smart watch phone stores mother Emma's and father David's numbers as emergency contacts. Mother Emma's smart band and cell phone have AI agent. Father David's cell phone also have AI agents.

Lily's smart watch phone has given consent for collecting, distributing and analysing data. Mother Emma's and Father David's AI agents are authorized to obtain and analyse data collected from Lily's smart watch phone.

The network stores the AI agent related information, e.g. AI agent capability.

* * * Next change * * * *

6.22.3	Service Flows

1.	Lily went to school wearing her smart watch phone. The smart watch phone routinely tracked her heart rate, body temperature, and physical activity every 30 minutes.

2.	During morning classes, the watch recorded consistent readings: heart rate between 72-75 BPM, body temperature ranging 36.3°C-36.5°C, with moderate activity levels. Based on the above parameters, the AI agent on Lily's watch determined Lily was in good health.

3.	In the afternoon, unusual patterns emerged: Lily's heart rate elevated to 80 BPM with a temperature of 37°C. The AI agent on Lily's watch.

4.	The AI agent on Lily's watch detects the unusual pattern, and:

a.	sends a notification to the health application on Lily's watch, the application immediately increased monitoring frequency to every 5 minutes;

b.	prepares to invoke a voice call to her emergency contacts (i.e. mother Emma and father David), and sends a request to the network for optimal contact selection and corresponding information obtainment; and

c.	invokes the location service of Lily's watch, to assist the disease prediction in the application on the watch.

5.	The network receives the request from AI agent Lily's watch, and:

a.	determines that the AI agent Lily's watch is an authorized AI agent;

b.	accepts the request based on the subscription data and user consent of Lily's watch;

c.	retrieves the stored information of Emma's and David's cell phone (e.g. connection status, AI agent capability, subscription information, location information);

d.	selects Emma as primary contact based on the information stored in the network (Emma's cell phone and smart band: connected status, 2km away from Lily, AI agent supported; David's cell phone: deregistered status, 7km away from Lily, AI agent supported);

e.	selects Emma's cell phone from Emma's device list based on the device's capability (Emma's cell phone: positioning supported, sensing supported, computing supported, and AI supported, Emma's smart band: AI supported); and

f.	 sends a response message to AI agent on Lily's watch, including the information of Emma's cell phone (e.g. connection status, AI agent capability, location information).

6.	Based on the response message received from the network, AI agent on Lily's watch:

a.	sends the location information obtained from the network to upper layer application. Based on Lily's health profile (age, medical history), location information, and recent prevalent disease in local area, the application calculated an 80% probability of influenza infection.

b.	invokes data transmission request to Emma's AI agent through the network. AI agent on Lily's watch sends Lily's health metrics, inferred disease data to Emma's AI agent through the network.

c.	initiated a voice call to Emma's cell phone.

7.	Upon receiving the call, Emma immediately goes to the school to pick up Lily. The 6G network establishes a data path between Emma's AI agent and Lily's AI agent. The data between Lily's AI agent and Emma's AI agent can be transferred through the 6G network. Emma's AI agent sends a location request to Lily's AI agent via the established data path. The request from Emma's AI agent triggers Lily's AI agent to invoke positioning service. Emma's AI agent also invokes positioning services and sensing services to sense the surroundings of Emma's phone. The network calculates and exposes the Emma's positioning results  and the sensing results  to Emma's AI agent, and Lily's positioning results to Lily's AI agent. Lily sends the positioning results via the data path to Emma's AI agent.

8.	Based on the positioning and sensing results received from the network and the Lily's data received from the data path, Emma's AI agent generates the optimal route for Emma to reach Lily. Every 5 minutes, Lily's AI agent updates Emma's phone with Lily's latest health metrics and positioning results through the data path via the 6G network. Following the navigation plan generated by the Emma's AI agent, Emma successfully retrieves Lily and drives her to the hospital.

Figure 6.22.3-1: Child health management assistant

* * * Next change * * * *

6.22.6	Potential New Requirements needed to support the use case

[PR 6.22.6-1] The 6G network shall support a mechanism to manage the information of AI application (e.g. AI agent application) on UE (e.g. related user).

[PR 6.22.6-2] The 6G network shall support a mechanism to authorize AI application (e.g. AI agent application) on UE to invoke 3GPP services.

[PR 6.22.6-3] Based on user consent and operator's policy, the 6G network shall support a secure mechanism to expose information of AI application (e.g. AI agent application) on UE (e.g. related user) to AI application (e.g. AI agent application) on other UE.

[PR 6.22.6-x1] Based on user consent and operator's policy, the 6G network shall support a secure mechanism to transport 3GPP service data (e.g., sensing results) from AI agent on a UE to another AI agent on another UE for a collaborative task.

[PR 6.22.6-x2] Based on the operator's policy, the 6G network shall support a mechanism to select an appropriate AI agent on UE belonging to a specific user.

* * * End of changes * * * *