3GPP TSG SA WG 1 Meeting #111 	S1-253227r2

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Huawei

pCR Title:	New Use Case on enhancement of voice service

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Fangyuan Zhu <zhufangyuan@huawei.com>
Chuting Yao <yaochuting@huawei.com>

Abstract: This document provides new use case on enhancement of voice service.

1. Introduction

The document explores 6G enhancements for legacy service.

2. Reason for Change

As a cornerstone of communication capability, how to enhance voice service in dynamic network environments should be considered for 6G.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22870 v.0.3.1.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[r1] 	3GPP TS 26.071: "Mandatory speech codec speech processing functions; AMR speech Codec; General description".

[r2] 	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[r3] 	I. Varga, S. Proust and H. Taddei, "ITU-T G.729.1 scalable codec for new wideband services," in IEEE Communications Magazine, vol. 47, no. 10, pp. 131-137, October 2009.

[r4]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[r5]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General overview".

* * * Second Change * * * *

### 5.7.x	Enhancement of voice service

#### 5.7.x.1	Description

Voice service, a fundamental communication service provided by operators, is a cornerstone of telecommunication networks. It enables real-time, bidirectional communication between users and is deployed through IP Multimedia Subsystem (IMS) core networks, ensuring seamless global connectivity.

Voice service utilizes codecs standardized by 3GPP for efficient digital representation of audio signals. The selection of a specific codec, e.g. AMR-NB (Adaptive Multi-Rate Narrowband), or AMR-WB (Adaptive Multi-Rate Wideband), depends on both network capabilities and terminal support, ensuring interoperability across the system. Each voice codec type has well-defined operating parameters, including bit rate ranges, sampling rates, and audio bandwidths. For example, the core parameters of AMR-NB [r1] and AMR-WB [r2] are detailed in Table 5.7.x.1-1.

Table 5.7.x.1-1:  The core parameters of AMR-WB and AMR-NB

| Codec (note) | Sampling rate (kHz) | Bit rate(kbit/s) |
| --- | --- | --- |
| AMR-NB [r1] | 8 | 4.75/5.15/5.90/6.70/7.40/7.95/10.20/12.20 |
| AMR-WB [r2] | 16 | 6.60/8.85/12.65/14.25/15.85/18.25/19.85/23.05/23.85 |
| NOTE: The narrowband codec corresponding audio bandwidth is 300–3400 Hz and the wideband codec corresponding audio bandwidth is 50–7000 Hz [r3]. |  |  |

With the rapid development of telecommunications technology, the emergence of the 6G era has introduced new expectations for user experience. As a critical service, the voice continuity is increasingly demanded, especially in dynamic network environments. Such scenarios include dynamic air interface environmental changes in terrestrial networks (e.g., high-speed rail or mobility between indoor and underground environments) and satellite communications (e.g., GEO-LEO inter-orbit transitions) These environments present significant challenges to maintain seamless voice service and consistent user experience.

In these dynamic environment, current voice codecs, such as AMR-NB[r1] and AMR-WB[r2], adjust their bit rates in response to network condition fluctuations. However, such adaptations often require voice codec switch, which can be perceived negatively by the user. For instance, when channel condition degrades (e.g., from high SNR to low SNR), the network forces UE to switch from AMR-WB (e.g. 23.85 kbit/s, 16 kHz) to AMR-NB (e.g. 4.75 kbit/s, 8 kHz) via signalling.

Figure 5.7.x.1-1: Codec switch (from AMR-WB to AMR-NB)

In the 4G and 5G era, 3GPP has specified advanced codecs such as EVS [r5] and IVAS [r4] to, in part, enable adaptive bitrate  and audio bandwidth within the codec. This enables seamless changes between codec modes which enables adapting to changing channel conditions gracefully. However, these advanced codecs do not generally specify how to prevent user-perceptible issues when switching codecs entirely during a call.

In the 6G era, the system is expected to provide enhanced voice services to operate seamlessly even under dynamic network conditions. The optimization prioritizes service continuity and user experience during network fluctuations. For instance, 6G voice services require to prevent user perceived audio disruptions caused by changing codecs.

#### 5.7.x.2	Potential New Requirements needed to support the use case

[PR 5.7.x.2-1] The multimedia telephony service provided by IMS shall optimize the voice service to ensure user -imperceptible transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation.

* * * End of Change * * * *