3GPP TSG-SA WG1 Meeting #111 	S1-253302

Goteborg, Sweden, 25-29 August 2025

Title:	Use case on 3D hyper-realistic video services

Draft Spec:	3GPP TR 22.870

Agenda Item:	8.1.6- lmmersive Reality

Source:	BUPT, Pengcheng Laboratory,  ZGC Institute of Ubiquitous-X Innovation and Application, MIGU Co.,Ltd, CMCC

Document for:	Approval

Contact:	Xiaodong Xu (xuxiaodong@bupt.edu.cn), Mengying Sun (smy_bupt@bupt.edu.cn), Nan Ma (manan@bupt.edu.cn), Sheng Jiang (shengjiang@bupt.edu.cn),  Haixiao Gao (haixiao@bupt.edu.cn), Zhe Zheng (zhengzh02@pcl.ac.cn),  Yunfei Luo (luoyunfei@migu.chinamobile.com)

Abstract: This document proposes a Use case on 3D hyper-realistic video services.

* * * First Change* * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TS 22.261: "Service requirements for the 5G system".

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

- 3GPP TS 22.156: "Mobile Metaverse Services".

- 3GPP TR 22.847: “Study on supporting tactile and multi-modality communication services; Stage 1”.

- 3GPP TR 26.998: “Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.

- 3GPP TR 26.806: “Study on Tethering AR Glasses – Architectures, QoS and Media Aspects”.

- Xuemin S., Jie Gao, “Toward Immersive Communications in 6G”, Frontiers of Computer Science, Volume 4 (2022), 11 January 2023

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change (all the text is new)* * * *

## 9.X	Use case on 3D hyper-realistic video services

### 9.X.1	Description

3D hyper-realistic video streaming in stadiums is one of the key application scenarios for 6G-enabled immersive communications. By deploying multi-angle holographic camera arrays and real-time 3D reconstruction systems within the stadium, and leveraging the high throughput, ultra-low latency, high-density access, and intelligent scheduling capabilities of 6G networks, users equipped with XR headsets or AR glasses can experience live sports events from arbitrary viewing angles in a fully immersive manner.

This use case involves real-time reception and interactive control of holographic video streams across multiple user terminals, which imposes stringent demands on the 6G network infrastructure, including the access network (6G RAN), edge computing nodes, and the intelligent control and resource management capabilities of the core network.

This use case poses several significant challenges to current 5G/ 5G-A networks. First, the massive volume of high-fidelity 3D video data demands ultra-high uplink and downlink capacity, far exceeding that of current wireless systems. Second, to ensure real-time viewpoint adaptation and smooth user interaction, the network must support end-to-end latency within a few milliseconds. Third, coordinating viewpoint synchronization and ensuring consistent Quality of Experience (QoE) for thousands of concurrent users introduces new complexity in resource allocation, network slicing, and AI model deployment.

### 9.X.2	Pre-conditions

6G networks provide live and broadcast services for stadium events, supporting 3D hyper-realistic video delivery to both on-site audiences and remote viewers.

- 6G RAN is deployed across the entire stadium area, enabling communications in both millimeter-wave and terahertz frequency bands.

NOTE:	The following terms 6G RAN/CN do not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

2) The core networks or edge servers are equipped with capabilities for real-time 3D rendering, semantic processing, and model inference.

3) The core network provides intelligent user path control, resource awareness, and AI model orchestration.

4) User terminals support high-rate 3D video reception and semantic-driven interactive control inputs.

### 9.X.3	Service Flows

Figure 9.X.3-1  Use case on 3D hyper-realistic video services

- A stadium is hosting a large-scale sports event, during which an array of cameras deployed throughout the stadium captures multi-angle high-definition video and depth information. Leveraging semantic communication mechanisms, the captured data is transmitted to the 6G RAN and forwarded to the cloud or edge server.  Multi-view video streams are simultaneously transmitted to the 6G RAN. Due to significant feature overlap across different camera angles, substantial redundant information exists. To enhance resource efficiency in multi-stream high-definition video transmission, it is essential to dynamically select and transmit key features from multiple streams.

- The edge or cloud server dynamically selects optimal viewpoints and performs 3D video rendering according to user-related action information such as location, region of interest, and head movement trajectory. Meanwhile, semantic communication and other AI techniques are applied to enhance the encoding of key regions, enabling task-relevant prioritized compression and transmission. This approach allows efficient 3D hyper-realistic video reconstruction even under packet loss conditions.

- User terminals access the 6G RAN, where the 6G RAN identifies service categories and reports the request to the core network or edge server. The core network or edge server allocates a corresponding “immersive media service” slice through network slicing management. By leveraging semantic communication or other AI techniques, encoding video streams are delivered to user terminals through the 6G RAN, under the scheduling control of 6G network, ensuring frame synchronization and smooth viewpoint switching. The 6G RAN ensures low-latency performance through localized caching and channel-aware resource scheduling mechanisms.

- User terminals restore, render and display the content in real time by AI model and send uplink control signals based on interactive actions (e.g., head movement, voice commands, or gestures) to request viewpoint switching. These control signals are received by the 6G network and quickly forwarded to the service provider for task adaptation.

- The 6G core network, equipped with intelligent orchestration modules, coordinates multi-user viewpoint synchronization and dynamically adjusts resource allocation strategies and AI model caching policies, thereby enhancing the overall Quality of Experience (QoE) and optimizing network resource utilization.

### 9.X.4	Post-conditions

- Upon completion of the viewing session, the system can store selected semantic information either locally or in the cloud to facilitate the training of user preference models, the optimization of recommendation strategies, or the post-event tactical analysis.

- Based on transmitted data, user behavior, and service quality metrics, the network dynamically optimizes resource orchestration strategies and AI model caching policies to enhance overall performance and responsiveness.

### 9.X.5	Existing features partly or fully covering the use case functionality

Currently, communication requirements as described in TS 22.261 clauses 7.1, 7.6.1 and 7.10 are provided. And it is assumed that there are always sufficient computing resources at the selected one node for running rendering server. 5G network lack mechanisms for intermediary data for collaborative tasks in large-scale rendering.

The split rendering architectures defined in TR 26.928 and TS 26.565, and the corresponding QoS requirements in TS 22.261, allow a UE to offload rendering requirements to the cloud for XR rendering. These architectures assume that the metadata including 3D models of the gaming and APP logic is pre-installed in the cloud, therefore only user pose information is needed to be transferred in the uplink and there is no dynamic offload decision on whether to render a specific scene in the cloud or with UE local resources (i.e. fully rely on the rendering result in the cloud). Therefore, the uplink requirement and dynamic offload decision requirement are not covered by the existing split rendering use cases and part of the downlink requirements on rendering result delivery are covered by the existing split rendering use cases.

An “Audio and Video Production”  use case which uses HD and UHD resolutions deployed over 5G system is introduced in TS 22.104. Some normative requirements related to support of media service are in TS 22.263.

### 9.X.6	Potential New Requirements needed to support the use case

[PR 9.X.6-1]

Subject to user consent and operator policy, the 6G network shall support optional efficient means to adress transmission redundancy scenarios and guarantee the quality of service (QoS) under packet loss conditions.

* * * End of Change * * * *