3GPP TSG SA WG 1 Meeting #111 	S1-253363r2

25-29 August 2025, Goteborg, Sweden	(revision of S1-253277)

Source:	Huawei, HiSilicon, Rakuten

pCR Title:	new use case on Privacy protection for accessing 6G new services from partner PLMNs incl. during data exposure

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

* * * Frist Change (all new text) * * * *

### 5.5.x	Use Case on privacy protection for accessing 6G new services in partner PLMNs

#### 5.5.x.1	Description

Our experience in previous generations of 3GPP technologies shows their different uptake patterns. Even for a particular generation (e.g. 2G, 3G, 4G, 5G), the beginning of the network deployment and service commercialization varied significantly across different regions, countries and among operators. This is presumably resulted from differences in market demand, operator investment capital and their market strategies, etc. The perspective that 6G is provide multiple beyond-connectivity services and enhanced communication service is considered a factor further affecting the extent of the range in which 6G adoption is going to vary among different operators.

However, as an individual subscriber who has subscription with a particular operator, he/she may want/need to enjoy quite a few new 3GPP/6G services, and it may well be the case that some are not yet deployed by the home network operator. To bridge that gap, the 3GPP-provided capabilities to satisfy such user needs constitute a valuable opportunity for operators with varied 6G deployment to collaborate as partners (e.g. via service agreement) specially in the initial phase of 6G adoption, by enabling subscribers to consume 6G services provided by any of the partner operators. The 6G services can be pre-defined in the service agreement. According to service agreements between the home operator and the partner operators, a subscriber is aware of 6G services from the partner operators via the home operator. The subscriber can consume these 6G services provided by different partner operators after the subscriber is authenticated and authorized to access these 6G services e.g. by/via the home operator. For example (as shown in Fig.1), subscriber Alain of home network operator A can consume passive sensing services from A’s partner B’s PLMN_B, and Alain can consume AI services from A’s partner C’s PLMN_C, potentially at different point in time/location , or Alain can consumer AI inference or  6G Computing Service from A’s partner B’s PLMN_C.

Fig. 5.5.x.1-1. Home-routed access partner’s 6G new service (e.g. AI model training service)

The goal is to preserve user’s confidentiality protection (e.g., avoiding correlation of UE’s identifiers) when the subscriber’s UE accessing networks or services). In case the privacy sensitive information about UE’s identifiers may be leaked to undesired parties, the privacy of the user/subscriber can be protected. This may suggest the need for the subscriber’s UE to access partner operators’ various PLMNs with different temporary identifiers, to avoid partner operators or 3rd party (e.g. data consumer Diana) to fathom out when and from which PLMNs the very user/subscriber Alain has accessed a given service.

#### 5.5.x.2          Pre-conditions

As show in Fig.1, an operator #A has a traditional connectivity service for users and does not support e.g. passive sensing services and AI services. operator #B deploys a 6G new AI training service into some areas (e.g., rural areas where using an operator #A connectivity infrastructure (e.g., base stations). The operator #A has service agreements with operator #B. According to service agreements, operator #B can use the operator #A’s connectivity infrastructure to provide its own 6G new services to consumers. According to service agreements, the operator #A has/knows basic service packages (e.g., service ID, description of service) of AI inference/training service. According to service agreements, operator #B control/keep its own system data related to their own 6G new services, the operator #A only keeps basic service packages of AI training service. Similarly, according to service agreements, the operator #A has another partner #C which provides computing services into some areas (e.g., rural areas where using an operator #A connectivity infrastructure (e.g., base stations).

Alain subscribes a traditional connectivity service with an operator #A (termed as a home operator). Alain uses an AI service for his daily life (e.g., time, location, behavior), including an intelligent assistant in his phone. The intelligent assistant requires computational tasks such as model training. Alain wants to offload the AI service to the network (e.g., model training). According to service agreements, the home operator #A provides operator #C (termed as its partner #C)’s AI service. Operator #C offers AI services to operator #A’s subscriber through its own resources to handle compute-intensive tasks offloaded by Alain. Alain is able to be aware of the service packages of AI service that is provided by home operator’s partner #B.

Subject to service agreement among operators, national/regional regulation, and privacy regulation, the subscriber obtains an identifier to access and consume AI inference/training service from operator #B.

#### 5.5.x.3            Service Flows

AI service

- Alain wants to access an AI services for his daily life from operator #B using its identifier. Alain’s UE sends a AI service request to the home operator about access the AI service from operator #B.

- According to service agreements (e.g., service packages from partners), the home operator selects an AI service that is provided by partner #B based on requirements of AI service, information (e.g., UE type) about the UE.

- The home operator #A asks the operator #B whether the AI service request can be satisfied, e.g. by verifying the AI resource availability (e.g., available model training resource, available GPU/memory).

- If the AI service request is satisfied, the home operator authenticates and authorizes Alain to access the AI service from operator #B. The home operator assigns a token for Alain to access the AI service.

- After authentication and authorization, Alain accesses the AI service with the assigned token.

#### 5.5.x.4          Post-conditions

The partner #B provides its AI service to Alain.

#### 5.5.x.5	Existing features partly or fully covering the use cases functionality

There are requirements for network capability exposure that are partially related, as indicated in clause 6.10.2 on TS 22.261 [14]. However, the requirement is for a security logging information of UEs.

Based on operator policy, the 5G network shall expose a suitable API to provide the security logging information of UEs, for example, the active 3GPP security mechanisms (e.g. data privacy, authentication, integrity protection) to an authorized third-party.

The 5G system shall be able to:

-	provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party's service.

Subject to regulatory requirements and based on operator policy, the 5G system shall provide a mechanism to support confidentiality to prevent exposure of data exchanged between the 5G network and a third-party service provider. Subject to regulatory requirements and based on operator policy, the 5G system shall provide a mechanism to support data integrity verification service to assure the integrity of the data exchanged between the 5G network and a third-party service provider.

The 5G system shall provide suitable means to allow use of a trusted and authorized third-party provided integrity protection mechanism for data exchanged between an authorized UE served by a private slice and a core network entity in that private slice.

The 5G system shall provide suitable means to allow use of a trusted and authorized third-party provided integrity protection mechanism for data exchanged between an authorized UE served by a non-public network and a core network entity in that non-public network.

In TS 22.261 clause 8.5 includes the requirements of identity management. The 5G system shall provide a mechanism for an operator to allow access from a UE using a temporary identifier (e.g. 5G-GUTI) that hides its subscriber identity. The 5G system shall provide a mechanism for an operator to allow access from a UE connected in an indirect network connection using a temporary identifier that hides its subscriber identity. The HPLMN shall be able to associate a temporary identifier to a UE's subscriber identity. The existing requirements does not cover when a UE accesses one or more multiple beyond-connectivity services provided/shared by other PLMN(s), how to prevent/protect against link-ability and trackability when different data such as 6G System Data related to a certain user is exposed to a 3rd party (e.g. using different/multiple identifiers tht hide subscriber identity).

TS 22.261 clause 6.21 includes the requirements for Indirect Network Sharing. Indirect Network Sharing is a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network. The existing Indirect Network Sharing doesn’t cover that a UE only with a subscription to HPLMN can access one or multiple beyond connectivity service shared by other HPLMN’s (non-roaming) partner PLMN(s).

TS 22.261 clause 6.1 includes the requirements for Network slicing. According to the TS 22.26, a network slice can provide the functionality of a complete network, including radio access network functions, core network functions (e.g. potentially from different vendors) and IMS functions. In addition, the following requirement requires UE to access more than one VPLMN simultaneously if the network slices are not available in a single VPLMN:

- For UEs that have the ability to obtain service from more than one VPLMN simultaneously, the following requirements apply:

-	When a roaming UE with a single PLMN subscription requires simultaneous access to multiple network slices and the network slices are not available in a single VPLMN, the 5G system shall enable the UE to:

-	be registered to more than one VPLMNs simultaneously; and

-	use network slices from more than one VPLMN simultaneously

Requirements for identifying UE applications and users have been studied since Rel-18 (e.g. TS 22.904) as part of the UIA (SP-180328) work item). In Rel-19, SA3 studied how to protect user identifier privacy (TR 33.370). SA3 was not able to discuss about how to protect UE privacy UE when UE accessing different networks/services. In Rel-19, SA2 studied how to add support for human user identities (TR 23.700-32 as part of study of the FS_UIA_ARC). SA2 was not able to study on how to support UE access different networks/services with different identifiers.

#### 5.5.7.6	Potential New Requirements

[PR 5.5.7.6-1] Subject to operator policy, national and regulatory requirements, the 6G system shall support an authenticated and authorized UE to access home provider’s various partners’ 6G networks for consuming their 6G services.

[PR 5.5.7.6-2] Subject to operator policy, national and regulatory requirements, the 6G system shall support identifier privacy protection (incl. avoiding correlation of UE’s identifiers) for UE access home provider’s various partners’ 6G networks for consuming their 6G services.

[PR-5.3.6.6-3] Subject to agreement among operator, national and regional regulation, the 6G system shall provide a mechanism to protect privacy of the user and UE’s identities (incl. avoiding correlation of UE’s identifiers), when the UE accesses home provider’s various partners’ 6G networks for consuming their 6G services (e.g. possibly with different UE identifiers).

* * * End of Change * * * *