3GPP TSG SA WG 1 Meeting #112 	S1-254328

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254294)

Source:	6G Study Rapporteurs

pCR Title:	Merged Proposed changes on Clause Y.1.10 (ISAC)

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Rapporteur’s collection of proposed changes to CPRs in Clause Y.1.10

1. Introduction

The following TDOCs submitted into SA1 # 112 have been collected into this document to faciliitate discussion and approval of CPRs.

S1-254001, 2st Draft Agenda for SA1#112 was used as the basis of this contribution.

- S1-54020 (Clause Y.1.10 from zip file)

- S1-254074 (China Mobile) - Table Y.1.10-1

- S1-254162 (ZTE Corporation) - Table Y.1.10-1

- S1-254300r1 (Huawei)

This TDOC is the output of the initial consolidation discussions for ISAC.

2. Reason for Change

There are several proposals to consolidate functional requirements. This combines the proposals to facilitate discussions in the meeting.

3. Proposal

It is proposed to disscuss and agree the following changes for inclusion into Clause Y in 3GPP TR 22.870 v0.4.1.

* * * First Change: Y.1.10 and Table Y.1.10-1 – ISAC* * *

## Y.1.10	Integrated Sensing and Communication (ISAC)

Editor’s Note: This includes (but not limited to) PRs from clause 7.

NEW Editor’s Note: PR 7.17.6-1, PR 7.17.6-2, PR 7.17.6-3, PR 7.17.6-4, and PR 7.25.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.10-1 at this time.

NEW Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Company proposals

S1-254300r1 (Huawei): For ISAC clause, we will provide additional feedback/suggestion.

Table Y.1.10-1 – ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.10-1-1 | Subject to operator policy, regulation, and user consent, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to - aggregate, collect, process, and store sensing measurement data from base stations, - collect non-3GPP sensing data from third party, and - provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. | PR 7.5.6-1 PR 7.5.6-4 PR 7.16.6-1 | Sensing Ops Measure, aggregate, collect, process, store, deliver Collection of non-3GPP sensing data Table Y.1.5-1 – Data Collection and Consumption? Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-2 | Subject to operator policies, the 6G Network shall provide mechanisms to - provide a means to enable efficient use of sensing resources for stable sensing operation, - provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). - configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. For example, once a UAV is detected, the sensing service can be used to determine the type of UAV it is (e.g. quadcopter or hexacopter) and then group together UAVs with similar characteristics in each class. NOTE: Mechanisms for sensing operation configuration are in scope of ongoing 5GA work, without explicit 5GA service requirements. | PR 7.8.6-2 PR 7.14.6-1 PR 7.23.6-1 | (Stable) Sensing Ops Sensing Modes Multiple targets Proposed to be changed by S1-254162 |
| Y.1.10-1-3 | The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. | PR 7.5.6-3 | Sensing Service Capacity Multiple targets Proposed to be changed by S1-254162 |
| Y.1.10-1-4 | Based on operator policy, regional and/or national regulations, the 6G network shall provide secure mechanisms to - collect sensing results with a specified level of accuracy - support sensing target density requests by the third party for a given sensing service - provide a means to ensure a latency upper-bound, requested by trusted third party, when providing sensing results to nearby UEs (e.g. AMRs) about environment situation. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.2.6-1 PR 7.7.6-1 PR 7.8.6-3 | Accuracy Target Density Latency 3rd Party Support Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-5 | Subject to regulation and operator policy, the 6G system shall provide a target prediction capability that includes target characteristics (e.g. size, shape, location, velocity) while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.5.6-2 PR 7.10.6-1 PR 7.10.6-2 | Prediction Exposure Privacy Third Party support Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-6 | Subject to regulatory requirements, operator’s policy, and user consent, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored collected sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 | Stored Sensing Data Usage Security Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-7 | The 6G system shall be able to prioritize communication, sensing and positioning together used in Network Assisted Smart Transportation. | PR 7.19.6-2 | Prioritization Proposed to be changed by S1-254162 |
| Y.1.10-1-8 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to support - the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer, - activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party NOTE 1: This requirement is intended to describe multiple UEs (AMRs) collaborating to provide useful information for each other in areas shared by human and AMRs. For example, a UE (AMR) will have an ample time to stop or slow down, using the information on presence of multiple human workers in a few seconds. | PR 7.8.6-1 PR 7.14.6-2 | Exposure Third party support Delivery synchronization Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-9 | Subject to operator’s policy, regulation and user consent, the 6G network shall be able to atuhorise and expose sensing results to UE or an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 | Exposure Proposed to be changed by S1-254074 Proposed to be changed by S1-254162 |
| Y.1.10-1-10 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | ISAC Linking Proposed to be changed by S1-254162 |
| Y.1.10-1-11 | Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker. NOTE: The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements. | PR 7.24.6-1 | Gesture Recognition Proposed to be changed by S1-254162 |
| Y.1.10-1-12 | Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker. NOTE: The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements. The 6G system shall be able to support the KPIs from Table 7.3.6-1: KPIs for safety assistance for vulnerable pedestrians | PR 7.3.6-1 | KPI Safety Assistance Vulnerable Peds Proposed to be changed by S1-254162 |
| Y.1.10-1-13 | The 6G system shall be able to support the KPIs. From Table 7.4.6-1: KPIs for high-resolution topographical maps | PR 7.4.6-1 | KPI Mapping |
| Y.1.10-41-1 | The 6G system shall be able to provide sensing with KPIs [9] [11] from Table 7.5.6-1: Performance requirements of sensing results for low-altitude UAV supervision | PR 7.5.6-5 | KPI Low Alt UAV supervision |
| Y.1.10-1-15 | The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental objects as described in the Table 7.6.6-1: KPIs for environment object reconstruction | PR 7.6.6-1 | KPI Environmental object reconstruction |
| Y.1.10-1-16 | The 6G system shall be able to support the KPIs from Table 7.7.6-1: Performance requirements for road digitalization | PR 7.7.6-2 | KPI Road digitalization |
| Y.1.10-1-17 | The 6G system shall be able to provide sensing service with larger coverage as described in Table 7.9.6-1: Performance requirements for 6G ship detection and tracking | PR 7.9.6-1 | KPI Large coverage (ship detection/tracking) |
| Y.1.10-1-18 | The 6G system shall support the KPIs in Table 7.13.6-1: Performance requirements of sensing results for enhanced XR navigation | PR 7.13.6-2 Should be -1 | KPI Enhanced XR nav |
| Y.1.10-1-19 | The 6G system shall be able to support the KPIs from Table 7.14.6-1: Proposed KPIs for collaborative robots using digital twinning | PR 7.14.6-3 | KPI Cobots using DT |
| Y.1.10-1-20 | The 6G system needs to be capable of sensing the infrastructure collapse with KPIs from Table 7.15.6-1: Performance requirements of sensing results for infrastructure collapse monitoring | PR 7.15.6-1 | KPI Infrastructure monitoring |
| Y.1.10-1-21 | The 6G system shall be able to provide sensing service by enabling multi-sensor fusion based sensing with KPIs from Table 7.16.6-1: Performance requirements for Multi-Sensor Fusion based sensing for UAV takeoff | PR 7.16.6-2 | KPI Multi-sensor fusion |
| Y.1.10-1-22 | The 6G system shall support the KPIs in Table 7.18.6-1: Performance requirements for Safe and Economic UAV transport | PR 7.18.6-1 | KPI UAV Transport |
| Y.1.10-1-23 | The 6G system shall be able to offer this Network assisted Smart Transportation service by ensuring the KPIs for communication, location and sensing according to Table 7.19.6-1: Performance requirements are fulfilled simultaneously. | PR 7.19.6-1 | KPI Simultaneous performance metrics |
| Y.1.10-1-24 | The 6G system shall be able to support the KPIs in Table 7.22.6-1: Performance requirements of sensing results for structural health monitoring | PR 7.22.6-1 | KPI Structural Monitoring |
| Y.1.10-1-25 | Table 7.23.6-1: Performance requirements for UAV Detection, Classification and Counting | Clause 7.23 (no PR#) | KPI |
| Y.1.10-1-26 | The 6G System shall support the sensing KPIs for gesture recognition in industrial environments from Table 7.24.6-1: KPIs for gesture recognition in industrial environments [94] | PR 7.24.6-2 | KPI Gesture Recognition |

Company Proposals:

- Y.1.10-1-1:

  - S1-254074: (China Mobile)

| Y.1.10-1-1 | Subject to operator policy, regulation, and user consent, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to - aggregate, collect, process, and store sensing measurement data from base stations, - collect non-3GPP sensing data from third party, and - collect the sensing results with a specified level of accuracy. - . | PR 7.5.6-1 PR 7.16.6-1 only this one can be consolidated as it is PR 7.2.6-1 The accuracy PR should be consolidated separately | Sensing Ops Measure, Aggregate, Collect, Process, Store, Deliver Collection of 3GPP and non-3GPP Sensing Data Collection Accuracy Table Y.1.5-1 – Data Collection and Consumption? |
| --- | --- | --- | --- |

b. S1-254162 (ZTE)

| Y.1.10-1-1 | Subject to operator policy, regulation, and user consent, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to - aggregate, collect, process, and store sensing measurement data from base stations. | PR 7.5.6-1 | Sensing Ops Measure, aggregate, collect, process, store, deliver Collection of non-3GPP sensing data Table Y.1.5-1 – Data Collection and Consumption? [ZTE] non-3GPP sensing data is not the sensing data from BS. It is suggested not to merge together. To provide sensing result to UE is another issue which should not be merged together. |
| --- | --- | --- | --- |
| Y.1.10-1-x (new 2) | Subject to regulatory requirements, operator’s policy, the 6G network should support suitable means to collect non-3GPP sensing data from third party if available. | PR 7.16.6-1 | [ZTE] Non-3GPP sensing data collection Considering the sensing data collection is specific, it is suggested to remain it in Y.1.10 |

- Y.1.10-1-2:

  - S1-254162: (ZTE)

| Y.1.10-1-3 | Subject to operator policies, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. | PR 7.23.6-1 | (Stable) Sensing Ops Sensing Modes Multiple targets [ZTE] It is suggested not to merge them together. Classify objects |
| --- | --- | --- | --- |
| Y.1.10-1-x (new 4) | Subject to operator policies, the 6G Network shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). | PR 7.14.6-1 | [ZTE] sensing modes configuration |
|  | The 6G system shall be able to provide a means to enable efficient use of sensing resources for stable sensing operation. | PR 7.8.6-2 | [ZTE] what is the meaning of “stable sensing operation”? |

- Y.1.10-1-3:

  - S1-254162: (ZTE)

| Y.1.10-1-5 | Subject to regulation and operator’s policy, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.5.6-3 PR 7.7.6-1 PR 7.12.6-2 | Sensing Service Capacity Multiple targets ]ZTE] Sensing QoS: capacity/density/latency upper-bound |
| --- | --- | --- | --- |
|  | Subject to regulation, operator policy and user consent, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data. NOTE: The processing of sensing data is compliance with applicable regulations, and other pre-defined service criteria. | PR 7.12.6-2 |  |

- Y.1.10-1-4:

  - S1-254074: (China Mobile)

| Y.1.10-1-4 | Subject to regulation and operator's policy, 6G system shall support sensing target density requested by the third party for a given sensing service. | PR 7.7.6-1 | Target Density |
| --- | --- | --- | --- |

  - S1-254162: (ZTE)

| Y.1.10-1-6 | Based on operator policy, regional and/or national regulations, the 6G network in the area of the disaster shall provide secure mechanisms to collect sensing results with a requested level of accuracy that can be used to generate real time maps. | PR 7.2.6-1 | Accuracy Target Density Latency 3rd Party Support [ZTE] it is suggested not to merge together which are targeting different things. 7.2.6-1 is for secure method to collect, 7.8.6-3 is for latency upper-bound while 7.7.6-1 is for density. |
| --- | --- | --- | --- |

- Y.1.10-1-5:

  - S1-254074: (China Mobile)

| Y.1.10-1-5 | Subject to operator’s policy and regulation, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s). | PR 7.10.6-1 | Prediction |
| --- | --- | --- | --- |

  - S1-254162: (ZTE)

| Y.1.10-1-7 | Subject to regulation and operator policy, the 6G network shall provide a target prediction capability to derive predicted target characteristics (e.g. size, shape, location, velocity), while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-1 PR 7.10.6-2 | Prediction Exposure Privacy Third Party support [ZTE] PR7.5.6-2 is moved to EE part Y.1.4. |
| --- | --- | --- | --- |

- Y.1.10-1-6:

  - S1-254074: (China Mobile)

| Y.1.10-1-6 | Subject to regulatory requirements, operator’s policy, and user consent, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 | Sensing Data Storage Usage & Security |
| --- | --- | --- | --- |

  - S1-254162: (ZTE)

| Y.1.10-1-8 | Subject to regulatory requirements, operator’s policy, and user consent, 6G system shall support the use of stored sensing data to provide a sensing service and protect data privacy during the processing of sensing data, ensure that only authorised entities are able to access the stored collected sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 PR 7.12.6-2 | Stored Sensing Data Usage Security |
| --- | --- | --- | --- |

- Y.1.10-1-7:

  - S1-254162: (ZTE)

| Y.1.10-1-9 | The 6G system shall be able to prioritize communication, sensing and positioning together used in Network Assisted Smart Transportation. | PR 7.19.6-2 | Prioritization |
| --- | --- | --- | --- |

- Y.1.10-1-8:

  - S1-254074: (China Mobile)

| Y.1.10-1-8 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to support - the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer, - activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party - a latency upper-bound requested by trusted third party, when providing sensing results to nearby UEs (e.g. AMRs) about environment situation. NOTE 1: This requirement is intended to describe multiple UEs (AMRs) collaborating to provide useful information for each other in areas shared by human and AMRs. For example, a UE (AMR) will have an ample time to stop or slow down, using the information on presence of multiple human workers in a few seconds. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.8.6-1 PR 7.14.6-2 PR 7.8.6-3 | Exposure Mechanism Enhancement Third party support Delivery synchronization Latecy upper bound |
| --- | --- | --- | --- |

  - S1-254162: (ZTE)

| Y.1.10-1-10 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party | PR 7.8.6-1 | Exposure Third party support Delivery synchronization ZTE: 7.8.6-1: Sensing result to UE: activate and deactivate; 7.14.6-2: Sensing result exposure sync.with other traffic, It is suggested to not merge together |
| --- | --- | --- | --- |
| Y.1.10-1-x (new 11) | The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer. | PR 7.14.6-2 | ZTE: 7.14.6-2: Sensing result exposure sync.with other traffic, |

- Y.1.10-1-9:

  - S1-254074: (China Mobile)

| Y.1.10-1-9 | Subject to operator’s policy, regulation and user consent, the 6G network shall - autthorise and expose sensing results to UE or an application on the UE for a specific service, - provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator provding sensing service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 | Sensing Results Exposure to UE |
| --- | --- | --- | --- |

  - S1-254162: (ZTE)

| Y.1.10-1-12 | Subject to operator’s policy, regulation and user consent, the 6G network shall be able to authorise UE and expose sensing results to an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 | Exposure to UE |
| --- | --- | --- | --- |
|  | Subject to operator’s policy, regulation and user consent, the 6G network shall be able to provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. | PR 7.5.6-4 |  |

- Y.1.10-1-10 & -11

  - S1-254162: (ZTE) – just renumbering due to inserted CPRs.

| Y.1.10-1-13 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | ISAC Linking |
| --- | --- | --- | --- |
| Y.1.10-1-14 | Subject to regulatory requirements and user permission, the 6G network shall be able to use the 6G sensing service to monitor and recognize human gestures. | PR 7.24.6-1 | Gesture Recognition |

- Y.1.10-1-12

  - S1-254162: (ZTE) – a comment, no proposed changes.

| Y.1.10-1-12 | Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker. NOTE: The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements. The 6G system shall be able to support the KPIs from Table 7.3.6-1: KPIs for safety assistance for vulnerable pedestrians | PR 7.3.6-1 | KPI Safety Assistance Vulnerable Peds [ZTE] what is the source of the content? |
| --- | --- | --- | --- |

- New CPRs to Table Y.1.10-1:

  - S1-254074: (China Mobile)

| Y.1.10-1-27 | Subject to user consent, regulation, and operator’s policy, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party, while maintaining the privacy of the sensing target(s). | PR 7.10.6-1 PR 7.10.6-2 | Sensing Data Security and Privacy Protection For Sensing Data, Prediction, Results and Targets |
| --- | --- | --- | --- |
| Y.1.10-1-28 | The 6G system shall be able to support energy-efficient sensing operations. | PR 7.5.6-2 To be consolidated with 7.8.6-2 | Energy-efficient Sensing Operations |

* * * End of Changes * * * *