3GPP TSG-SA WG1 Meeting #112	S1-254423

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254018r3, S1-253383)

Source:	Ericsson, Itron, AT&T, Verizon, Qualcomm, Sony, EDF, Nokia, KPN, Philips

pCR Title:	Update Utility use case 10.3

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.7 Massive Communication

Document for:	Agreement

Contact:	Daniel.lonnblad at ericsson.com

Abstract: This contribution proposes requirements and additional KPI for use case 10.3

1. Introduction

This is introducing requirements and additional KPI for use case 10.3

2. Reason for Change

The use case for the Massive Communication is updated with KPI’s for the latency, device density and trafic inteval etc. Further more a requirements for low complexity is added as well as optimization of energy needs to be considered both in network and device.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1 chapter 10.3

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	 EPRI report “Falling Conductor Protection” https://distribution.epri.com/wildfire/public/wildfire-tech-database/grid-monitoring/downed-conductor-detection/

* * * Second Change * * * *

### 10.3.5	Existing features partly or fully covering the use case functionality

In TS 22.261 [14] clause 6.4 Resource efficiency states some general requirements for IoT devices.

The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

The 5G system shall minimize control and user plane resource usage for stationary UEs (e.g. lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of continuous uplink data that requires both high data rate (e.g. 10 Mbit/s) and very low end-to-end latency (e.g. 1-10 ms).

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g. 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	type of mobility support;

-	communication pattern (e.g. send-only, frequent or infrequent);

-	characteristics of payload (e.g. small or large size data payload);

-	characteristics of application (e.g. provisioning operation, normal data transfer);

-	UE location;

-	timing pattern of data transfer (e.g. real time or non-delay sensitive).

The 5G system shall efficiently support service discovery mechanisms where UEs can discover, subject to access rights:

-	status of other UEs (e.g. sound on/off);

-	capabilities of other UEs (e.g. the UE is a relay UE) and/or;

-	services provided by other UEs (e.g. the UE is a colour printer).

The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g. consolidating data transmissions to 1 larger rather than many smaller), when applicable (e.g. providing service in an area subject to power outages).

The 5G system shall support small form factor UEs with single antenna.

NOTE:	Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wavelength.

In TS 22.261 [14] clause 6.15.2 Resource efficiency states some general requirements for IoT devices.

The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on maximum pulse and continuous current).

In TS 22.261 [14] clause 8 Security, security requirement for IoT devices is listed.

In TS 22.104 [64] Annex A4.7 Advanced metering following proposed requirement can be found.

Table 10.3.5-1: Communication KPI for advanced metering (Table A.4.7-1 from TS 22.104 [64])

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 – Advanced Metering | > 99.99 |  | Accuracy fee control: < 100 (note 1); General information data collection: < 3000 | UL: < 2 M DL: < 1 M | – | – | – | stationary | < 10 000/ km2 (note 2) | – |
| NOTE 1: One-way delay from 5G IoT device to backend system. The distance between the two is below 40 km (city range). NOTE 2: It is the typical connection density in today city environment. With the evolution from centralised meters to socket meters in the home, the connection density is expected to increase 5 to 10 times. |  |  |  |  |  |  |  |  |  |  |

### 10.3.6	Potential New Requirements needed to support the use case

[PR 10.3.6-1] The 6G system shall support communication service with the following KPI’s in Table 10.3.6-1

[PR 10.2.6-3] The 6G system supporting Massive Communication shall support low-complexity devices.

NOTE: 	Low-complexity devices are needed for massive deployment into next-generation smart meters. This may include low-power capabilities and limitations in the amount of antennas.

[PR 10.2.6-4] Based on operator policy, the 6G system supporting Massive Communication shall support an authorized 3rd party to be able to set and configure parameters e.g., if the application depends on MO or MT traffic, small or high-data volumes.

Table 10.3.6-1: KPI for M-IoT Smart Grid monitor and control

| Profile |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link (Mbit/s) | Bit rate up link (Mbit/s) | End-to-end latency: maximum (ms) (NOTE 4) | Payload size | # of UEs connection | Communication service availability: target value (NOTE 4) | Transfer Interval (sec) (NOTE 5) |
| Smart Grid (monitor and control) | Peak 5 | Peak5 | 100 (NOTE 2) | 0.5Kbyte (NOTE 1) | 10 000/km2 | > 99.99 | 60 |
| Smart Grid (software download) | Peak [5] |  | - | Up to 20MB (NOTE 3) | 10 000/km2 | > 99.99 | - |
| NOTE 1: Typical message sizes are 500-byte payload meter readings and control. NOTE 2: Reference for requirement of 100ms EPRI report “Falling Conductor Protection” [y]. NOTE 3: up to 20MB for Firmware upgrade. NOTE 4: As defined in TS 22.261[14] NOTE 5: As defined in TS 22.104[64] |  |  |  |  |  |  |  |

* * * End Change * * * *