3GPP TSG-SA WG1 Meeting #112	S1-254435

Dallas, USA, 17-21 November 2025	(revision of S1-254351, 4188r2, 3254)

Source:	Nokia

pCR Title:	New Use case on Energy-aware data collection considering UE preference

Draft TS/TR:	3GPP TR 22.870 v.0.4.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to introduce a new use case and requirements related to energy-aware data collection. In particular, a UE preference is defined to temporarily avoid to collect data from the UE, which could drain the device battery.

R4435

- Changed “exclude” to ‘limit interactions related to”, and “from the UE” to “involving the UE” [Qualcomm]

- Added “/permission” to “subscriber preference” [Huawei]

- Replaced “user” with “subscriber” in NOTE [Huawei]

Revision 4351

- Simplified the use case to focus on PR1

- Removed PR2 & 3 accordingly

Revision r2

- Updated description and PR1 to be more explicit re “data collection” [vivo, Huawei]

Revision SA1#112 r1

- Fixed typos [TMO US]

- Added step 5 in service flows (to align with figure) [TMO US]

- Added ref to the figure in the text [TMO US]

- Added ref to TS 22.261 [TMO US]

- PR1: reverted to “temporarily exclude” vs “limit interactions” with the UE [Qualcomm, ZTE]. Clarified relationship with data collection at the UE [ZTE, Huawei] and that the requirement applies “during processing to satisfy requests” [Futurewei].

- PR2: replaced “system” with “network” [Qualcomm]

Revision SA1#112

- Reworded

  - “network API “ to “network exposure”

  - “involvment of the UE” with “interaction with the UE”

  - “fulfillment” with “serving”

- Clarified description and service flows

- Clarified that the UE preference has no impact on UE with regard to 3GPP services beyond network exposure

- Add examples beyond location (e.g. sensing, analytics, data collection in general)

---------- Use Case template ----------

FIRST CHANGE (new text)

### 5.8.x	Use case on Energy-aware data collection considering UE preference

#### 5.8.x.1	Description

Several new 6G services such as sensing, high precision positioning, data and AI-based services (e.g. AIML model training) are expected to trigger some interaction with the UE, e.g. to collect data.

This has a direct impact on the energy consumption of the UE, especially for repeated requests over long periods of time for e.g. tracking of IoT devices. Furthermore, this can correspond to background traffic which the user of the UE could not be aware of.

Collecting data from the UE upon network decision increases UE energy consumption, which may not be the user preference when e.g. the UE battery is low or to make it last longer.

#### 5.8.x.2	Pre-conditions

John is managing the subscriptions and services related to the fleet of his company assets, including vehicles, smartphones, and various devices equipped with trackers displaced at various company locations, or rent to customers.

His operator provides him with a web portal to track the locations of the various assets and other information or statistics requiring some data collection at these UEs.

John can set a preference to minimize the interaction with some of its UEs to collect data e.g. based on their battery level, battery saving mode, or roaming status.

#### 5.8.x.3	Service Flows

- John sets its UE preference settings for data collection in the portal. Such preference settings may be applicable for all or selected services.

- These settings are provided by the web portal to the network.

- The network considers the UE preference to select the best method for collecting data from the UE if needed.

- (optional) Only if needed and if the UE preference allows, the network interacts with the UE to collect data.

The service flows are illustrated in Figure 5.8.x.3-1.

Figure 5.8.x.3-1: Energy-aware data collection considering UE preference

#### 5.8.x.4	Post-conditions

John’s UE(s) preference settings can apply to all or selected services. The network takes into account John’s UE(s) preference settings for data collection.

#### 5.8.x.5	Existing features partly or fully covering the use cases functionality

N/A

#### 5.8.x.6	Potential New Requirements needed to support the use case

[PR 5.8.x.6-1] Subject to operator’s policy and regulatory requirements, the 6G system shall provide means (e.g. via a subscriber’s preference/permission) to temporarily limit interactions related to data collection involving the UE.

NOTE:	A subscriber can prefer that its UE(s) will not be involved in data collection, due to e.g., low battery, privacy concerns, roaming status etc.

-

END OF CHANGES