SA WG2 Meeting #171	S2-2508445

13 – 17 Oct, 2025, Wuhan , CN

Source:	TCL

Title:	[WT#5] Data Framework Working Task Scope

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The contribution propose that a new data plane shall be studied in WT#5..

Discussion

In the last meeting, whether the end-to-end data framework adopts new data plane or enhancing CP/UP is not determined. From our view, it is essential to design a new data plane rather than enhancing CP/UP. The reasons are mentioned as follows.

- In 5G system, there have been some methods to transmit data (e.g., AI training data, AI model, sensing data, etc.) through control plane, such as MDT, LPP, L1/L3 measurements, etc. While such kind of transmission methods cannot guarantee the quality of service and the transmission capacity is limited. In addition, data transmission through CP could influence the control information delivery and the reliability of control plane may not be necessary for the requirements of the data.

- For the user plane, although QoS of data transmission can be guaranteed, all data should pass the UPF. However, considering the security and privacy of user and the low latency requirement of tasks, it is suboptimal to transmit data to core network or data network.

Therefore, enhancing CP/UP may reduce the workload of the specification setting work, but a new data plane will be better for future 6G data services.

Proposal

It is proposed to add the following modifications of the WT#5 scope to the TR 23.801-01 “Study on Architecture for 6G System”.

**** First Change ****

A.X 	Work Task X: 6G unified data framework

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

Data categories can be handled/supported by 6G data framework studied in SA2:

  - Data from Core network and 3rd AF

Option 1:

Editor’s Note: whether the following data can also be handled/supported by 6G data framework in SA2 will be revisited:

  - Data from UE (e.g. for Sensing, UE-sided model training or NF-sided model training, etc.)

  - Data from RAN (e.g. for Sensing, two-sided model training or NF-sided model training, etc.)

  - Data/analytics from OAM (e.g. invoking MDAF service by CN for NF-sided model training)

Option 2:

Editor’s Note: what other data can also be handled/supported by 6G data framework in SA2 will be revisited.

The scope of the WT includes the following aspects:

- Design end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane and interfaces, to support efficient and scalable data handling functionalities, e.g.:

- data collection

- data transfer and distribution

- data processing e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs

- data storage within 6G system

- data access

- data exposure within 6G system and to outside of 6G system

NOTE 1:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

- Study from SA2 architectural and system level on user consent, privacy, security and data governance to support the 6G data framework.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

Study how to support quality of data which is processed and provided based on the requirements from data requester.

**** End of Changes ****