3GPP TSG-SA WG2 Meeting #171	S2-2508529

# October 13-17, 2025 Wuhan, China

Source:	Skylo Technologies

Title:	New Sol to KI#2

Agenda item:	20.8.1

Document for:	Agreement

Work Item / Release: FS_SMS2EC_ARC/Rel-20

Abstract of the contribution: This paper propose sol to KI#2

# 1. Discussion

To support the SMS delivery to the Emergency Response Centre via NAS based on local regulations, following aspects are included for KI#2:

-	study how to route the SMS via NAS procedures for the support of SMS delivery to local Emergency Response Centre, e.g. UE location determination, via SMS service centre of the serving PLMN;

NOTE 1:	Identification of SMS delivery to Emergency Response Centre is based on the conclusion of other KIs.

-	how to support Emergency Response Centre callback for sending short messages using regular SMS delivery service.

The key issue covers SMS delivery via NAS both in EPS and 5GS.

NOTE 2:	The solutions for EPS and 5GS are based on same principles.

The key issue covers both roaming and non-roaming scenarios.

The proposed solution addresses the above aspects.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-65 V0.1.0.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| #X |  | X |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

* * * Second Change * * * *

## 6.X		Solution #X: SMS over NAS in 4G/5G to Support Emergency SMS

### 6.X.0		High-level solution Principles

To support SMS delivery to Emergency Response Centres via NAS, the following high-level solution principles apply for both EPS and 5GS:

- The UE may include an explicit emergency SMS indication towards MME/AMF.

- For cases where the UE cannot detect the emergency SMS, the message is transmitted as a normal SMS, with identification of emergency SMS performed by the network.

- Upon detection of an emergency SMS, the MME/AMF may trigger UE location retrieval.

- Location information may be provided together with the emergency SMS towards the SMSC/SMS Router for delivery to the Emergency Response Centre.

- The MME in EPS or SMSF in 5GS forwards the SMS TPDU to the appropriate SMS-GMSC/SMS-IWMSC/SMS Router for onward routing.

- The SMSC, based on emergency header information, forwards the SMS to the operator’s dedicated emergency service network or directly to the Emergency Response Centre, in accordance with local regulatory requirements.

These principles apply to both roaming and non-roaming scenarios. Callback from the Emergency Response Centre to the UE is supported by reusing the regular SMS delivery service. The SMS from the Emergency Response Centre is delivered via the standard SMS path through the serving PLMN’s SMSC.

### 6.X.1	Description

In EPS, the UE delivers the emergency SMS using SMS over NAS. If the UE is in idle mode and detects the intent to send an emergency SMS, it initiates the UE-triggered Service Request procedure. If both the UE and the MME support Control Plane (CP) CIoT EPS Optimisation for SMS transfer, the UE initiates a Control Plane Service Request (CPSR).

When CP CIoT EPS Optimisation is used, the UE encapsulates the SMS-SUBMIT TPDU in a CPSR message towards the MME. Otherwise, the emergency message is encapsulated within the Uplink NAS Transport message. The UE may include an explicit emergency SMS indicator in the NAS message to inform the MME that the payload contains an emergency message in the SMS TPDU.

If the SMS is identified as an emergency SMS, the UE may include an emergency indication. The MME may use this indication to trigger UE location retrieval using EPC Network Induced Location Request (EPC-NI-LR) as specified in clause 9.1.17 of TS 23.271 [x], and/or bypass applicable barring checks. The MME then forwards the SMS TPDU, along with the obtained location information, to the SMS-GMSC/SMS-SMS Router over the SGd interface using the MO-Forward-Short-Message-Request procedure. The SMSC subsequently delivers the emergency SMS to the Emergency Response Centre in accordance with local regulatory requirements.

In 5GS, the UE delivers the emergency SMS via an Uplink NAS Transport message towards the AMF. When the SMS is identified as an emergency SMS, the UE includes an emergency indication in the NAS message. The AMF may use this indication to bypass barring checks and/or to trigger UE location retrieval using the 5GC Network Induced Location Request (5GC-NI-LR) as specified in clause 6.10 of TS 23.273 [y].

The AMF forwards the SMS to the serving SMSF using the Nsmsf_SMService_UplinkSMS operation. The SMSF examines the SMS header and, if the message is an emergency SMS, forwards it to the appropriate SMS-GMSC/SMS-IWMSC/SMS Router, which delivers the message to the SMSC. The SMSC, based on emergency header information, routes the SMS to the Emergency Response Centre as required by local regulation.

Callback from the Emergency Response Centre to the UE is supported using the regular SMS delivery service. The Emergency Response Centre sends the SMS via the operator’s SMSC, and the SMS is delivered to the UE over the standard downlink SMS path.

### 6.X.2			Procedures

#### 6.X.2.1	SMS in MME for 4G

Figure 6.X.2.1-1 shows the procedures for SMS delivery to the Emergency Response Centre via SMS over NAS using the SMS in MME architecture option as described in Annex C of TS 23.272 [7].

Figure 6.X.2.1-1: Procedures for Emergency SMS Delivery via SMS over NAS in EPC

- The UE attaches to the network using either Initial attach or Emergency Attach.

- If the UE is in idle mode and detects the intent to send an emergency SMS, it initiates the UE-triggered Service Request procedure. If both the UE and the MME support Control Plane (CP) CIoT EPS Optimisation for SMS transfer, the UE initiates a Control Plane Service Request (CPSR).

When CP CIoT EPS Optimisation is used, the CPSR message carries the SMS-SUBMIT TPDU directly. Otherwise, in step 1a, the emergency message is encapsulated within the Uplink NAS Transport message.

The UE may include an explicit emergency SMS indicator in the NAS message to inform the MME that the payload contains an emergency message in the SMS TPDU.

2.	If the UE does not know its own location, the UE may retrieve its location information from the network. Or based on the emergency indication, the MME may trigger UE location retrieval using EPC Network Induced Location Request (EPC-NI-LR) in step 2a as specified in clause 9.1.17 of TS 23.271 [x], and/or bypass applicable barring checks.

3.	The MME selects the target SMS-GMSC, SMS-IWMSC, or SMS Router, and forwards the SMS TPDU along with the location information using the MO-Forward-Short-Message-Request (OFR) message over the SGd interface, as specified in TS 29.338 [x1].

4.	The SMS-GMSC, SMS-IWMSC, or SMS Router, based on the destination number and SMS-TPDU header information, routes the message to the appropriate SMSC.

5.	The SMSC, based on the emergency header information, forwards the message to the operator’s dedicated emergency service network. For example:

  - Advanced Mobile Location (AML) over NAS/SGd, defined in GSMA NG.119 [x2], allows the visited SMSC to send the SMS directly to the PSAP or Emergency Call Center network.

  - Text-to-9-1-1, as specified in J-STD-110 [x3], enables the SMSC to forward the SMS via the SMPP protocol to the Text Control Center (TCC), which functions as a gateway to the PSAP.

6.	The Delivery Report is sent to UE.

#### 6.X.2.2		SMS over NAS for 5G

Figure 6.X.2.2-1 shows the procedures for SMS delivery to the Emergency Response Centre using SMS over NAS for 5G.

Figure 6.X.2.2-1: Procedures for Emergency SMS Delivery via SMS over NAS in 5G

- The UE follows the procedures specified in clause 4.13.3 of TS 23.502 [3] for registration with SMS over NAS support. During the Registration procedure in 5GS, the UE includes an “SMS supported” indication in the Registration Request to indicate its capability for SMS over NAS transport.

- If the UE is in CM-IDLE state and intends to send an uplink SMS message, the UE and the network perform the UE-triggered Service Request procedure as defined in clause 4.2.3.2 of TS 23.502 [3] to establish a NAS signalling connection to the AMF.

If the UE is in CM-CONNECTED state, this step is skipped, and the procedure continues with step 2.

- The UE builds the SMS message.

For an emergency SMS, the UE encapsulates the SMS in an Uplink NAS Transport message and includes an emergency SMS indication. The Uplink NAS Transport message is then sent to the AMF.

For an emergency SMS that is not detectable by the UE (e.g. based on number or configuration), the SMS is encapsulated in an Uplink NAS Transport message without the emergency indication and sent to the AMF.

- If the UE does not know its own location, the UE may retrieve its location information from the network. Or based on the emergency SMS indication, the AMF may trigger UE location retrieval, using the 5GC Network Induced Location Request (5GC-NI-LR) in step 3a, as specified in clause 6.10 of TS 23.273 [y], and/or bypass applicable barring checks.

- The AMF forwards the SMS to the appropriate SMSF serving emergency SMS by invoking the Nsmsf_SMService_UplinkSMS service operation.

- The SMSF inspects the SMS header.

If the message is identified as an emergency SMS, the SMSF forwards it to the appropriate SMS-GMSC, SMS-IWMSC, or SMS Router.

The SMSF then returns the SMS-Submit Report to the AMF using the Namf_Communication_N1N2MessageTransfer service operation. The AMF delivers the report to the UE via Downlink NAS Transport as described in clause 4.13.3.3 of TS 23.502 [3].

- The SMS-GMSC, SMS-IWMSC, or SMS Router, based on the destination number and SMS-TPDU header information, routes the message to the appropriate SMSC.

- The SMSC, based on the emergency header information, forwards the message to the operator’s dedicated emergency service network. For example:

  - Advanced Mobile Location (AML) over NAS/SGd, defined in GSMA NG.119 [x2], enables the visited SMSC to deliver the SMS directly to the PSAP or Emergency Call Center network.

  - Text-to-9-1-1, as specified in J-STD-110 [x3], enables the SMSC to forward the message via the SMPP protocol to the Text Control Center (TCC), which functions as a gateway to the PSAP.

- The Submit Report is sent to UE.

### 6.X.3	Impacts to Services, Entities and Interfaces

## MME:

- MME uses Emergency SMS indication to trigger location request

AMF:

- AMF uses Emergency SMS indication to trigger location request

UE:

- UE indicates Emergency SMS indication towards MME/AMF

SMSC:

- SMSC uses Emergency SMS header information to forward the message to a proper emergency service network

* * * Third Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[7]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[8]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[9]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[x]	3GPP TS 23.271: “Functional stage 2 description of Location Services (LCS)”.

[y]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”.

[x1]	3GPP TS 29.338: “Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)”.

[x2]				GSMA NG.119: “Emergency Communication”.

[x3]		ATIS J-STD-110: “Joint ATIS/TIA Implementation Guideline for J-STD-110, Joint ATIS/TIA 	Native SMS/MMS Text to 9-1-1 Requirements and Architecture Specification”.

* * * End of Changes * * * *