SA WG2 Meeting #171	S2-2508603

13 – 17 October, 2025, Wuhan, CN

Source:	Apple

Title:	[KI’s #4, #5] Revision of Solution #23

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposes a revision of Solution #23 to address ENs.

# 1.	Discussion

This contribution proposes a revision of Solution #23 to address ENs.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.700-14.

**** First Change ****

## 6.23	Solution #23: Distributed Sensing Data Processing

This solution addresses KIs #4 and #5.

### 6.23.0	High-level solution principles

The following high-level solution principles apply to this solution:

-	A solution supporting multiple sensing data consumer entities (e.g. Sensing Processing Function).

-	The solution supports multiple sensing processing and consumers of the 5GS by leveraging 5GS functions and services (i.e. processing, consumption of sensing data is done in the 5GS).

### 6.23.1	Description

It is anticipated that while sensing is ongoing, one or multiple sensing entities will generate sensing data that requires processing to obtain a final sensing result. This processing may require multiple and independent processing steps (e.g. a user, sensing subscriber, has to include proprietary fusion algorithms or external AIML models for sensing processing). To ensure that processing is fast, reliable, flexible and scalable, it is proposed that multiple Sensing Processing Functions (SePF) are supported. This might require supporting the chaining of multiple SePFs to get a final Sensing Result (or multiple Sensing Results). Furthermore, this proposal allows the selection of a single SePF or multiple SePFs. In case multiple SePFs are used, this solution provides a procedure which allows to deploy a Sensing Processing chain, which is scalable and flexible.

Hence, the motivation for this solution is computational flexibility for sensing processing. It is indeed anticipated that complex processing techniques may be employed on the sensing measurements, compounded by the fact that the measurements may be collected in multiple Sensing Entities. Splitting the processing by means of concatenating and applying different computational techniques on potentially different parts of the measurement datasets can bring benefits in the accuracy of the sensing results.

However, the proposed split of the processing need not be mandated for all scenarios and use cases and it can be optionally applied on the cases requiring processing of high complexity and modularity. The term processing is used as a general term which includes different kinds of compute and inference processes such as fusion, AIML and rule-based processing. It is also possible that a SePF may be specialized only in one type of computing algorithms, e.g. AI/ML. How the processing is implemented is out of scope of this solution.

The proposed procedures enable, for instance, an MNO to offer 5G services that meet customer requirements and provide flexibility for processing in the 5G network.

NOTE:	The split of sensing processing can also be left up to SePF implementation.

#### 6.23.1.1	Definitions

The following definitions are applied in this solution:

Sensing Service Consumer (SSC): The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result.

Sensing Service Request (SSR): The service operation used by the SSC to request sensing service.

### 6.23.2	Procedures

#### 6.23.2.1	Procedure for Sensing Data Processing in the Core Network

Figure 6.23.2.1-1: Procedure for Sensing Data Processing in the Core Network

The following is a description of the procedure for processing in the 5GC only:

0.	The procedures of registering network functions and services, evaluating the Sensing Service Request (SSR) by the SCF and discovering network functions and services required to fulfil the SSR are performed here. The SSR itself is not shown in Figure 6.23.2.1-1. The SSR was triggered before by the Sensing Service Consumer (SSC)

Each SePF shall previously register its capabilities in its NF Profile stored in the NRF. The NF Profile for an SePF may include parameters such as:

-	Supported Processing Capabilities

NOTE 1:	Processing capabilities account for sensing processing algorithms, e.g.  Fusion, AI/ML-Object-Recognition-Model-XYZ, RAD-Analysis (RAD, Range-Angular-Doppler.

-	Supported-Data-Formats (input and output)

-	Current-Load and Capacity metrics

1.	[SensingSePFDiscovery]: The SCF may discover suitable SePFs to fulfil the SSR by querying the NRF with the required processing capabilities. If multiple SePFs are required for the processing, the SCF is responsible for the entire chain orchestration. In that case:

-	SCF discovers all required SePFs from the NRF.

-	SCF constructs an ordered list of the SePFs required for the processing chain.

2.	[SensingEntityConfigurationReq]: The SCF sends a configuration request to a Sensing Entity (SE, i.e. gNB). As an example, the message signature may include an identifier and description of sensing parameters. The identifier is required to identify a SSR during the whole procedure of a sensing session. The description of parameters may include more detailed information,  required to configure sensing, for example:

-	Parameters which are specific to the SE (gNB) which needs to be configured for sensing (e.g. sensing resolution, sensing accuracy, QoS, etc.).

-	Whether the SSR requires a periodic response or a one-off response.

-	Optionally, information regarding which SePF the SE (gNB) should provide the sensing data to, as well as sensing measurements and format type.

NOTE 2:	In this solution, direct, point-to-point interfaces are proposed between the SE (gNB) and the 5GC NFs (i.e. SCF, SePF), tailored for the specific needs of sensing communication for control and data.

Editor's note:		Protocol Interactions between the gNB and the 5GC requires coordination with RAN WGs.

3.	[SensingEntityConfigurationRsp]: The SE (gNB) responds with a message indicating if the configuration was successful or not.

NOTE 3:	After this message the Sensing Entity may start the measurements.

4.	[SensingProcessingReq]: The SCF sends a processing configuration request to the SePFs. The request includes the SSR identifier and additional parameters which may include other SePF(s) involved in sensing processing.

As part of this step, the SCF sends the complete, ordered list of SePFs to the first SePF in the processing chain, as well the final destination address (either NEF or the trusted SSC) to the last SePF in the processing chain as part of the configuration signaling.

5.	[SensingDataRsp]: The SE (gNB) responds with sensing data. Depending on the configuration this event can occur periodically or only once. After SePF #1 has received data from the SE (gNB), SePF #1 processes the data.

6.	[Optional] After processing the data, the SePF #1 needs to know before proceeding with step 7 and step 8 which the next processing entity is. To figure that out, the following options are possible:

-	a static pre-configuration at SePF with a list of the next processing entities. In addition, which entity to choose from in this list might be depending e.g. on the sensing result.

-	the SePF may trigger a service discovery on the service bus to identify the next processing entity. Parameters to discover the next entity via a service discovery request are provided by the parameters in step 4.

-	the parameter-set in the request from step 4 may include an ordered list of next possible entities. The SePF  follows that list.

NOTE 4: 	As an example, the top of the list may be the next target processing entity to visit. After identifying the next entity, the SePF may remove the top from the list and with that the next entry in the list becomes the new top entry. Removing the entity form the top of the list modifies the parameter-set. This changed parameter-set has then to be provided by the actual SePF to the next entity as described in step 7.

-	Each SePF, after completing its task, forwards the intermediate result to the next SePF in the list it received. This eliminates the need for dynamic discovery at each step.

7.	[SensingProcessingReq, Optional]: SePF #1 may send a SensingProcessingReq message to the SePF #2 to notify that new data for processing will arrive. This step is optional and triggered when not all required SePFs were notified by the SCF in step 4.

8.	[SensingResult, Optional]: After processing the data, the result is sent via a sensing result message to SePF #2. It is required to add all required parameters described in steps 4, 7 to this message as well to ensure that the SePF #2 knows from which entity to request new or additional data (if required), and optionally to which entity to send the result. This allows that multiple SePFs can be chained in a controlled way. The sensing result message might be an unsolicited message since SePF #2 was notified before by a SensingProcessingReq message that it has to expect data. SePF #2 confirms the result calculation and may provide it to SePF #1, depending on implementation.

9.	The sensing results are delivered by the SePF to the SSC as follows:

-	Untrusted External AF as SSC: The final SePF sends the result to the SSC. The SePF forwards the result to the NEF, which exposes it to the external untrusted AF via the existing Nnef_EventExposure service.

-	Trusted AF/NF as SSC: The final SePF can deliver the result directly to the SSC using a direct service-based interface without involvement of NEF.

10.	[SensingProcessingRsp]: After the sensing result is delivered to the SSC, SePF#1 will send a response message back to the SCF (or to the requester of the SensingProcessingReq message as in the optional step 6) to signal the success or failure of the request. The response includes a response code as well to indicate the success/failure of the operation.

### 6.23.3	Impacts on services, entities and interfaces

SePF:

-	Supports the operation of multiple SePF instances.

SCF:

-	Supports the management and control of multiple SEs (i.e. gNBs) and SePFs, within the 5GS.

**** End of Changes ****