SA WG2 Meeting #171	S2-2508739

13 – 17 October 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	LG Electronics, Nokia, Lenovo

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

Discussion

There are 8 solutions for KI#1 "Network energy related information exposure" TR 23.700-67v0.2.0.

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution # & title | 1 | 2 | 3 |
| 1: Enhancement of Exposure API to include UE categorization data | X |  |  |
| 2: Policy Control Considering Slice or 5GC Energy Consumption | X | X |  |
| 8: Estimation of Renewable Energy consumption | X |  |  |
| 9: Consumption energy source exposure | X |  |  |
| 10: Energy consumption calculation enhancements | X |  |  |
| 11: Energy Consumption estimation considering idle energy consumption | X |  |  |
| 12: Enhancement of energy related information exposure | X |  |  |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | X |  | X |

Proposal#1: It is proposed to discuss Sol#2 when handling interim agreements/conclusions for KI#2 because Sol#2 is considered more related to KI#2 "Support of service adjustments for the UE".

Proposal#2: It is proposed to discuss Sol#22 when handling interim agreements/conclusions for KI#3 because Sol#22 is considered more related to KI#3 "NF (re)selection and/or UP path adjustment based on energy related information".

Proposal#3: It is proposed to discuss other solutions than Sol#2 and Sol#22 related to KI#1 interim agreements.

Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1.Y	Agreed Principles for KI#1

The following principles are agreed for KI#1.

-	The following new/additional energy related information is exposed by EIF:

-	UE categorization data at the desired granularity e.g. percentile of the consumption of the UE at the granularity of the estimate as proposed by Solution#1 described in clause 6.1.

-	Application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session as proposed by Solution#12 described in clause 6.12.

NOTE 1:	As described in clause 6.12.1, Solution#12 exposes the application ranking related information only to AF(s) related to those applications. For example, vehicle UEs can be the target of Solution#12 as vehicle UEs can operate various applications related to an AF managed or owned by car OEM such as road safety related applications, applications for In-Vehicle Infotainment (e.g. entertainment, navigation, connectivity with smartphones), AI/ML related applications (e.g. Federated Learning).

NOTE 2:	For Solution#12, it is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

7.2.Y	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

-	New formulas taking idle energy consumption into account as proposed by Solution#11 described in clause 6.11.

NOTE 1:	Whether to introduce new formula proposed by Solution#11 as another alternative EC estimation formula in the Annex T of TS 23.501 [2] and leave it to implementation and operator policy to use the appropriate estimation formula by EIF need to be decided.

-	New formulas aiming at minimizing the impact of idle energy consumption on the calculation as proposed by Solution#10 described in clause 6.10.

NOTE 2:	Solution#10 needs to clarify the accuracy aspect due to the measurement error of the node-level energy consumption.

-	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF, as proposed by Solution#8 and Solution#9 described in clause 6.8 and clause 6.9, respectively.

NOTE 3:	The renewable energy information per node-level has dependency on SA5 to discuss whether renewable energy information or other KPIs are more appropriate to capture "green" objectives - e.g. nuclear is not renewable but green.

* * * * End of Changes * * * *