3GPP TSG-WG SA2 Meeting #171	S2-2508792

Wuhan, CN, Oct 13 – 17, 2025	(revision of S2-250xxxx)

Source:	Sony

Title:	[KI#2] New solution: Network Triggered Registration Procedure

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2 / Rel-20

Abstract: A new solution for 5GC to support registration of AIoT devices.

# 1. Introduction/Discussion

A new solution is proposed to support the registration of AIoT devices.

The proposed solution re-use the Rel-19 security solutions related the privacy and mutual authentication before sending commands in either UL or DL directions.

As rel-19 is based on non-registered devices and the network must trigger non-registered devices to transmit UL messages, then the proposed solution is based on this concept that the network must trigger non-registered devices to register.

If this concept is not applied, then new security solution is needed to allow non-registered devices to register.

# 2. Text Proposal

It is proposed to approve the following changes in the TR 23.700-03.

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2.

[7]	3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things".

[8]	3GPP RP-251884: "New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices".

[9]	3GPP RP-251885: "New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2".

[10]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[xx]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

SECOND CHANGE (All text is new)

## 6.X	Solution #X: Network Triggered Registration

### 6.X.0	High-level solution principles

The key technical principles proposed in this solution are summarized below:

- Network triggers the AIoT Device to Register using Inventory procedure to trigger un-registered devices to register.

- Reuse of Rel-19 Inventory and Command procedure to support registration of an AIoT Device as specified in TS 23.369 [3].

- Reuse of Rel-19 security solution for authentication and command protection as specified in TS 33.369 [xx].

### 6.X.1	Description

This solution proposes to reuse the Inventory procedure and command procedure to support Device registration to the network. This allows re-use of rel-19 security procedures i.e., authentication, privacy protection and command protection.

Key Procedures:

- AF invokes a new network service to register AIoT Device(s).

- The AIOTF uses the Inventory procedure to trigger the AIoT Device to check if it is registered to the network. The PLMN ID or NID must be provided in the paging message (see the procedure below for more details).

- Un-registered AIoT Device(s) includes an indication to register in the Inventory Response message. As specified in rel-19 the Inventory Response message includes protected information that the network uses to derive the AIoT Device Permanent Identifier and to authenticate the AIoT Device.

- The AIOTF sends a Command (based on rel-19 functionality) with a Registration Accept message. This message includes the necessary information for the device to enable DO-A transmission. The details of the parameters and information will be identified in solutions for the DO-A transmission and security aspects specified by SA3 for DO-A transmissions.

### 6.X.2	Procedure

Figure 6.x.2-1: Procedure for network triggered registration.

- The AIoT device profile per AIoT Device is provisioned in the ADM.

- The AF invokes Nnef_AIoT_Registration Request in service operation request to the NEF. The Request includes same parameters as Nnef_Inventory Request as specified in TS 23.369 [3].

- The NEF may further authorize the AF request, selects AIOTF and invokes the Naiotf_AIoT_Registration service operation towards each of the selected AIOTF(s). The Request includes same parameters as Naiotf_Inventory Request as specified in TS 23.369 [3].

- The AIOTF receives the Naiotf_AIoT_Registration request and checks the parameters included in the request. The AIOTF may perform authorization of the AF as specified in clause 5.6 in TS 23.369 [3]. The AIOTF prepares the Inventory Request to be sent to the NG-RAN. The AIoT Identification Information to be included in the paging message includes the PLMN ID or NID i.e., the Filtering Information includes the PLMN ID or NID part.

Editor's note:	It is FFS whether and how to provide PLMN ID or NID when single AIoT Device is paged with a Temporary ID.

- AIOTF sends the AIoT Registration Service Response to the NEF containing the accept or reject result for the AIoT service operation request based on step 3.

- NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT service operation request as specified in clause 8.3 in TS 23.369 [3].

- Same as step 7 in the Inventory procedure specified in TS 23.369 [3].

- Same as step 9 in the Inventory procedure specified in TS 23.369 [3].

- The AIoT Device determines whether it matches the AIoT Identification Information as specified in step 9 of the Inventory procedure specified in TS 23.369 [3].

The AIoT Device checks the PLMN ID or NID and whether the AIoT Device is registered or not. In case the AIoT Device is not registered, the AIoT Device includes a Registration Request Indication in the AIOT NAS message. The AIOT NAS message includes the information as specified in step 9 of the Inventory procedure specified in TS 23.369 [3] and the Registration Request Indication.

- Same as step 10 in the Inventory procedure specified in TS 23.369 [3].

- Same as step 11 in the Inventory procedure specified in TS 23.369 [3].

- For each successful Registration Request received, the AIOTF prepares a NAS Registration Accept message and includes the NAS Registration Accept message in the Command Request message. The rest of this step is as specified in step 8 in Command procedure specified in TS 23.369 [3].

NOTE X:	Detailed list of parameters to be included in the NAS Registration Accept message will be determined in coordination with SA3 to support DO-A message transmissions.

- Same as specified in step 9 in Command procedure specified in TS 23.369 [3].

- Same as specified in step 10 in Command procedure specified in TS 23.369 [3]. The Command Response includes the NAS Registration Complete message.

- Same as specified in step 11 in Command procedure specified in TS 23.369 [3]. The Command Response includes the NAS Registration Complete message.

- The AIOTF updates the AIoT Device Context and the AIoT device profile in the ADM that the AIoT device is registered with the network.

- The AIOTF reports the result of the Naiotf_AIoT_Registration request to the NEF by sending the Naiotf_AIoT_Registration Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), Registration status and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Registration Notify messages from multiple AIOTFs.

NOTE Y: The above parameters are the same as included in the Naiotf_AIoT_Inventory Notify message.

17.	The NEF informs the AF of the result of the Nnef_AIoT_Registration request by sending the Nnef_AIoT_Registration Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), Registration status and optionally location of each AIoT Device), AF ID, [Last Report Indication]).

### 6.X.3	Impacts on Services, Entities and Interfaces

NEF:

  - Support new Nnef_AIoT_Registration service.

AIoT Device:

  - Support Registration procedure including:

    - Verify whether it is registered in the specific network,

    - Receive Registration Accept message and

    - Use the parameters included in the accept message and respond with a Register Complete message.

AIOTF:

  - Support Registration service procedure:

    - Invocation of the service by the NEF/AF

    - Receiving in the NAS Inventory Response and indication that the AIoT Device requests to register in the network.

    - Configure the AIoT Device with parameters included in the Registration Accept message.

ADM:

  - Maintain the AIoT Device Profiles with registration status information.

NG-RAN:

  - None

AIoT Reader:

  - None

END OF CHANGES