3GPP TSG SA WG2 Meeting #171	S2-2508904

Wuhan, China, 13 – 17 October 2025	(revision of S2-250xxxx)

Source:	Lenovo

Title:	KI#1: Interim conclusions on controllability and visibility

Document for:	Agreement

Agenda Item:	20.3.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:.

# 1. Discussion

Interim Conclusions on controllability.

Currently the following points are for further consideration as per clause 7.2.1 of 3GPP TR 23.700-04

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

From CN perspective the role of the NF responsible for data collection is to relay a request for standardised data from a UE as requested by a UE-side server. As such, the role of the NF responsible for data collection is to indicate to, selected UE(s), the requirements on the data that needs to be transferred by the UE.

Observation 1: The NF responsible for data collection is to indicate to, selected UE(s), the requirements on the data that needs to be transferred by the UE

How the data collected by the UE are transferred to the NF responsible for data collection are part of how the 5GC NF controls the UE data transfer.

Controlling UE data transfers involves the following aspects:

1.-	Configuring the UE with the address of the NF responsible for data collection so as the UE to establish an IP connection and transfer collected data

2.-	Configuring the UE on how to transfer the collected data via an IP connection via a user plane connection over the 3GPP network (i.e. indicate to the UE how to map the transfer of data to a PDU session) and how the UE should prioritise the transfer of standardised data against other type of active IP connections the UE may have and

3. -	Configuring the UE when to transfer the data (i.e. when the standardised data need to be transferred via an IP connection).

Point 2 is supported using existing mechanisms where the PCF provides appropriate URSP rules to the UE that helps the UE to determine the PDU session to use to transfer the data as well as the SM PCF can provide appropriate QoS rule to help the UE determine how to prioritise the transfer of data against other active application sessions.

For point 3, the NF responsible for data collection can provide conditions for the UE on when to report data, i.e., when to transfer the data via a session with the data collection function (e.g. periodic, once UE buffer is full, time window etc). The UE then transfers the collected data and prioritises the transfer of data according to provisioned QoS rules from an SM PCF.

Observation 2: Controlling UE data transfer involves the configuring the UE with the address of the Data Collection Function to transfer standardised data, configuring the UE how to transfer the standardised data over a PDU session, configuring the UE how to prioritise the transfer of data against other user plane activity the UE may have and configuring the UE with conditions to initiate the transfer of data.

Observation 3: Configuring the UE how to transfer standardised data over a PDU session is based on existing mechanisms using URSP rules

NOTE: This is already captured within interim agreements.

Observation 4: Configuring the UE how to prioritise the transfer of data against other user plane activity the UE may have is based on existing mechanisms based on QoS rules

Observation 5: The NF responsible for data collection may provide conditions on when the transfer of data should take place (e.g. a time window).

In addition, an open question is also whether the trigger of data transfer is also a trigger for data collection. From CN perspective, there is no visibility on what measurement configuration(s) are supported in each RAN node and as such the NF responsible for data collection has no information available to decide when each UE should start data collection to transfer standardised data.

Observation 6: The NF responsible for data collection does not trigger UE to start (or stop) data collection

Based on the points raised above it is proposed to agree the following:

-	Controlling of data transfer involves:

-	Configuring the UE with the address of the NF responsible for data collection to establish an IP session and transfer standardised data,

-	Configuring the UE how to transfer the standardised data to an NF responsible for data collection over a PDU session,

-	Configuring the UE how to prioritise the transfer of data to an NF responsible for data collection against other user plane activity the UE may have and

-	Configuring the UE with conditions to initiate the transfer of data to an NF responsible for data collection

-	The NF responsible for data collection indicates to, selected UE(s), the standardised data to transfer based on standardised data requested by a UE-side server.

-	The NF responsible for data collection may indicate to, selected UE(s), conditions to initiate transfer of data, such as time window, periodic requirements etc. The UE then transfers the collected data and prioritises the transfer of data according to provisioned QoS rules.

-	The NF responsible for data collection does not trigger UE to start/stop data collection procedure with a RAN node.

Interim Conclusions on visibility.

Currently the following point is for further consideration as per clause 7.2.1 of 3GPP TR 23.700-04

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

It is proposed to conclude that the visibility requirement on UE transferring standardized data is to be supported by the NF responsible for data collection. How the visibility requirements is fulfilled (i.e. how the NF checks that the UE transfers only standardized data) is internal implementation at the NF out of scope of 3GPP.

Observation 7: The NF responsible for data collection verifies that the UE transfers standardized data. How the verification is carried out is implementation specific.

# 2. Proposal

The following interim conclusions are proposed.

**** First Change****

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	Controlling of data transfer involves:

-	Configuring the UE with the address of the NF responsible for data collection to establish an IP session and transfer standardised data,

-	Configuring the UE how to transfer the standardised data to an NF responsible for data collection over a PDU session,

-	Configuring the UE how to prioritise the transfer of data to an NF responsible for data collection against other user plane activity the UE may have and

-	Configuring the UE with conditions on when to initiate the transfer of data to an NF responsible for data collection.

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	The NF responsible for data collection indicates to, selected UE(s), the standardised data to transfer based on standardised data requested by a UE-side server.

-	The NF responsible for data collection may indicate to, selected UE(s), conditions to initiate transfer of data, such as time window, periodic requirements etc. The UE then transfers the collected data and prioritises the transfer of data according to provisioned QoS rules.

-	The NF responsible for data collection does not trigger UE to start/stop data collection procedure with a RAN node.

-	The visibility requirement on UE transferring standardized data is to be supported by the NF responsible for data collection. How the visibility requirement is fulfilled (i.e. how the NF checks that the UE transfers only standardized data) is internal implementation at the NF out of scope of 3GPP.

**** Second Change****

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	Whether and how to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

**** End of Change ****