SA WG2 Meeting #171	S2-2508935

13 – 17 Oct, 2025, Wuhan, CN

Source:	Apple

Title:	[WT#1.1] Evolved NAS for 6G

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution makes an analysis of the current NAS design in 5GS, assumes what additions might be made in the Core Network of 6G, and proposes to update work task and key issue descriptions based on the progress made in SA2#170.

# 1   Justifications

The design of NAS protocol in 5GS mandates a connection between the UE and the AMF, a so-called NAS/MM/Signalling Connection, over which the UE can also send and receive NAS messages to/from other Network Functions (NFs) such as SMF, PCF, SMSF, etc., making the communications between the UE and other NFs dependent on the existence of the connection between the UE and AMF.

Some contributions to SA2#170 argued for a new version of the NAS protocol in 6G, which would allow for more flexibility and less dependence on the mobility anchor in the core network. As such, a new NAS design has been suggested, which would make the UE – NF communications completely independent of the UE – AMF NAS/MM/Signalling connection. At the same time, existing network deployments of 5G systems have not really shown any major issues with the current NAS design. It is, therefore, not clear or justified why the communication between the UE and legacy NFs such as SMF, PCF, SMSF, etc., should be changed or adopt a new design.

Based on these arguments, the source company thinks the 6G Core Network should inherit an AMF-like mobility anchor functionality to provide communication between the UE and legacy NFs. However, the current trend of technology points to the assumption that, compared to 5G, there will be more NFs in the 6G Core Network facilitating new services and future use cases. From the perspective of scalability to a higher number of NFs it is, therefore, a reasonable assumption that SA2 needs to investigate how to extend the control plane framework for NF connectivity between UE and CN in 6G and whether an evolved design of NAS protocol could be thought of.

### Proposals

Following the discussions in SA2#170 and considering the observations above, this paper proposes a Work Task revision for Evolved NAS based on the postponed document S2-2508109 at SA2#170. Two changes are introduced with respect to S2-2508109, which can be explained as follows:

- We would like to propose an alternative to the term “operator services” which seems a bit open ended as it includes a lot more than just new 6G services (refer to e.g. 22.011). On the other hand, connectivity services (or traditional UE connectivity-based communication services) and beyond connectivity services would be acceptable to us as well, though we acknowledge based on the discussion in SA2#170 that there may not be quorum for this either. Hence, we are proposing an alternative.

- Adding a NOTE to clarify that the study also covers routing of control signalling for NAS functionalities and 6G services offered by the 3GPP system, including the termination point and interfaces between UE and CN.

In addition, a Key Issue description is proposed based on the same Work Task structure with additional details, synthesizing selected contents from S2-2507988 and S2-2508021 noted at SA2#171. The proposal is to cover all topics in a single Key Issue to avoid overlap and conflicting objectives in multiple KIs. The Key Issue further proposes to study functionality that may be commonly used by the UE and the CN as it appears necessary to consider the interaction between traditional and non-traditional (new) functional aspects from an overall perspective of Control Signalling for 6G as a whole.

# 2   Proposed Changes to 23.801-01

**** First Change ****

## Annex A.X. WT#1.1 Control signalling for 6G

WT#1.1 Study the support for control signalling for 6G System for NAS functionalities and access to 6G services offered by the 3GPP system, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to access 6G services offered by the 3GPP system.

NOTE X: 	It is assumed that this WT covers impacts to 6G System procedures for functionalities such as mobility management, session management and NAS identifiers.

NOTE Y: It is assumed that this WT studies the routing of control signalling for NAS functionalities and 6G services offered by the 3GPP system, including the termination point and interfaces between UE and CN.

**** Second Change (all new text) ****

## 5.X. Key Issue #X: Control signalling for 6G

This Key Issue covers the following aspects:

- Support for control signalling for 6G System for NAS functionalities and access to 6G services offered by the 3GPP system, including the following:

  - Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

  - Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

  - Whether and how to develop generic mechanisms for UE-Core Network interaction to access 6G services offered by the 3GPP system.

- How to support routing of control signalling for NAS functionalities and 6G services offered by the 3GPP system, including the termination point and interfaces between UE and CN.

- For 6G services offered by the 3GPP system, identify functionality that may be commonly used (e.g., service discovery, service authorization, authentication, session configuration and establishment, security, etc.) and study how to support it, while considering aspects such as target NF selection and functional interactions between UE and CN.

NOTE X: 	It is assumed that this KI covers impacts to 6G system procedures for functionalities such as mobility management, session management and NAS identifiers.

**** End of Changes ****