SA WG2 Meeting #172	S2-2509903

17 - 21, November, 2025, Dallas, USA

Source:	ZTE

Title:	KI#2 UC#1 interim conclusion update

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes interim conclusion update.

# 1. Discussion

As captured in solution#33, the NWDAF can detect whether the traffic flow (identified by the IP-5 tuple or QFI) or the source of traffic (identified by the application ID) is abnormal/malicious or not. This is a simple binary classification task which usually has high accuracy. Therefore, it is proposed to add this feature into agreed principle part.

# 2. Proposal

This paper proposes to update solution for Key Issue #2 to TR 23.700-04 as follows.

---------- Start of change ----------

7.1.2.2	Agreed Principles for Use Case #1

- The service consumer of the analytics service may be SMF, PCF, OAM, and UPF and AF. The consumer may take the output Analytics into account.

- To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and/or predictions of:

-	identification of the traffic/traffic flow (e.g. traffic flow filters)

-	Whether the traffic flow(s) identified by the traffic flow filters or the traffic source is abnormal or not

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

- To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters (e.g. IP 5-tuple)

- 	QFI

-	identification of corresponding UEs

- 	Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations

- UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations)

-	adjust packet processing resources,

NOTE Y: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.

- The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE Z: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

---------- End of changes ----------