SA WG2 Meeting #172	S2-2509909

17 – 21 November 2025, Dallas, US

Source:	LG Electronics

Title:	KI#2: Conclusions

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions for KI#2.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

8.2	Conclusion on KI#2

The key issue #2 covers “Authorization and Revocation to Support Sensing Service”.

The following aspects common to the sensing service are concluded as principles for normative work:

-	The Sensing Service authorization consists of two steps:

-	The authorization of the AF for sensing service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9].

-	The Sensing Function (SF) performs authorization of the sensing service request based on the sensing authorization information.

-	Table 8.2-1 shows the sensing authorization information that is required for the SF to perform authorization of the sensing service request.

Table 8.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, object detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

NOTE:	The further detail of the sensing authorization information will be determined during the normative phase.

-	The sensing authorization information is pre-configured or configured by OAM in the SF.

-	The SF can revoke the Sensing Service Request when the authorization criteria are no longer met.

-	The AF can cancel the on-going Sensing Service Request when needed. Then, the SF terminates the on-going sensing service operation between the SF and the SE upon cancellation.

* * * * End of Changes * * * *