SA WG2 Meeting #172	S2-2509935

17-21 November 2025, Dallas, Texas, USA	(was S2-250xxxx)

Source:	NEC

Title:	[General aspects] Architectural Assumptions

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Architectural Assumptions.

Proposal

The contribution builds on the baseline of S2-2509777, which was postponed at the end of SA2#171.

This contribution proposes to update TR 23.801-01 to include the following changes.

**** First Change (Same as S2-2509777) ****

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

6G Core Network (6G CN): In this study, it refers to the Core Network for the 6G System.

NOTE: To what degree 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by 3GPP RAN WGs [ref: RP-252912].

6G System (6GS): 3GPP System consisting of 6G RAN, , 6G CN and UE and supports also connectivity of UEs via specific non-3GPP access(es).

NOTE: Which specific non-3GPP access(es) are supported is studied under WT#1.3.

**** Next Change (Based on S2-2509777) ****

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2. 	This study assumes that control and user plane of the 6G RAN connects to a single core network type, i.e. 6G CN. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

NOTE 1: This does not exclude network sharing scenarios such as Multi-Operator Core Network.

NOTE 2: Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

3.	The 6G system is assumed to maintain the RAN and CN functionality split as in 5GS.

3b.	The study aims to avoid duplication of functionality between 6G RAN and 6G CN.

4.	Voice and video services under operator’s control and in 3GPP scope will be provided by IMS.

6. 	The 6G system is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

**** End of Changes ****