3GPP TSG-WG SA2 Meeting #172	S2-2509989

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#1 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#1. The conclusions are drafted based on the principles agreed in the interim agreements for KI#1 in Clause 7 of 3GPP TR 23.700-14 V1.1.0.

The following conclusions are arrived at based on the interim agreements with the following updates,

- The proposed new NF can be named as Sensing Management Function (SeMF). As it manages the entire sensing service operations, it is appropriate to coin the NF name as Sensing Management Function. The corresponding NOTE 3 has been removed

- The SeMF shall be split into two logical functions i.e., SCF and SPF. The clarification on the functionalities of SCF and SPF has been added

- A new Gateway Sensing Function is not required as the NEF can authorize the AF acting as an entry point. The NEF can discover and select suitable SeMF via NRF or pre-configuration to forward the AF request. The SeMF will do authorization of the sensing service request

- Addition of a new principle on the communication between the Sensing Entity and the SeMF. Considering the various use-case requirements, we feel both direct and indirect communication should be supported in the system

- Removal of NOTE 5

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.a	Conclusion for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded and is recommended for normative work:

Principle 1: One new Network Function i.e., Sensing Management Function (SeMF) is defined to support Sensing Service. The SeMF may contain two logical entities i.e., Sensing control functionality (SCF) and Sensing Processing functionality (SPF). The SCF handles the control plane interactions for sensing procedures. The SPF handles the sensing data to generate the sensing result.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SeMF, e.g. co-located or separated, can be discussed during the normative phase.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

Principle 4: The communication between the Sensing Entity and the SeMF for control signalling (i.e. sensing capability transfer, sensing configuration) and sensing data transfer may be through a direct interface or indirect connection via AMF.

NOTE 3: The protocol for all communication between the gNB and SeMF will be decided by RAN3 and is required to proceed in the normative phase.

Principle 5: The NEF should support the following functionalities,

- Authorization of sensing service consumer i.e., AF outside the trusted domain

- Discovery and Selection of SeMF either via NRF or via pre-configuration

- Sensing Result Exposure

NOTE 4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

# * * * * End of changes * * * *