3GPP TSG-WG SA2 Meeting #172	S2-2510088

Dallas, USA, 17th – 21st November, 2025

Source:	Ericsson

Title:	Adding direct P2P interface between NG-RAN and SF as principles for system architecture (KI#1)

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-19

Abstract: Principles for data transmission and connection establishment between the gNB and Sensing Function (SF) are discussed

# 1. Introduction

This paper discusses principles related to KI#1.

# 2. Discussion

## 2.1	Direct vs Indirect Connection

Mainly three types of architecture have been introduced for the connection and association between gNB and SF, which are shown in the figure below:

Figure 1: schemes of different gNB and SF connections.

Below is the analysis of the solutions provided in the TR:

- Direct architecture (architecture A) is considered in 22 solutions (solutions # 1, 6, 11, 12, 15, 16, 19, 20, 21, 22, 25, 26, 27, 28, 29, 30, 31, 33, 34, 35, 36, 37)

- Hybrid architecture (architecture B) is considered in 6 solutions (solutions # 2, 4, 16, 18, 24, 35)

- Indirect architecture (architecture C) is proposed in 2 solutions (solutions # 5, 7)

Observation 1: majority of the solutions are based on the direct architecture (architecture A).

Both indirect and hybrid architectures, depend on the AMF enhancements. Based on the solution, the impact on AMF is either by definition of a new service or enhancement of a current service for AMF. This dependency results in extra complexity in solutions.

Observation 2: In hybrid and indirect architectures, the involvement of the AMF introduces additional complexity to the solutions.

AMF is primarily responsible for handling access and mobility functions per UE and relaying UE-specific information to other NFs (e.g., LMF).

Although currently AMF is used to carry some non-UE associated traffic (e.g., communication with TSCTSF), the burden is limited. However, the increasing volume of sensing measurements that is relayed to the SF (in indirect solutions) introduces a considerable non-UE load, leading to imbalances in AMF functionalities.

Additionally, LMF supports to send non-UE associated message to NG-RAN via AMF, but prior to the non-UE associated messages, there must be always a UE associated message to be sent first for a UE positioning session. In case of broadcasting Assistance Data, LMF also sends non-UE associated messages to NG-RAN via AMF but final consumer of the service are the UEs

Observation 3: Increasing the non-UE traffic in indirect architecture introduces imbalances in AMF functionalities.

Unlike hybrid and direct architecture, the indirect one does not require establishment of a new link between RAN and SF. This reduces the RAN impact for transferring the sensing measurements to SF. However, it is important to emphasise that for the indirect architecture, the primary impact on the RAN arises from acquisition of the sensing measurements, which is independent of the mentioned data transfer architectures. 

Observation 4: The choice of direct or indirect/hybrid architectures does not significantly change the level of impact on RAN.

With indirect solutions, relaying the sensing measurements introduces extra end-to-end latency and increases energy consumption proportional to the data volume, which can be avoided by implementing a direct connection between gNB and SF. 

Observation 5: Relaying sensing measurements via AMF in indirect architecture introduces additional end to end delay and increases energy consumption for sensing service.

Both indirect and hybrid architectures involve more complex discovery and selection processes for AMF and gNB, which require enhancements for AMF discovery and AMF profile in NRF. 
 
Observation 6: Indirect and hybrid architecture involve more complex discovery and selection procedures.

To summarize, comparing to both indirect and hybrid architecture, the direct architecture avoids AMF impact, reduces the solution complexity and simplifies discovery and selection procedures. Furthermore, compared to indirect architecture, the direct one avoids imbalances in AMF functionalities and improves latency and energy consumption for sensing services. Considering these advantages, it is proposed to adopt direct architecture.

# 3. Proposal

It is proposed to adopt the direct architecture and to include following changes in the TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: SF has direct P2P interface with NG-RAN node. SF uses this direct interface to send request to NG-RAN. NG-RAN provides sensing data to SF also via direct P2P interface.

>>>> Next Change <<<<

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.1	Conclusions for KI#1 on System Architecture to Support Sensing

The following are the principles for KI#1:

-	SF has direct P2P interface with NG-RAN nodes.

-	SF uses this direct interface to send request to NG-RAN.

-	NG-RAN provides sensing data to SF via direct P2P interface.

>>>> End of Changes <<<<