SA WG2 Meeting #172																				S2-2510218

17 - 21 November 2025, Dallas. USA	(was S2-2509823)

Source:	Ericsson (contact point), LGE

Title:	[WT#1.2, Policy] Policy Framework architecture in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides our view on the issues that require further consideration and proposes some clarifications on those aspects that were considered more stable for the scope of the WT#1.2 Policy. It also proposes a Key Issue description.

# 1. Discussion

There have been questions on what type of simplification to the Policy Framework is needed when taking the 5G Policy Framework as starting point of discussion. Diverse aspects have been proposed for several SA2 meeting, majority to SA2#170 where the initial discussion took place, and companies provided detailed motivations, some of the proposals are summarized listed below, although the list may not be complete, based on the motivations and the proposed items to study, bullet 1 is rephrased to state that identification of the areas of simplification will be performed and addresses during the study:

|  |  | Source company | Motivation according to each Source company |
| --- | --- | --- | --- |
| S2-2506469 | 23.801-01: [WT#1.2] 6G Policy Framework | OPPO | UE Policies related: For UE Policy, consistent handling among all types of UE Policy (e.g., URSP, ANDSP, V2XP, ProSeP, RSLPP, A2XP, other new Policy, etc..) should be investigated. |
| S2-2506580 | 23.801-01: [WT#1.2, Policy Framework] Scope and New Key Issue for Policy framework | LG Electronics | AM, SM or UE Policy Association related: In 5GS, multiple PCFs are used for a single UE, i.e. PCF for AM policy association, PCF for UE policy association, and PCF for SM policy association. If a UE establishes multiple PDU Sessions, multiple SM PCFs can be used. In some cases, however, network needs to select the same PCF, e.g. for multiple PDU Sessions, for PCF for UE and PCF for PDU Session. In addition, AMF always selects the same PCF for AM policy and UE policy association. Therefore, it is not clear whether separate PCF needs to be used for each policy association. |
| S2-2506638 | 23.801-01: [WT#1.2, Policy framework] Policy framework for 6G | vivo | UE Policies related: For UE policy aspects, there are some following limitation or restrictions: 1. Huge burden on storage and transmission: to adapt to all services or applications, the UE needs to be pre-configured with a complete, comprehensive, and integrated UE policy, which causes a huge burden on storage and transmission; 2. Low utilization rate: The UE policy is complete, comprehensive and integrated, but only a very small portion is frequently used due to user habits; 3. Poor efficiency: for every newly detected application, it might be necessary to traverse the entire URSP, which is inefficient. 4. Insufficient commercial support: the insufficient supports for URSP in the UE, leading to URSP has not been commercially used on a large scale. Therefore, it is highly necessary to design a simple, efficient and precise UE policy in 6G policy framework. |
| S2-2506927 | 23.801-01: WT#1.2, Policy Framework - Study proposal for Policy and Charging Control | Nokia et al | AM, SM or UE Policy Association related: Currently there are several types of policies (i.e., AM, UE, SM and ProSe) that are provided by the PCF which considers various inputs through communication with other NFs (and other PCF) to carry-out the policy decisions, which is required be studied to optimize by also considering the scalability aspects. For example, the number of policy associations grows as the number of PDU Sessions are created, and there is additional intra PCF communication which could be studied further to optimize, by also considering additional aspects and capabilities that could be added in 6G. |
| S2-2506983 | 23.801-01: [WT#1.2, policy] Policy delivery in 6G | MediaTek Inc. | UE Policies related: The policy provisioning does not consider whether the UE needs the policy and enforce the UE to spend time on signaling procedure to always accept the policy rules provided or updated from the network side. The network does not know about the service or application condition triggered in the UE side |

Other aspects:

- Bullet 2: It is clarified that user preferences is not the same concept as defined in 5G Policy. The intention is study if and how the user preferences can be considered when evaluating UE Polices, as in the existing 5G Policy framework the UE evaluates RSD in the order of precedence.

- Bullet 5: Possible overlap with QoS WT bullet 4 that also study what the UE provides to network as part of bullet 4.

- Bullet 4: Clarification on what changes are notified to the AF.

- Bullet 5, Overlap with bullet 4 on QoS that needs to be resolved, removing bullet 4 until some input from QoS WT is available is one option.

- Bullet 6 about roaming marked as not agreed yet. Main aspect to study when considering the policy aspects of the new 6G services, roaming should be considered.

**** First Change (new text) ****

# Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

To support the Policy Framework in 6GS, the policy and charging control Framework defined in 5G for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

- Identify and address areas of simplification of the policy control framework for 6G, for example:

a)  whether and how to reduce the number of services or interactions to efficiently provide policies to the different policy enforcement points.

b)  whether and how to support the UE requests UE Policies from the 6G CN and the 6G CN to provide UE Policies to the UE.

2. 	 Whether and how to consider user preferences during UE policy evaluation at UE.

NOTE: user preferences refer to the user capability to influence the evaluation of UE policies.

- Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE: General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure.

- Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that policy and charging control, e.g. charging key, changed).

- Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN.

NOTE:  The mechanism for the UE to provide the input for PCC decisions to the 6G CN is studied in WT.1#2 Network exposure, WT#1.1 control signalling for 6G System and the WT1.2 QoS. The AF input defined in 5G can be treated as starting point for discussion.

NOTE: The UE can decide if and when to initiate the request to the network.

NOTE: The UE is not assumed to know whether AF input for QoS is available.

- Whether, how and what to support for policy and charging control in the different roaming scenarios in 6G.

NOTE: 		Roaming aspects of the policy control framework for 6G services will be addressed here.

NOTE:		This WT takes energy efficiency and consumption aspects into account for policy decisions.

NOTE:  	This WT has IWK aspects that are scoped under WT#2.

NOTE: 		This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs.

NOTE:		This WT depends on the overall 6G roaming architecture.

NOTE: 	This WT will consider the output of WT1.2 slicing on improvements of the application traffic mapping to network slice(s)/user plane connection(s).

**** Next Change ****

The work task will be copied here and proposed as key issue.

*** End of Changes ****