3GPP TSG-SA WG2 Meeting #171	S2-2510231

Dallas, USA, 17th – 21st November, 2025

Source:	Ericsson

Title:	KI#2 interim agreements, add BDT Re-negotiation Considering Energy Info When BDT Is Ongoing

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Add PDTQ re-negotiation and BDT (re-)negotiation considering energy info into KI#2 interim agreements.

# 1. Discussion

Solution#2 describes BDT procedure can be enhanced for energy saving based on coarse-grained energy consumption (EnC) information.

Solution#25 proposes to trigger Background Data Transfer (BDT) policies re-negotiation for energy optimization based on energy-related triggers when BDT is ongoing.

The BDT can take a long time to finish if the data volume to be transferred is large. The energy related information of the network and the involved UPF/RAN may change when BDT is ongoing, which requires the core network to apply energy optimization operations before the BDT is finished.

The PDTQ considering energy relating info was agreed in the KI#2 interim agreements. PDTQ and BDT are scheduled data transfer services. There is no reason why only PDTQ is considered while BDT is not.

It is therefore proposed to add PDTQ re-negotiation and BDT (re-)negotiation considering energy info into KI#2 interim agreements.

# 2. Text Proposal

It is proposed to agree the following to 3GPP TR 23.700-67

First change

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The interim agreements on principles for KI#2 are as follows:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	SM policy control.

NOTE 3:	Based on operator policy, it should have the balance between energy saving and user experience.

-	PDTQ policy (re-)negotiation with the AF, e.g. adjusting the time window and/or QoS for  before data transfer and when data transfer is ongoing.

-   BDT policy (re-)negotiation with the AF, e.g. adjusting the time window and/or QoS for before data transfer and when data transfer is ongoing.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editor's note:	Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

# * * * * End of changes * * * *