SA WG2 Meeting #172	S2-2510247

17 - 21 November, 2025, Dallas, USA                  	(revision of S2-2508433)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0074 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Command Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to clarify the command procedure: In step.8 of the command procedure, the Command Request message is sent as a NGAP AIoT information to the NG-RAN directly or via an AMF; in step.11 of the command procedure, the Command Response message is sent as a NGAP AIoT information to the AIOTF directly or via an AMF. The AIOTF should remove the stored mapping between the serving Reader ID and AIoT device ID if the Permanent Disable command is successfully operated. Transaction ID and AF Transaction ID are missing in several steps in the procedures. The service names are Naiotf_AIoT_Notify and Nnef_AIoT_Notify rather than Naiotf_AIoT_Command Notify and Nnef_AIoT_Command Notify in Step. 12 and 13 (in both step description and figure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the NGAP AIoT information and add the description of removing the mapping between the Reader ID and device ID in the command procedure. Clarify Transaction ID and AF Transaction ID in several steps. Correct service name for Naiotf_AIoT_Notify and Nnef_AIoT_Notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The step description for command procedure is inaccurate and the removing of the mapping between the Reader ID and device ID for Permanent Disable command is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications:

-	If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead.

The NEF generates an AF Transaction ID corresponding to this AIoT Command Request.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6.

The AIOTF may perform AMF selection as described in clause 5.3.4.

The AIOTF generates a Transaction ID corresponding to this AIoT Command Request.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, Transaction ID, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, AF Transaction ID, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) as a NGAP AIoT information to the NG-RAN directly or via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

NOTE:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) as a NGAP AIoT information to the AIOTF directly or via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

If the Command Type is "Permanent Disable" and the command is successfully operated, the AIOTF should remove the stored mapping between the serving Reader ID and AIoT device ID.

12.	The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, Transaction ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Command Notify messages from multiple AIOTFs.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location.

When the last report is sent, the AIOTF ends the AIoT Session.

13.	The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, AF Transaction ID, [Last Report Indication]).

END OF CHANGES