SA WG2 Meeting #172	S2-2510263

13 – 17 November, 2025, Dallas, USA

Source:	Ericsson

Title:	Key Issue #1: ADC establishment in alerting phase for early media

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution introduces editor’s notes for all solutions given for KI-1 which addresses establishment of application data channel in alerting phase.

# 1. Discussion

This pCR introduces changes for all solutions given for the Key Issue #1:”…. how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.” by including the multiple early dialog possibilities during alerting phase. This issue has not yet been addressed by any of the proposed solutions and needs to be resolved.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * start change * * * *

# 6	Solutions

## 6.1	Solution #1: ADC establishment in alerting phase initiated by the UE

### 6.1.0	High-level solution Principles

This solution addresses Key Issue #1: DC establishment in alerting phase for early media.

The solution reuses Rel-19 TS 23.228 [2] with the addition that

-	Based on the UE request (via a specific root URL) , the DSCF sends to the UE(s) an application list related with the alerting phase-	The calling party receive BDC from the terminating network during alerting phase.

-	The UE(s) decide whether and which application to download (as for Rel-19 specifications).

-	The UE(s) decide when to release an application that was running in alerting phase; they may decide to continue this application after the alerting phase has finished. (as for Rel-19 specifications).

The solution may e.g. be used for the case where the originating UE1 calls a help centre and is immediately provided with an App that the user will be using when interacting with the call centre.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * next change * * * *

## 6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription

### 6.2.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

-	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel establishment based on UE subscription data when receives INVITE from remote UE.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	IMS AS activates the application data channel upon receiving 180 Ringing from the serving UE.

-	The application data channel may coexistent with other early session services, e.g. CAT, CRS.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * next change * * * *

## 6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer

### 6.3.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * next change * * * *

### 6.4.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * next change * * * *

## 6.5	Solution #5: I Have No Title

Editor’s Note: Solution Title to be provided

### 6.5.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Alerting Tones (CAT) service which is subscribed by the called party and it will provide customized content to the calling party as an early media during the calling process.

The solution has the following principles:

-	The called UE B has subscribed to the DC based CAT service.

-	The calling UE A and called UE B have the DC based early media capability, and both of them have negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the called UE B notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the calling UE A.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * next change * * * *

## 6.6	Solution #6: Support Alerting phase P2A ADC establishment through Autolaunch Parameter

### 6.6.0	High-level solution Principles

This solution supports application data channel establishment during the alerting phase. This solution is based on the following high-level solution principles:

-	The bootstrap data channel media negotiation will follow the existing procedure defined in TS 23.228 [2] Annex AC.7, and that the bootstrap data channel will be established before SIP 180 message from the terminating party.

-	The subscriber will not be able to select any data channel application from the application list. Instead, the autolaunch parameter defined in TS 23.392 [7] will be used by the UE to automatically launch the application that need to be used during the alerting phase. The configuration of data channel application profile is defined in TS 23.392 [7].

NOTE:	Allowing the subscriber to select the data channel application during the alerting phase violates the basic principle of alerting phase, which is to inform the user of incoming call. Any subscriber-requested DC application could be used after the call establishment.

-	Only P2A application data channel is supported. Any P2P or P2A2P application data channel should be established after the 200 OK of initial SIP INVITE.

Editor’s note: This solution proposes the establishment of application data channel during alerting phase. In alerting phase, the SIP session is in “early” phase when multiple parallel early dialogs are possible. If/when a subsequent early dialog is answering an offer from the calling party it is possible that the answerer gives bootstrap data channel answers to the offer. How this extra bootstrap data channel co-exist within the already established bootstrap data channels is FFS.

# * * * * End of changes * * *