3GPP TSG-SA WG2 Meeting #172	S2-2510269

Dallas, USA, November 17–21, 2025

Source:	NEC

Title:	KI#2 Final conclusions

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes the final conclusions for KI#2.

1. Introduction

Final conclusions are proposed for KI#2 “Support of service adjustments for the UE” as per the interim conclusions agreement in clause 7.2 of TR 23.700-67.

Note 4 is updated to avoid a restriction in selecting the best options for the normative spec updates.

2. Proposal

It is proposed to agree the following final conclusions for KI#2 into TR 23.700-67.

* * *   start of change (all new text)   * * * *

## 8.2	Conclusions for KI#2

The conclusions for KI#2 are as follows:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	SM policy control.

NOTE 3:	Based on operator policy, there should be a balance between the energy saving and the user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editors note:	Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Whether existing AF services can be updated or new AF services introduced for the AF request will be decided during the normative phase.

* * *   end of change   * * * *