3GPP SA WG2#172	S2-2510325

Dallas, USA, 17-21 November, 2025

Source:	Tejas Networks Limited

Title:	Update to Solution#12: Sensing service authorization

Spec:	3GPP TR 23.700-14

Agenda item:	20.2.1

Work Item / Release: FS_Sensing_ARC/ Rel-20

Document for:	Approval

Abstract of the contribution: This contribution proposes an update to Solution#12 proposed for Key Issue#2: Authorization and Revocation to Support Sensing Service based on interim agreements in SA2#171.

1. Introduction

Solution#12 addressing key issue #2 i.e., service authorization and revocation ) of sensing service was accepted in SA2#169 meeting.

This paper proposes updates to the accepted solution based on the interim agreements in SA#171.

2. Text Proposal

It is proposed to adopt the following text within TR 23.700-14.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> | <Key Issue #6> |
| 12 |  | X |  |  |  |  |

* * * * First Change* * * *

## 6.X	Solution #X: Authorization and Revocation of Sensing Service

### 6.X.1	High-level solution principles

The solution provided in this paper is based on the following principles:

- A new network Functions (NF)  known as Sensing Function (SF) has been introduced for sensing. It may be further logically split into SCF and SPF. The functionality of SCF and SPF is described below:

  - Sensing Control Function (SCF): The SCF handles control plane signaling related to sensing. SCF is responsible for selecting sensing modes and the participants in the sensing activity in co-ordination with other NFs based on the details provided by the Application Function (AF).

  - Sensing Data Function (SPF): The SPF is a network function that is responsible for processing the sensing data within the core network. It is responsible for routing the sensing data and also for providing an identifier to the tracked object/entity. This function is also responsible for ensuring that the sensing metrics are met.

- The choice of whether the SF is split as described above or colocated is left to implementation.

- The solution defines the authorization and updation/revocation procedure for sensing in two separate callflows.

### 6.X.1	Description

For establishing a new sensing service within the 5G system, the AF initiates the authorization procedure by sending a sensing service request. Following successful authorization, the service is provisioned by the Sensing Function (SF). In case, the parameters provided to the sensing service need to be changed during the service, or to revoke an ongoing sensing task, the SF can initiate an Update procedure.

### 6.X.2	Procedures

### 6.X.2.1	Authorization of sensing service.

Figure 1 shows the procedure for authorization of a sensing service. The AF uses Nnef_ServiceParameter service to provide the service specific parameters to the SF.

Figure 1:Authorisation of sensing service.

- The AF creates a sensing service request. To create a new request, the AF invokes an Nnef_ServiceParameter_Create service operation. The request may include subscription information to the report of the outcome of sensing request.

  - To update or revoke an existing request, the AF invokes an Nnef_ServiceParameter_Update or Nnef_ServiceParameter_delete service operation together with the corresponding Transaction Reference ID which was provided to the AF in Nnef_ServiceParameter_Create response message.

  - The content of this service operation (AF request) includes the information described in Section 6.X.2.1.1.

  - If the AF subscribed to the outcome of sensing, the AF indicates where the AF receives the corresponding notifications.

- The AF sends its request to the NEF. The NEF authorizes the AF request. In case of a trusted AF, the sensing request can be directly sent to the SF.

- The NEF may perform the following:

  - (In the case of Nnef_ServiceParameter_Create): The NEF assigns a Transaction Reference ID to the Nnef_ServiceParameter_Create request.

  - (In the case of Nnef_ServiceParameter_Create or Update): The NEF may need to authorize the service specific parameter provisioning request with the SF by sending a Nsf_ServiceSpecificAuthorisation_Create service operation.

The NEF sends Nsf_ServiceSpecificAuthorisation_Create Request including the Service Description address to receive updates of the authorization from SF.

  - In case the SF(SCF) is unable to process this request then the SF responds to the NEF with a reject message and a failure cause in the Nsf_ServiceSpecificAuthorisation_CreateResponse message (Step 8). The NEF then forwards this response to the AF in the Nnef_ServiceParameter_CreateResponse message.

- The SF (SCF) authorizes the AF request received from the NEF and stores the information in the SF  as "Target Mapping Data". The Service Parameters are delivered to one or more entities in the target area by the SF (SCF) at/before the start of the sensing service.

SF (SCF) maps service description included in the request from the NEF to set of NFs and sensing entities. The mapped NFs depends on the type of sensing activity and the participating sensing entities and sensing functions.

Editor’s Note:  Further authorization of the sensing request if needed, will be based on decisions in SA2.

- The SF (SCF) then sends a “SensingEntityCapabilityInfo Request” to obtain the sensing capability (e.g., whether it is allowed to sense a given target area) and resource availability information for serving the sensing request for the mapped entities.

- The sensing entities provide a “ SensingEntityCapability_Response” to indicate their resource availability status to the SF (SCF).

Editor’s Note: Details of the  SensingEntityCapability  Request/Response are FFS and can be co-ordinated with RAN3..

- The received information including the authorization status, resource availability status etc is mapped to a service descriptor is then updated/stored by the SF(SDR).

- The SF then responds to the NEF with Nsf_ServiceSpecificAuthorisation_Create Response message

- The NEF then forwards this response to the AF in the Nnef_ServiceParameter_CreateResponse message.

- Additionally, whenever the service parameters are updated, the AF is notified of this event using the Nnef_ServiceParameter_Notify.

Editor’s Note:. The interactions between SCFand SPF are left to be implementation dependent.

Editor’s Note: It is FFS whether and how the data collection mechanism is compliant to the data collection framework defined for other 5G NFs.

### 6.X.2.1.1 	Parameters for creation of an Application Function request

For provisioning a service, the Application Function (AF) provides a request to the NEF with the information listed below. If the AF is designated as a trusted AF, then the following parameters can be exchanged with the SCF directly.

- Service Description

Service Description is the information to identify a specific service request. This identifier should be unique within a given AF.

2)	Service Parameters.

Service Parameters are the service specific information which needs to be provisioned in the network, and possibly the sensing entities (transmitter(s) and/or receiver(s)) to support the service identified by the Service Description. This information may comprise response time, reporting frequency and duration, Quality of Service (QoS) Class details e.g., best-effort, sensing accuracy for e.g.  horizontal or vertical accuracy, possibility of false detection to be in a certain range etc.,

3)	Target Area.

This may comprise of a geographical area for which the sensing service is requested. The target area comprises of a set of sensing entities which may vary at different instants of time. The service parameters are provisioned to one or more sensing entities based on if they are participating in the sensing activity. The authorization of an entity can be given based on the capability of the sensing entity, availability of the entity and other area such as if the entities are marked for certain uses.

- Tracked Object Characteristics.

AF may optionally provide details regarding the type of object to be tracked, along with its physical characteristics such as RCS, mean velocity etc., altitude etc.

- Expected Service Time.

The sensing request may be required for a certain time duration. This is also passed to the NEF.

4)	Subscription to events.

The AF may subscribe to notifications about the sensing outcome due to service specific parameter provisioning. This event may be a one- off event, information regarding the presence of an aerial objector periodic data, where the information related to the aerial object (such as velocity, size, geographical co-ordinates etc.) may be transferred to the AF.

### 6.X.2.2	Updating the sensing service.

Figure 2 shows the procedure for updation/revocation of an ongoing sensing service. The initial steps illustrated using dashed lines (steps 0a and 0b) are optional and occur when the AF initiates the updation/revocation procedure.

Figure 2: Updation/revocation of sensing service.

0a. The AF sends an Nnef_SensingService_Authorization_Update request to the NEF. The request may include details such as Service Descriptor, transaction reference ID, Information, Status, Cause for updation.

0b. The NEF forwards the update request as a Nnef_SensingService_Authorization_Update message to the SF. This message may contain previously shared details such as Service Descriptor, transaction reference ID, Information, Status, Cause for updation etc.

0.SF(SCF) provides a successful authorization for the updating the service specific parameters/ updating authorization details for the service e.g., due to a revocation request from the AF, expiration of the sensing request duration, authorization changed in any of the participating NFs or sensing entities.

- The SF sends a Nsf_SensingEntity_Authorisation_Update Message containing (Service Descriptor, transaction reference ID, Information, Status, Cause) to the sensing entities involved in the sensing service to update authorization status for one or more sensing entities. The transaction reference ID denotes a specific transaction within a session (identified by service descriptor) between an NEF and an AF.

- SF then sends a Nsf_ServiceSpecificAuthorisation_UpdateNotify Request (Service Descriptor, transaction reference ID, Information, Status, Cause) message to the NEF to update authorization status for one or more NFs.

- The NEF sends Nsf_ServiceSpecificAuthorisation_UpdateNotify Response message to the SF to acknowledge the authorization update.

- If the authorization is revoked, the service specific parameters present in SF(SDR) are deleted and NEF removes the service specific parameters from other participating NFs.

- The NEF informs the AF that the service parameters authorisation status has changed by sending Nnef_ServiceParameter_Notify Request (Service Descriptor, Status, Cause) message to the AF to update the authorization/ inform that the service has been revoked.

- The AF responds to the NEF with Nnef_ServiceParameter_Notify Response message.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts are envisioned on the existing network functions:

NEF:

- For supporting sensing services, NEF performs functions such as exposure of capabilities and events for other NFs, translation of information between an AF and another NF, support of non-IP data delivery etc.

NRF:

- NRF maintains the NF profile of available NF instances related to sensing (e.g., SF/ (SCF andSPF)) and their supported services

AF:

- For supporting sensing service, the functionality of AF is enhanced to provide service parameters.

* * * * End of Changes * * * *