3GPP TSG-SA WG2 Meeting #172	S2-2510348

# November 17-21, 2025 Dallas, USA	(a revision of S2-2508289)

Source:	Skylo Technologies, ViaSat, Boost Mobile Network, EchoStar

Title:	Interim Conclusion for KI#2

Agenda item:	20.1.1

Document for:	Agreement

Work Item / Release: FS_5GSAT_Ph4_ARC /Rel-20

Abstract of the contribution: This paper proposes Interim conclusions for KI#2

# 1. Discussion

As specified in Subclause 7.4.2 of TS 22.261 [1], the Call Setup Time (CST) for voice over GEO links (1–3 kbps) shall be less than 30s. This allowance accounts for the inherent propagation delay of GEO links.

In practice, ITU-T Rec. E.807 [2] identifies 10s as the maximum CST most users will tolerate before perceiving the service as unreliable. This benchmark is critical for the primary GEO voice use case: emergency communications in remote or off-grid areas. In such scenarios, long CSTs are not acceptable. A delayed connection may cause the caller to abandon the attempt, assuming the call has failed, with potentially life-threatening consequences. Therefore, while the standard permits 30s, the practical requirement for GEO voice, especially for emergency calls, is rapid connection, ideally within the 10s QoE target.

Table 1 provides CST results calculated according to Annex A of TR 23.700-19 [3] and Reference [4]. Cases 1-3 are based on text-based SIP signalling over IP with B2BUA deployment. The IMS message sizes follow Sol#12 (four small messages or two large messages, no pre-condition) and Sol#15 (two large messages, no pre-condition). Cases 4-7 are based on binary-encoded signalling in Sol#4 and Sol#20, with IMS message sizes according to Sol#20.

The analysis shows clear advantages of binary-encoded signalling with non-IP delivery and B2BUA:

- At 1 kbps, binary-encoded signalling achieves 9.58s CST, about half of SIP signalling (18.01 s).

- In the 1-3 kbps range, the CST when using binary-encoded signalling remains below 10s, meeting the QoE benchmark in ITU-T Rec. E.807 [2].

- SIP clear-text signalling results in CSTs between 10.62s and 20.84s, which may not be acceptable for emergency call scenarios over GEO.

Table 1: Call Set Up Time (in Seconds) Analysis for Different Bit Rate Values

|  | Case 1: SIP (4 small message, IP, with B2BUA) | Case 2: SIP (2 big messages, IP, with B2BUA) | Case 3: SIP (2 small messages, IP, with B2BUA) | Case 4: Binary-encoded (IP, no B2BUA) | Case 5: Binary-encoded (IP, with B2BUA) | Case 6: Binary-encoded (non-IP, no B2BUA) | Case 7: Binary-encoded (non-IP, with B2BUA) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1kbps | 18.01 | 20.84 | 13.59 | 16.07 | 10.35 | 13.39 | 9.58 |
| 2kbps | 13.86 | 14.99 | 11.36 | 13.89 | 9.74 | 12.55 | 9.36 |
| 3kbps | 12.48 | 13.04 | 10.62 | 13.16 | 9.54 | 12.27 | 9.29 |

These results indicate that binary-encoded signalling offers clear benefits in GEO scenarios. As such, it may be appropriate to consider binary-encoded alternatives alongside SIP clear-text signalling, to ensure users receive the best QoE that can be technically achieved.

Based on above observations, we propose the following high-level principles for KI#2

- UNI interface should be optimized to support voice call over GEO.

- Due to the transmission constraints over satellite link, a binary-encoded optimized IMS protocol shall be used.

# 2. Proposal

It is proposed to include the following changes in TR 23.700-19 V1.1.0.

# 3. Reference

[1] 3GPP TS 22.261: "Service requirements for the 5G system".

[2] ITU-T Rec. E.807: “Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service”.

[3] 3GPP TR 23.700-19: “Study on Integration of satellite components in the 5G architecture”.

[4] 3GPP SA2 tdoc: available at https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_171_Wuhan_2025-10/INBOX/DRAFTS/Rel-19%2620%205GSAT/Discussion%20on%20GEO%20voice%20signalling.pdf

* * * First Change * * * *

### 8.x	Interim conclusions for Key Issue #2

The following interim conclusions apply for KI#2

- The voice over GEO signalling data shall be transported over the NB-IoT (GEO) user plane.

- Due to the transmission constraints over satellite link, a binary-encoded optimized IMS protocol shall be used.

NOTE: the actual protocol selection is left to CT1.

* * * End of Changes * * * *