3GPP TSG-WG SA2 Meeting #172	S2-2510367

Dallas, USA, Oct 17 - 21, 2025

Source:	China Mobile

Title:	Interim Agreements for KI#2 – DOA Device Registration

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes an Interim agreement for KI#2 for the FS_AmbientIoT_Ph2_ARC TR 23.700-30.

1	Discussion

Based on TR 23.700-30 V0.2.0, Solutions #1-#19 aim to address KI#2 Support for DO-A Capable AIoT Devices. The DOA Device Registration details are summarized below:

Solutions #1, #2, #4, #9, #11, #13, #14, #18, and #19 need registration or similar procedures. Devices must actively register with the network—either through device-initiated or network-triggered methods—to verify their presence, establish the AIoT Device context, and enable further DO-A data exchange transmission.

Solutions #5 and #6 do not need registration, as they depend on pre-set routing or event-triggered reporting. Devices automatically transmit DO-A data without prior network registration.

Solutions #11, #13, #14, and #18 highlight the importance of mobility registration updates. Devices are encouraged to notify the network whenever they move beyond registered areas. The specific process for triggering this update — such as detecting when a device leaves its assigned zone — might require some coordination with the RAN WG.

Solutions #11, #13, and #14 also take into account periodic registration updates. The update process will be initiated once the periodic timer runs out, ensuring everything stays current smoothly and efficiently.

2	Proposal

It is proposed to include the following changes in TR 23.700-30.

* * * * Start of Changes (All texts are new) * * * *

### 7.1.2	Agreed Principles for KI#2

The following principles have been agreed upon for KI#2:

- DO-A capable AIoT devices support initial registration to autonomously inform the network of their presence.

- DO-A capable AIoT devices support mobility and periodic registration.

Editor’s note:  The ability of the reader to broadcast area information to DO-A capable AIoT devices depends on RAN2.

- The Temporary ID is assigned by the AIOTF and is provided to the DO-A capable AIoT device within the Registration Response message. This Temporary AIoT device ID contains the AIOTF ID.

- The DO-A capable AIoT Device includes the Temporary AIoT device ID in NAS requests.

- The AIOTF ID in the Temporary AIoT device ID is used for routing (e.g., gNB decides the AIOTF, and the AIOTF chooses the previous serving AIOTF).

- Once registration is successful, the AIoTF generates an AIoT Device Context and updates the AIoT Device Profile within the ADM.

NOTE 1: Each DO-A capable AIoT device has a single, unique AIoT Device Context in the AIOTF if registered successfully.

* * * * End of Changes * * * *