3GPP TSG-SA WG2 Meeting #172	S2-2510383

Dallas, USA, 13 – 17 November 2025

Source:	ZTE

Title:	KI#2: Update on Solution#20

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes to update solution #20 for KI#2.

1. Discussion

This paper focuses on modifying the CST for MO in solution#20.

Proposed Modification to Clause 6.20.2.4 – Call Setup Time (CST):

The current CST estimation assumes a propagation delay of 285 ms per signaling message, without considering the additional time required for uplink resource scheduling. However, as specified in Annex A1.2, the estimated time for IMS signaling exchange should reflect the uplink scheduling delay. Specifically, the delay for uplink signaling should be multiplied by a factor of 3, resulting in an effective delay of 855 ms per uplink message, while the downlink delay remains at 285 ms.

In complete IMS call setup procedure, the number of signaling messages exchanged before the 200 OK and ACK is 8. Among these, the number of uplink signaling messages is 3 for MO and 5 for MT.

- For MO scenarios, the CST calculation shall be updated to reflect the uplink resource scheduling delay. Each uplink signaling message shall be considered to incur a delay of 855 ms, while each downlink signaling message shall retain the 285 ms delay assumption.

- For MT scenarios, it is proposed to add a note in the specification indicating that, compared to MO, the MT procedure includes 2 additional uplink signaling messages. Thus, an additional delay of: 2 × 285 ms × (5-3) = 1140 ms, shall be added to the CST estimation for MT calls to account for the extra uplink scheduling time.

Proposal: It is proposed to update the CST for MO and MT in this solution.

* * * Start of Change * * * *

6.20	Solution #20: Signalling Optimization via IWK in P-CSCF

6.20.0	High-level solution Principles

This solution addresses KI#2 to support IMS voice call over NB-IoT via GEO connecting to EPC.

The following principles are applied to this solution:

-	Binary encoding of SIP over UNI using the framework defined, e.g. in TS 24.294 [14] for I1 encoding. The UNI reference point is labelled GEO Gm for the purpose of this solution.

NOTE:	The solution uses the I1-like protocol.

-	No restrictions are applied in this solution on UNI signalling in terms of what the UE negotiates, e.g. pre-conditions, 100rel, etc.

-	Only necessary information elements are included in the UNI to enable interoperability and support originating/terminating UE negotiated needs.

-	In summary, the UE, and P-CSCF support for interworking purpose a UE proxy functionality during IMS call-related procedures(i.e. it is not applied to IMS registration procedure), as depicted in Figure 6.20.1-1, that performs the following:

-	Translation from binary encoded SIP to text-based SIP encoding and vice versa.

-	Maintaining an association between the incoming and outgoing sessions in the P-CSCF to ensure compliance to GEO Gm and Mw reference points for interworking purposes. Separate session Identifiers are used for the incoming and outgoing sessions.

-	Storing and managing needed information elements, and updating stored values where applicable, to ensure compliance to the GEO Gm and Mw reference points. -	The UE proxy functionality does not hold session state machine and does not interfere with the state machine implemented in the P-CSCF.

-	IMS Registration over the GEO Gm reference point is performed as is today, i.e. it complies to TS 24.229 [16]. No binary encoding is performed. Hence the P-CSCF supports Gm for the purpose of IMS Registration for GEO satellite access. P-CSCF can dedicate a UDP port for GEO satellite access supporting Gm for IMS Registration and GEO Gm for IMS Call-related procedures which can be notified to UE e.g. via PCO.

Editor's note:	The use of two different ports for handling Gm and GEO Gm is FFS.

Editor's note:	How to distinguish the messages via between Gm and GEO Gm is FFS.

-	This solution does not require any update to the remote end for interworking. Additionally, the solution assumes dedicated bearer for IMS voice.

Additional Assumptions:

-	The identity used for initiating an IMS session is the default IMPU.

-	P-CSCF removes the SIP headers/ SIP Header fields, which do not need to be sent to the UE. The P-CSCF stores the removed information should this be needed for the Mw interactions with the IMS Core.

-	The P-CSCF does not perform additional security checks in relation to IPSec during IMS session initiation or termination procedure.

Editor's note:	This assumption needs to be confirmed by SA WG3.

-	UE Location formatted according to 24.229 [16] PANI encoding is not required. Only the Cell ID is sent, when needed, using the TS 23.003 [22] ECGI encoding.

-	P-Asserted-Id coding is supported over the UNI towards the UE when needed.

-	P-CSCF does not need to send to terminating UEs Information elements whose values have not changed. Similarly, UE does not need to transmit to the P-CSCF IEs whose values have not changed. This is however not reflected in the tables below.

-	SDP negotiation is replaced by pre-configuration where pre-configuration is possible (e.g. codec to be used). For cases where negotiation is required such as UE preference (e.g. 100rel) an IE conveying the preference is used between P-CSCF and UE.

-	Support for privacy header (OIR) can be included with no impact on call session setup time.

-	AGW not impacted by binary encoding of SIP.

6.20.1	Description

The Figure below depicts the functional architecture for this solution. In this architecture; GEO Gm is the new reference point for the UNI for binary encoding of SIP using the TS 24.294 [14] framework.

GEO P-CSCF is the P-CSCF that performs the necessary interworking between the GEO Gm and Mw, based on the high-level principles depicted above and detailed below. As depicted in the figure, the UE Proxy functionality is strictly restricted to signalling interworking between binary-encoded SIP and text-based SIP, with no SIP state machine.

Figure 6.10.1-1: Architecture to Support GEO IMS audio sessions, and Messaging

In this architecture, SIP Registration is proposed to be used as is today, according to TS 24.229 [16], i.e. SIP Registration is text-based, where Gm interface exists also but not shown in the figure.

The solution supports the two alternatives for transporting IMS voice over the dedicated bearer within an IMS IP PDN connection: as non-IP (NIDD) traffic, or as IP traffic. The difference being the SIP message sizes in both cases and well as the interaction between the P-CSCF and PCC/PCRF during dedicated bearer setup.

The Geo GM interface messages are based on modified binary-encoded I1 interface messages. The comparison and modification are illustrated below.

The initial structure has a minimum length of 7 octets. The size of any GEO Gm message varies based on the necessary information elements needed to support the specific aspects being negotiated within an IMS session.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol version number |  |  |  | Protocol identifier |  |  |  |  | 1 |
| Message type |  |  |  |  | R |  | Reason |  | 2 |
| Reason |  |  |  |  |  |  |  |  | 3 |
| Call-Identifier (Part-1) |  |  |  |  |  |  |  |  | 4 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 5 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 6 |
| Sequence-ID |  |  |  |  |  |  |  |  | 7 |
| Information element #1 |  |  |  |  |  |  |  |  |  |
| Information element #2 |  |  |  |  |  |  |  |  |  |

Figure 6.20.1-1: I1 message structure

To ensure compliance to TS 24.229 [16] the protocol over I1 reference point has been extended with:

-	new protocol Messages: ACK, PRACK and UPDATE.

-	additional IEs depicted in yellow in the table below.

The use and processing of these elements and others is described in the procedure clause and referenced tables therein. Additionally, some of the elements currently supported in I1 framework, indicated in green, are not required in this solution as indicated in the two tables below.

Table 6.20.1-1 below is intended to be used for the case of NIDD for IMS voice.

NOTE:	User plane and bearer types including content is handled in other solutions and may impact some of the IEs in the tables below.

Table 6.20.1-1: SIP proposed headers in case of NIDD of IMS Voice

|  |  | Information element | Presence | Format | Length | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol Information | M | V | 1 |  |
|  |  | Message Type | M | V | 2 |  |
| E |  | Call ID | M | V | 3 |  |
| XI |  | Message Sequence Number | M | V | 1 |  |
| S |  | Timestamp | O | TLV | 1 |  |
| TI |  | To-id | O | TLV | 3-X | If e.164 numbers are used the size is 2+7 (for 13 digits) |
| NG |  | From-id | O | TLV | 2-X | If the main IMPU is used the size is only 2 octets |
|  |  | Accept Contact | O | TLV | 5 | not needed for GEO |
|  |  | ERAccept Contact | O | TLV | 3-X | not needed for GEO |
|  |  | Reject Contact | O | TLV | 5 | not needed for GEO |
|  |  | P-asserted-Id | O | TLV | 3-X | Same encoding as for To / From with new IEC |
|  |  | Cell ID | O | TLV | 2+8 | Using TS 23.003 [22] ECGI encoding. No access-type is needed since P-CSCF already knows from Register the UE is in GEO access |
| N |  | Supported | O | TLV | 2+1 | each bit has a predefined value |
| E |  | Required | O | TLV | 2+1 | each bit has a predefined value |
| W |  | Precondition & Codec Negotiation | O | TLV | 2+2 | One octet for Codec profile and one octet for precondition negotiation aligned with SDP. Each bit has a predefined value in both octets. |
|  |  | Rsq/Rack | O | TLV | 2+1 | Rseq and Rack cannot be in the same message. |

Table 6.20.1-2 below is intended to be used for the case of IP transport for IMS voice.

Table 6.20.1-2: SIP proposed headers in case of IP of IMS Voice

|  | Information element | Presence | Format | Length | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Protocol Information | M | V | 1 |  |
|  | Message Type | M | V | 2 |  |
|  | Call ID | M | V | 3 |  |
|  | Message Sequence Number | M | V | 1 |  |
|  | Timestamp | O | TLV | 1 |  |
|  | To-id | O | TLV | 3-X | If e.164 numbers are used the size is 2+7 (for 13 digits) |
|  | From-id | O | TLV | 2-X | If the main IMPU is used the size is only 2 octets |
|  | Accept Contact | O | TLV | 5 | not needed for GEO |
|  | ERAccept Contact | O | TLV | 3-X | not needed for GEO |
|  | Reject Contact | O | TLV | 5 | not needed for GEO |
|  | P-asserted-Id | O | TLV | 3-X | Same encoding as for To / From with new IEC |
|  | Cell ID | O | TLV | 2+8 | Using TS 23.003 [22] ECGI encoding. No access-type is needed since P-CSCF already knows from Register the UE is in GEO access |
|  | Supported | O | TLV | 2+1 | each bit has a predefined value |
|  | Required | O | TLV | 2+1 | each bit has a predefined value |
|  | Precondition & Codec Negotiation | O | TLV | 2+2 | One octet for Codec profile and one octet for precondition negotiation aligned with SDP. Each bit has a predefined value in both octets. |
|  | Rsq/Rack | O | TLV | 2+1 | Rseq and Rack cannot be in the same message. |
|  | AGW-MediaIPAddress | O | TLV | 2+20/40 | IPv4 /IPV6 Address |
|  | AGW-MediaPort | O | TLV | 2+2 | Port |
|  | UE MediaPort | O | TLV | 2+2 | Port |

6.20.2	Procedures

6.20.2.1	IMS Session Initiation with preconditions and 100rel

The call flow depicts an example for session establishment using preconditions and reliable provisional response for 183.

The tables referenced in the call flow steps assume a NIDD for IMS voice.

It is also assumed that the UE obtains a P-CSCF address information supporting GEO-Gm interface during the IMS PDN connection establishment procedure.

Figure 6.20.2.1-1: Originating IMS Session

The following is a brief description in the call flow:

-	In step 1 the UE registers as per existing procedures in TS 24.229 [16]. The P-CSCF stores the contact information without the feature tags, Cell-ID information, Allow, and the registered identity information. Or the UE can decide to register using new Geo GM message-based registration.

NOTE 1:	The UE using existing text-based SIP registration or Geo GM based SIP registration can be based on UE's knowledge of serving PLMN supporting voice services over GEO NTN access.

-	In step 2, a UE is in IDLE mode performs the Service Request Procedure to get connected and subsequently initiates an IMS session.

-	In step 3, the P-CSCF receives a GEO Gm INVITE that includes the information in Table 6.20.2.1-1 per the column entitled "IE included in Geo Gm" under the INVITE UL banner. The P-CSCF stores the received call ID, Cseq, To-id, Cell-ID, Supported, Require and precondition & codec negotiation information for later use.

-	In step 4, the P-CSCF answers with a GEO Gm progress (Reason 100) message to the UE that includes the call ID, and appropriate Cseq. No other information is required in this message.

-	In step 5 the P-CSCF reserves the necessary resources in the AGW using existing procedures in TS 23.228 [6]

-	In step 6, P-CSCF generates a SIP INVITE towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.20.2.1-1 per the column entitled "P-CSCF processing towards the next hop (Mw) under the "INVITE (UL)" banner. For interworking purposes, P-CSCF holds an association between the incoming and outgoing session with the appropriate IE stored for each session.

-	In step 7, P-CSCF receives a 183 Session Progress from the remote end, including the SDP Answer.

-	In step 8, P-CSCF initiates the dedicated bearer establishment using existing procedures. The detail of the procedure varies depending on how IMS Voice is transported over the dedicated bearer:

-	If IMS voice transport as IP traffic is negotiated, then current behaviour does not change.

-	If NIDD for IMS voice is negotiated, then the P-CSCF includes the address of the AGW for forwarding the traffic. This IP address and port will be added by the PGW for UL traffic and removed for DL traffic. The PGW maintains a binding between the UE and the AGW IP address allocated by the AGW for the duration of the session.

NOTE 2:	Other solutions will provide details for this step.

-	In step 9, P-CSCF sends a GEO Gm progress message to the UE that includes the IEs per the column entitled "IE included in Geo Gm" under the "183 with SDP (DL)" banner in Table 6.20.2.1-1. The P-CSCF handling of SIP headers received in the 183 SDP Session Progress from the remote end and the formulation of elements in the GEO GM progress message is depicted in the column entitled "P-CSCF processing towards the next hop (UE)" under the "183 with SDP" (DL) banner.

-	In step 10, the P-CSCF receives a GEO Gm PRACK that includes the information in Table 6.20.2.1-2 per the column entitled "IE included in Geo Gm" under the "PRACK UL" banner.

-	In step 11 P-CSCF generates a SIP PRACK towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.20.2.1-2 per the column entitled "P-CSCF processing towards the next hop (Mw)" under the "PRACK (UL)" banner.

-	In step 12, P-CSCF receives a SIP 200 OK acknowledging the SIP PRACK.

-	In step 13, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled "IE included in Geo Gm" under the "200 OK PRACK (DL)" banner in Table 6.20.2.1-2. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled "P-CSCF processing towards the next hop (UE)" under the "200 OK PRACK (DL)" banner.

-	In step 14, the P-CSCF receives a GEO Gm UPDATE to indicate that precondition is fulfilled at the UE, and that includes the information in Table 6.20.2.1-3 per the column entitled "IE included in Geo Gm" under the "UPDATE UL" banner.

-	In step 15 P-CSCF generates a SIP UPDATE towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.20.2.1-3 per the column entitled "P-CSCF processing towards the next hop (Mw)" under the "UPDATE (UL)" banner.

-	In step 16, P-CSCF receives a SIP 200 OK acknowledging the SIP UPDATE.

-	In step 17, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled "IE included in Geo Gm" under the "200 OK UPDATE (DL)" banner in Table 6.20.2.1-3. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled "P-CSCF processing towards the next hop (UE)" under the "200 OK UPDATE (DL)" banner.

-	In step 18, P-CSCF receives a 180 Ringing from the remote end.

-	In step 19, P-CSCF sends a GEO Gm progress message to the UE that includes the IEs per the column entitled "IE included in Geo Gm" under the "180 Ringing (DL)" banner in Table 6.20.2.1-4. The P-CSCF handling of SIP headers received in the 180 ringing from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled "P-CSCF processing towards the next hop (UE)" under the "180 Ringing (DL)" banner in depicted in Table 6.20.2.1-4.

-	In step 20, P-CSCF receives a SIP 200 OK final response.

-	In step 21, P-CSCF opens the gate for traffic to start flowing in both directions

-	In step 22, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled "IE included in Geo Gm" under the "200 OK (DL)" banner. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled "P-CSCF processing towards the next hop (UE)" under the "200 OK (DL)" banner as depicted in Table 6.20.2.1-4.

NOTE 3:	In the tables below P-CSCF is assumed to be GEO P-CSCF.

Table 6.20.2.1-1: Originating INVITE/100/183 Signalling Information

| I83 with SDP |  |  | PRACK |  | 200 OK PRACK |  | UPDATE |  | 200 OK UPDATE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Used in IR.92 | Needed for GEO? | Comments | Used in IR.92 | Needed for GEO? | Used in IR.92 | Needed for GEO? | Used in IR.92 | Needed for GEO? | Used in IR.92 | Needed for GEO? |
|  |  |  | Yes |  |  |  | Yes |  |  |  |
| yes |  | stored by P-CSCF TH | Yes |  | Yes | Yes | Yes |  | Yes |  |
|  |  |  | Yes |  |  |  | Yes |  |  |  |
| Yes |  |  |  |  |  |  |  |  |  |  |
| Yes |  |  |  |  |  |  | Yes | No | Yes |  |
| Yes | No |  | Yes | No | Yes | No | Yes | No | Yes | No |
| Yes | No |  | Yes | No | Yes | No | Yes | No | Yes | No |
| Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | No |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Yes | No | Yes | No | Yes | No |
| Yes | Yes |  | Yes | No | Yes | Yes |  |  | Yes | Yes |
| Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | Yes | No |  |  | Yes | No |  |  |
|  |  |  | Yes | No |  |  | Yes | No |  |  |
|  |  |  | Yes | No |  |  | Yes | No |  |  |
| Yes | Yes |  | Yes | No |  | Yes | Yes | Yes | Yes | Yes |
|  |  |  | Yes | No |  |  | Yes | No |  |  |
| Yes | No |  |  |  |  |  | Yes | No | Yes |  |
| Yes | No |  | Yes | No | Yes | No | Yes | No | Yes |  |
| Yes | No |  |  |  | Yes | No |  |  | Yes |  |
| Yes | No |  | Yes | No |  |  |  |  |  |  |
| Yes | No |  | Yes |  | Yes | No | Yes | No | Yes |  |

Table 6.20.2.1-2: PRACK/200 OK Signalling Information

|  |  | INVITE (UL) |  |  |  | I83 with SDP (DL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) |
| Request URI | No | Yes |  |  | P-CSCF can use the To value (RFC 3261) |  |  |  |  |
| Via | No | Yes |  |  | Generates new one towards Mw | Yes |  |  | Removed |
| Route | No | Yes |  |  | Uses what is stored by P-CSCF at Registration |  |  |  |  |
| Record Route |  |  |  |  |  | Yes |  |  | Stored for later use, removed towards UE. |
| Contact | No | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  | Stored for later use. |
| To | To-id (Note 1) | Yes | 9 | Yes | Stored and used | Yes |  |  | Removed |
| From | From-id (Note 1) | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  | Removed |
| Call-ID | Call ID (Note 1) | Yes | 3 | Yes | P-CSCF Generates new one and associate it to the stored incoming call-id | Yes | 3 | Yes | P-CSCF uses what was stored |
| Cseq | Sequence-ID (Note 1) | Yes | 1 | Yes | Generates new one towards MW. Incoming value is stored. | Yes | 1 | Yes | P-CSCF creates what is needed based on stored value |
| Rseq/Rack |  |  |  |  |  | Yes | 3 | Yes | Use what is received |
| Accept | No | Yes |  |  | uses what was received at Registration to generate new one |  |  |  |  |
| Allow | No | Yes |  |  | Pre-provisoined or copued during Registration and used |  |  |  | Store, and removed. |
| Supported | No | Yes | 3 | Yes | Stored and used | Yes | 3 | Yes | Use what is received |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  | Yes |  |  | preprovisoned with audio |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No | Yes |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes |  |  |  | Yes | 9 | Yes | Stored and used |
| P-Preferred-Service | No | Yes |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes | 10 | Only Cell-ID is sent | Stored and used to generate PANI towards Mw |  |  |  |  |
| P-Early-Media |  | Yes |  |  |  |  |  |  |  |
| Privacy | No |  |  |  |  |  |  |  |  |
| Session-expires | No | Yes |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes |  |  |  | Yes | 3 | Yes | Use what is received |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  |  |
| Recv-Info |  |  |  |  |  | Yes |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  |  |  |  |  | Yes |  |  | Removed, but may be stored for later use |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation |  | 4 | Yes | Stored and Used |  | 4 | Yes | Stored and used |
|  |  | Length | 33 |  |  |  | 29 |  |  |

Table 6.20.2.1-3: UPDATE/200 OK Signalling Information

|  |  | INVITE (UL) |  |  |  | I83 with SDP (DL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) |
| Request URI | No | Yes |  |  | Inserted by P-CSCF |  |  |  | Removed by P-CSCF |
| Via | No | Yes |  |  | Inserted by P-CSCF | Yes |  |  | Removed by P-CSCF |
| Route | No | Yes |  |  | Inserted by P-CSCF |  |  |  | Removed by P-CSCF |
| Record Route |  |  |  |  |  |  |  |  |  |
| Contact | No | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  |  |
| To | To-id | Yes |  |  | P-CSCF uses what was stored . | Yes |  |  |  |
| From | From-id | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  |  |
| Call-ID | Call ID | Yes | 3 | Yes | Use the corresponding one stored in P-CSCF for UL | Yes | 3 | Yes | Use the correspoding one stored in P-CSCF |
| Cseq | Sequence-ID | Yes | 1 | Yes | Use the proper value for UL | Yes | 1 | Yes | Use the correspoding one stored in P-CSCF |
| RseqRack |  |  |  |  |  |  |  |  |  |
| Accept | No |  |  |  |  |  |  |  |  |
| Allow | No | Yes |  |  |  | Yes |  |  |  |
| Supported | No |  | 3 | Yes | Update what is stored and use | Yes | 3 | Yes | Update what is stored and use |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  |  |  |  |  |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No |  |  |  |  |  |  |  |  |
| P-Asserted-Identity | No |  |  |  |  |  |  |  |  |
| P-Preferred-Service | No |  |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes | 10 | Cell ID (if it changes) | Update stored value and use |  |  |  |  |
| P-Early-Media |  |  |  |  |  |  |  |  |  |
| Privacy | Privacy |  |  |  |  |  |  |  |  |
| Session-expires | No |  |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes | 3 | Yes | Update stored value and use | Yes | 3 | Yes | Update what is stored and use |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  | Remove |
| Content-Length | No | Yes |  |  |  | Yes |  |  | Remove |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  | Remove |
| Recv-Info |  |  |  |  |  |  |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  | Yes |  |  |  | Yes |  |  | Remove |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| Precondition & Codec Negotiation | SDP Preconditio/Codec Negotiation | In SDP | 4 | Yes | Stored and Used | In SDP | 4 | Yes | Update what is stored and use |
|  |  | Total | 27 |  |  |  | 17 |  |  |

Table 6.20.2.1-4: 180/200 OK Signalling Information

|  |  | 180 Ringing (DL) |  |  | Y | 200 OK (DL) |  |  | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) | Yes | Message Length | Yes | P-CSCF processing towards the next hop (UE) |
| Request URI | No |  |  |  |  |  |  |  |  |
| Via | No | Yes |  |  | Removed | Yes |  |  | Removed |
| Route | No |  |  |  |  |  |  |  |  |
| Record Route |  | Yes |  |  | Removed | Yes |  |  | Removed |
| Contact | No | Yes |  |  | Stored and removed | Yes |  |  | Stored and removed |
| To | To-id | Yes |  |  | Removed | Yes |  |  | Removed |
| From | From-id | Yes |  |  | Removed | Yes |  |  | Removed |
| Call-ID | Call ID | Yes | 3 | Yes | P-CSCF uses what was stored for DL | Yes | 3 | Yes | P-CSCF uses what was stored for DL |
| Cseq | Sequence-ID | Yes | 1 | Yes | P-CSCF uses what was stored for DL | Yes | 1 | Yes | P-CSCF uses what was stored for DL |
| RseqRack |  |  |  |  |  |  |  |  |  |
| Accept | No |  |  |  |  | Yes |  |  | Removed |
| Allow | No | Yes |  |  | Removed | Yes |  |  | Update stored value and use |
| Supported | No | Yes | 3 | Yes | Update stored value and use | Yes | 3 | Yes | Update stored value and use |
| User-Agent | No |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No |  |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes |  |  | Removed | Yes | 9 | Yes | Update stored value and use |
| P-Preferred-Service | No |  |  |  |  |  |  |  |  |
| P-Access-Network-Info | No |  |  |  |  |  |  |  |  |
| P-Early-Media |  |  |  |  |  |  |  |  |  |
| Privacy | Privacy |  |  |  |  |  |  |  |  |
| Session-expires | No |  |  |  |  | Yes |  |  | Removed |
| Max-Forwards | No |  |  |  |  |  |  |  |  |
| Proxy-Require | No |  |  |  |  |  |  |  |  |
| Require | No |  | 3 | Yes | Update stored value and use | Yes | 3 | Yes | Update stored value and use |
| Security Verify | No |  |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No |  |  |  |  |  |  |  |  |
| Content-Length | No | Yes |  |  | Removed | Yes |  |  | Removed |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  | Yes |  |  | Removed | Yes |  |  | Removed |
| Recv-Info |  | Yes |  |  | Removed | Yes |  |  | Removed |
| Feature-Caps |  | Yes |  |  | Removed | Yes |  |  | Removed |
| Session-ID |  | Yes |  |  | Removed | Yes |  |  | Removed |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| Precondition & Codec Negotiation | SDP Precondition/Codec Negotiation | In SDP |  |  |  | In SDP | 4 | Yes | Stored |
|  |  | Total | 13 |  |  |  | 26 |  |  |

6.20.2.2	Terminating IMS Session with preconditions and 100rel

The same Figure 6.20.2.1-1 applies in this case as well. However, the focus will be on the terminating UE.

For the terminating call case the same principles described for originating call apply, however with focus on the terminating call leg and UE.

The next 4 tables depict the treatment by the P-CSCF.

Table 6.20.2.2-1: Terminating INVITE/100/183 Signalling Information

|  |  | INVITE (DL) |  |  |  | I83 with SDP (UL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) |
| Request URI | No | Yes |  |  |  |  |  |  |  |
| Via | No | Yes |  |  |  | Yes |  |  | P-CSCF creates its own |
| Route | No | Yes |  |  |  |  |  |  |  |
| Record Route |  |  |  |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| Contact | No | Yes |  |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| To | To-id (NOTE 1) | Yes |  |  |  | Yes |  |  | P-CSCF uses what is stored |
| From | From-id (NOTE 1) | Yes | 9 |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| Call-ID | Call ID (NOTE 1) | Yes | 3 | Yes | P-CSCF creates a new one and bind it to the stored incoming value. | Yes | 3 | Yes | P-CSCF uses what was stored for the UL |
| Cseq | Sequence-ID (NOTE 1) | Yes | 1 | Yes | P-CSCF creates a new one, and store the incoming value for use in DL. | Yes | 1 | Yes | P-CSCF uses what was stored for the UL |
| Rseq/Rack |  |  |  |  |  | Yes | 3 | Yes | Stored and used |
| Accept | No | Yes |  |  |  |  |  |  |  |
| Allow | No | Yes |  |  | Stored and may be used |  |  |  | Pre-configured |
| Supported | No | Yes | 3 | Yes | Stored and used | Yes | 3 | Yes | Use what is received |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  | Yes |  |  |  |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No | Yes |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes | 9 | Yes | Stored and used | Yes | 10 | Yes | Only cell ID is sent over UNI |
| P-Preferred-Service | No | Yes |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes |  |  |  |  |  |  |  |
| P-Early-Media |  | Yes |  |  |  |  |  |  |  |
| Privacy | No |  |  |  |  |  |  |  |  |
| Session-expires | No | Yes |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes |  |  |  | Yes | 3 | Yes | Use what is received |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  |  |
| Recv-Info |  |  |  |  |  | Yes |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  |  |  |  |  | Yes |  |  |  |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation |  | 4 | Yes | Stored and Used |  | 4 | Yes | Stored and used |
|  |  | Length | 32 |  |  |  | 30 |  |  |

Table 6.20.2.2-2: PRACK/200 OK Signalling Information

|  |  | PRACK (DL) |  |  |  | 200 OK PRACK (UL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) |
| Request URI | No | Yes |  |  | Removed |  |  |  |  |
| Via | No | Yes |  |  | Removed | Yes |  |  | Removed |
| Route | No | Yes |  |  |  |  |  |  |  |
| Record Route |  |  |  |  |  |  |  |  |  |
| Contact | No |  |  |  |  |  |  |  |  |
| To | To-id | Yes |  |  | Removed | Yes |  |  | Removed |
| From | From-id | Yes |  |  | Removed | Yes |  |  | Removed |
| Call-ID | Call ID | Yes | 3 | Yes | P-CSCF uses what was stored for DL | Yes | 3 | Yes | P-CSCF uses what was stored for UL |
| Cseq | Sequence-ID | Yes | 1 | Yes | P-CSCF uses what was stored for DL | Yes | 1 | Yes | P-CSCF uses what was stored for UL |
| Rseq/Rack | No | Yes | 3 | Yes | Copied and Used |  |  |  |  |
| Accept | No |  |  |  |  |  |  |  |  |
| Allow | No |  |  |  |  | Yes |  |  | P-CSCF uses what is stored |
| Supported | No | Yes |  |  |  | Yes |  |  | P-CSCF uses what is stored |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact | No |  |  |  |  |  |  |  |  |
| Reject-Contact | No |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No |  |  |  |  |  |  |  |  |
| P-Asserted-Identity | No |  |  |  |  |  |  |  |  |
| P-Preferred-Service | No |  |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes |  |  |  |  |  |  |  |
| P-Early-Media |  |  |  |  |  |  |  |  |  |
| Privacy |  |  |  |  |  |  |  |  |  |
| Session-expires | No |  |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes |  |  |  |  |  |  |  |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No |  |  |  |  |  |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  | Removed |
| Recv-Info |  | Yes |  |  |  |  |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  | Yes |  |  |  | Yes |  |  | Removed |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation | In SDP |  |  |  |  |  |  |  |
|  |  | Length | 10 |  |  |  | 7 |  |  |

Table 6.20.2.2-3: UPDATE/200 OK Signalling Information

|  |  | UPDATE (DL) |  |  |  | 200 OK UPDATE (UL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) |
| Request URI | No | Yes |  |  |  |  |  |  |  |
| Via | No | Yes |  |  | Removed | Yes |  |  | P-CSCF creates its own |
| Route | No | Yes |  |  |  |  |  |  |  |
| Record Route |  |  |  |  |  |  |  |  |  |
| Contact | No | Yes |  |  | Update stored value | Yes |  |  | Use the corresponding one stored in P-CSCF |
| To | To-id | Yes |  |  | Removed | Yes |  |  | Use the corresponding one stored in P-CSCF |
| From | From-id | Yes |  |  | Removed | Yes |  |  | Use the corresponding one stored in P-CSCF |
| Call-ID | Call ID | Yes | 3 | Yes | Use the corresponding one stored in P-CSCF for UL | Yes | 3 | Yes | Use the corresponding one stored in P-CSCF for UL |
| Cseq | Sequence-ID | Yes | 1 | Yes | Use the corresponding one stored in P-CSCF for UL | Yes | 1 | Yes | Use the corresponding one stored in P-CSCF fpr UL |
| RseqRack |  |  |  |  |  |  |  |  |  |
| Accept | No |  |  |  |  |  |  |  |  |
| Allow | No | Yes |  |  | Update stored value | Yes |  |  |  |
| Supported | No |  | 3 | Yes | Update stored value and use | Yes | 3 | Yes | Update what is stored and use |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  |  |  |  |  |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No |  |  |  |  |  |  |  |  |
| P-Asserted-Identity | No |  |  |  |  |  |  |  |  |
| P-Preferred-Service | No |  |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes |  |  | Update stored value |  | 10 | Yes | Only Cell Id if it changed |
| P-Early-Media |  |  |  |  |  |  |  |  |  |
| Privacy | Privacy |  |  |  |  |  |  |  |  |
| Session-expires | No |  |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes | 3 | Yes | Update stored value and use | Yes | 3 | Yes | Update what is stored and use |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  |  |
| Recv-Info |  |  |  |  |  |  |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  | Yes |  |  |  | Yes |  |  |  |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation | In SDP | 4 | Yes | Stored and Used | In SDP | 4 | Yes | Update what is stored and use |
|  |  | Length | 17 |  |  |  | 27 |  |  |

Table 6.20.2.2-4: 180/200 OK Signalling Information

|  |  | 180 Ringing (UL) |  |  | Y | 200 OK UPDATE (UL) |  |  | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) | Yes | Message Length | Yes | P-CSCF processing towards the next hop (Mw) |
| Request URI | No |  |  |  |  |  |  |  |  |
| Via | No | Yes |  |  | P-CSCF inserts its own | Yes |  |  | P-CSCF inserts its own |
| Route | No |  |  |  |  |  |  |  |  |
| Record Route |  | Yes |  |  | P-CSCF uses what is stored | Yes |  |  | P-CSCF uses what is stored |
| Contact | No | Yes |  |  | P-CSCF uses what is stored | Yes |  |  | P-CSCF uses what is stored |
| To | To-id | Yes |  |  | P-CSCF uses what is stored | Yes |  |  | P-CSCF uses what is stored |
| From | From-id | Yes |  |  | P-CSCF uses what is stored | Yes |  |  | P-CSCF uses what is stored |
| Call-ID | Call ID | Yes | 3 | Yes | P-CSCF uses what is stored for the UL | Yes | 3 | Yes | P-CSCF uses what was stored for UL |
| Cseq | Sequence-ID | Yes | 1 | Yes | P-CSCF uses what is stored for the UL | Yes | 1 | Yes | P-CSCF uses what was stored for UL |
| RseqRack |  |  |  |  |  |  |  |  |  |
| Accept | No |  |  |  |  | Yes |  |  |  |
| Allow | No | Yes |  |  |  | Yes |  |  |  |
| Supported | No | Yes |  |  |  | Yes | 3 | Yes | Update stored value and use |
| User-Agent | No |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No |  |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes |  |  |  | Yes |  |  |  |
| P-Preferred-Service | No |  |  |  |  |  |  |  |  |
| P-Access-Network-Info | No |  |  |  |  |  |  |  |  |
| P-Early-Media |  |  |  |  |  |  |  |  |  |
| Privacy | Privacy |  |  |  |  |  |  |  |  |
| Session-expires | No |  |  |  |  | Yes |  |  |  |
| Max-Forwards | No |  |  |  |  |  |  |  |  |
| Proxy-Require | No |  |  |  |  |  |  |  |  |
| Require | No |  |  |  |  | Yes | 3 | Yes | Update stored value and use |
| Security Verify | No |  |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No |  |  |  |  |  |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  | Yes |  |  |  | Yes |  |  |  |
| Recv-Info |  | Yes |  |  |  | Yes |  |  |  |
| Feature-Caps |  | Yes |  |  |  | Yes |  |  |  |
| Session-ID |  | Yes |  |  |  | Yes |  |  |  |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation | In SDP |  |  |  | In SDP | 4 | Yes | Update stored value and use |
|  |  | Length | 7 |  |  |  | 17 |  |  |

6.20.2.3	Support for IP PDN connection for IMS Voice

The impacted messages from using an IP PDN connection for IMS voice in an originating scenario with impact on session setup time is an initial INVITE, a 183-session progress message. The impacts are illustrated in the table 6.20.2.3-1 below.

Table 6.20.2.3-1: Originating INVITE/100/183 Signalling Information - IP PDN connection

|  |  | UPDATE (UL) |  |  |  | I83 with SDP (DL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) |
| Request URI | No | Yes |  |  | P-CSCF can use the To value (RFC 3261) |  |  |  |  |
| Via | No | Yes |  |  | Generates new one towards Mw | Yes |  |  | Removed |
| Route | No | Yes |  |  | Uses what is stored by P-CSCF at Registration |  |  |  |  |
| Record Route |  |  |  |  |  | Yes |  |  | Stored for later use, removed towards UE. |
| Contact | No | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  | Stored for later use. |
| To | To-id (NOTE 1) | Yes | 9 | Yes | Stored and used | Yes |  |  | Removed |
| From | From-id (NOTE 1) | Yes |  |  | P-CSCF uses what was stored during Registration | Yes |  |  | Removed |
| Call-ID | Call ID (NOTE 1) | Yes | 3 | Yes | P-CSCF Generates new one and associate it to the stored incoming call-id | Yes | 3 | Yes | P-CSCF uses what was stored |
| Cseq | Sequence-ID (NOTE 1) | Yes | 1 | Yes | Generates new one towards MW. Incoming value is stored. | Yes | 1 | Yes | P-CSCF creates what is needed based on stored value |
| Rseq/Rack |  |  |  |  |  | Yes | 3 | Yes | Use what is received |
| Accept | No | Yes |  |  | uses what was received at Registration to generate new one |  |  |  |  |
| Allow | No | Yes |  |  | Pre-provisioned or copied during Registration and used |  |  |  | Store, and removed. |
| Supported | No | Yes | 3 | Yes | Stored and used | Yes | 3 | Yes | Use what is received |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  | Yes |  |  | pre-provisioned with audio |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No | Yes |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes |  |  |  | Yes | 9 | Yes | Stored and used |
| P-Preferred-Service | No | Yes |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes | 10 | Only Cell-ID is sent | Stored and used to generate PANI towards Mw |  |  |  |  |
| P-Early-Media |  | Yes |  |  |  |  |  |  |  |
| Privacy | No |  |  |  |  |  |  |  |  |
| Session-expires | No | Yes |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes |  |  |  | Yes | 3 | Yes | Use what is received |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  |  |
| Recv-Info |  |  |  |  |  | Yes |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  |  |  |  |  | Yes |  |  | Removed, but may be stored for later use |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation |  | 4 | Yes | Stored and Used |  | 4 | Yes | Stored and used |
| AGW IP media Address | In SDP |  |  |  |  |  | 44 | Yes | Stored and Used |
| AGW IP port | In SDP |  |  |  |  |  | 4 | Yes | Stored and used |
| IP port | IN SDP |  | 4 | Yes | Stored and used |  |  |  |  |
|  |  | Length | 37 |  |  |  | 77 |  |  |

For a terminating scenario, the impacted messages are an incoming INVITE and 183-session progress message from the UE. The impacts are illustrated in the table 6.20.2.3-2 below.

Table 6.20.2.3-2: Terminating INVITE/100/183 Signalling Information - IP PDN connection

|  |  | INVITE (DL) |  |  |  | I83 with SDP (UL) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIP Headers | I1 correspondent | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (UE) | Used in IR.92 | Message Length | IE included in GEO Gm | P-CSCF processing towards the next hop (Mw) |
| Request URI | No | Yes |  |  |  |  |  |  |  |
| Via | No | Yes |  |  |  | Yes |  |  | P-CSCF creates its own |
| Route | No | Yes |  |  |  |  |  |  |  |
| Record Route |  |  |  |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| Contact | No | Yes |  |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| To | To-id (NOTE 1) | Yes |  |  |  | Yes |  |  | P-CSCF uses what is stored |
| From | From-id (NOTE 1) | Yes | 9 |  | P-CSCF stores the information for use in DL | Yes |  |  | P-CSCF uses what is stored |
| Call-ID | Call ID (NOTE 1) | Yes | 3 | Yes | P-CSCF creates a new one and bind it to the stored incoming value. | Yes | 3 | Yes | P-CSCF uses what was stored for the UL |
| Cseq | Sequence-ID (NOTE 1) | Yes | 1 | Yes | P-CSCF creates a new one, and store the incoming value for use in DL. | Yes | 1 | Yes | P-CSCF uses what was stored for the UL |
| Rseq/Rack |  |  |  |  |  | Yes | 3 | Yes | Stored and used |
| Accept | No | Yes |  |  |  |  |  |  |  |
| Allow | No | Yes |  |  | Stored and may be used |  |  |  | Pre-configured |
| Supported | No | Yes | 3 | Yes | Stored and used | Yes | 3 | Yes | Use what is received |
| User-Agent | No |  |  |  |  |  |  |  |  |
| Accept-Contact |  | Yes |  |  |  |  |  |  |  |
| Reject-Contact |  |  |  |  |  |  |  |  |  |
| P-Preferred-Identity | No | Yes |  |  |  |  |  |  |  |
| P-Asserted-Identity | No | Yes | 9 | Yes | Stored and used | Yes | 10 | Yes | Only cell ID is sent over UNI |
| P-Preferred-Service | No | Yes |  |  |  |  |  |  |  |
| P-Access-Network-Info | No | Yes |  |  |  |  |  |  |  |
| P-Early-Media |  | Yes |  |  |  |  |  |  |  |
| Privacy | No |  |  |  |  |  |  |  |  |
| Session-expires | No | Yes |  |  |  |  |  |  |  |
| Max-Forwards | No | Yes |  |  |  |  |  |  |  |
| Proxy-Require | No | Yes |  |  |  |  |  |  |  |
| Require | No | Yes |  |  |  | Yes | 3 | Yes | Use what is received |
| Security Verify | No | Yes |  |  |  |  |  |  |  |
| Security -Client | No |  |  |  |  |  |  |  |  |
| Content-Type: | No | Yes |  |  |  | Yes |  |  |  |
| Content-Length | No | Yes |  |  |  | Yes |  |  |  |
| Expires | No |  |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  |  |  |  |  |
| Server |  |  |  |  |  | Yes |  |  |  |
| Recv-Info |  |  |  |  |  | Yes |  |  |  |
| Feature-Caps |  |  |  |  |  |  |  |  |  |
| Session-ID |  |  |  |  |  | Yes |  |  |  |
| Path |  |  |  |  |  |  |  |  |  |
| Service-Route |  |  |  |  |  |  |  |  |  |
| SDP Negotiation | SDP Negotiation |  | 4 | Yes | Stored and Used |  | 4 | Yes | Stored and used |
| AGW IP media Address | In SDP |  | 44 | yes |  |  |  |  |  |
| AGW IP port | In SDP |  | 4 | Yes |  |  |  |  |  |
| IP port | IN SDP |  |  |  | Stored and used |  | 4 | Yes | Stored and used |
|  |  | Length | 80 |  |  |  | 34 |  |  |

6.20.2.4	IMS Session Setup Time calculation

6.20.2.4.1	IMS session setup time in case of NIDD for IMS Voice

The table below shows the IMS session up time, under good conditions and excluding the time needed by EPS activities, to be below 10 seconds, in the worst-case scenario of overhead (Orange column) with IPSec, IPv6, TCP, and no ROHC.

NOTE 1:	TCP is not required in this case.

The SIP size (Grey column) is based on the four tables in clause 6.20.2.1.

Table 6.20.2.4.1-1

| Message type | SIP Size (bytes) | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (best case IPv4+UDP+IPSec - 56 Bytes)) | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (worse case IPv6+UDP+IPSec - 76 Bytes) | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (worse case IPv6+TCP+IPSec - 88 Bytes) | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INVITE | 33 | 1.12 | 89 | 1.57 | 109.0 | 1.73 | 121 | 1.82 |
| 100 TRYING | 7 | 0.34 | 63 | 0.79 | 83 | 0.95 | 95 | 1.05 |
| 183 | 29 | 0.52 | 85 | 0.97 | 105.00 | 1.13 | 117 | 1.22 |
| PRACK | 16 | 0.98 | 72 | 1.43 | 92.00 | 1.59 | 104 | 1.69 |
| 200 OK (PRACK) | 10 | 0.37 | 66 | 0.81 | 86.00 | 0.97 | 98 | 1.07 |
| UPDATE | 17 | 0.99 | 73 | 1.44 | 93.00 | 1.60 | 105 | 1.70 |
| 200 OK (UPDATE) | 17 | 0.42 | 73 | 0.87 | 93.00 | 1.03 | 105 | 1.13 |
| 180 Ringing | 10 | 0.37 | 66 | 0.81 | 86.00 | 0.97 | 98 | 1.07 |
| Success 200 OK | 26 | 0.49 | 82 | 0.94 | 102.00 | 1.10 | 114 | 1.20 |
| Adding ACK | 13 | 0.39 | 69 | 0.84 | 89.00 | 1.00 | 101 | 1.09 |
| Total size (bytes) | 178 |  | 675 |  | 855 |  | 963 |  |
| CST over 1Kb/s - (INVITE to 180) (s) |  | 5.11 |  | 8.69 |  | 9.97 |  | 10.75 |

NOTE 2:	The CST values in the last row excludes Success 200 OK and ACK as they are not relevant for session setup time.

NOTE X:	The CST values for MT should be increased by an additional 1.14s to account for the two extra uplink signaling messages.

6.20.2.4.2	IMS session setup time in case of IP transport for IMS voice

The table below shows the IMS session up time, under good conditions and excluding the time needed by EPS activities, to be below 10 seconds, in the worst-case scenario of overhead (Orange column) with IPSec, IPv6, TCP, and no ROHC.

Table 6.20.2.4.2-1

| Message type | SIP Size (bytes) | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (best case IPv4+UDP+IPSec - 56 Bytes)) | times increase | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (worse case IPv6+UDP+IPSec - 76 Bytes) | times increase | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink | SIP+Transport overhead size (worse case IPv6+TCP+IPSec - 88 Bytes) | times increase | GEO transport delay including propagation delay of 855ms for uplink and 285 ms for downlink |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INVITE | 37 | 1.15 | 93 | 2.5 | 1.60 | 113.0 | 3.1 | 1.76 | 125 | 3.4 | 1.86 |
| 100 TRYING | 7 | 0.34 | 63 | 9 | 0.79 | 83 | 11.9 | 0.95 | 95 | 13.6 | 1.05 |
| 183 | 75 | 0.89 | 131 | 1.7 | 1.33 | 151.00 | 2.0 | 1.49 | 163 | 2.2 | 1.59 |
| PRACK | 16 | 0.98 | 72 | 4.5 | 1.43 | 92.00 | 5.8 | 1.59 | 104 | 6.5 | 1.69 |
| 200 OK (PRACK) | 10 | 0.37 | 66 | 6.6 | 0.81 | 86.00 | 8.6 | 0.97 | 98 | 9.8 | 1.07 |
| UPDATE | 17 | 0.99 | 73 | 4.3 | 1.44 | 93.00 | 5.5 | 1.60 | 105 | 6.2 | 1.70 |
| 200 OK (UPDATE) | 17 | 0.42 | 73 | 4.3 | 0.87 | 93.00 | 5.5 | 1.03 | 105 | 6.2 | 1.13 |
| 180 Ringing | 10 | 0.37 | 66 | 6.6 | 0.81 | 86.00 | 8.6 | 0.97 | 98 | 9.8 | 1.07 |
| Success 200 OK | 26 | 0.49 | 82 | 3.2 | 0.94 | 102.00 | 3.9 | 1.10 | 114 | 4.4 | 1.20 |
| Adding ACK | 13 | 0.39 | 69 | 5.3 | 0.84 | 89.00 | 6.8 | 1.00 | 101 | 7.8 | 1.09 |
| Total size (bytes) | 228 |  | 725 | 3.3 |  | 905 | 4.1 |  | 1013 | 4.6 |  |
| CST over 1Kb/s - (INVITE to 180) (s) |  | 5.51 |  |  | 9.08 |  |  | 10.36 |  |  | 11.16 |

NOTE:	The CST values in the last row excludes Success 200 OK and ACK as they are not relevant for session setup time.

NOTE X:	The CST values for MT should be increased by an additional 1.14s to account for the two extra uplink signaling messages.

6.20.3	Impacts to Services, Entities and Interfaces

P-CSCF:

-	Implements UE proxying capability to manage protocol interworking between UNI and Mw.

UE:

-	Implements UNI protocol based on GEO Gm.

* * * End of Change * * * *