3GPP TSG-WG SA2 Meeting #172	S2-2510409

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Conclusion for System Architecture to Support Sensing

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#1 on System Architecture to Support Sensing.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, and the agreed document S2-2509659, this paper provides the conclusion on KI#1 on System Architecture to Support Sensing.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.1	Agreed Principles for KI#1 System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE 4:	Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI’s interim conclusions will be aligned with KI#1 conclusions.

The reference architecture in figure 7.1.1-1 shows the direct connection between the gNB and the Sensing Function. The NS1 reference point is composed by Control Plane protocol and Data connection protocol which are terminated in separate endpoints in SF. In case of split of SF in SCF and SPF deployment, SCF and SPF correspond to the endpoints for sensing control messages and sensing data, respectively.

Figure 7.1.1-1: Direct connection for 5G System Architecture for Sensing.

NOTE 6:	Protocols used over NS1 reference point supporting control messages and data connection procedures will be specified in coordination with RAN WGs.

The 5G System Architecture for supporting Sensing service contains the following service-based interfaces:

Nsf:	Service-based interface exhibited by SF.

NOTE 8:	In case of deployment split in SCF and SPF the Nsf is offered by SCF.

The 5G System Architecture for supporting Sensing service contains the following reference points:

NS1:	Reference point between the gNB and the SF.

NOTE 9:	NS1 supports separate endpoints for Control Plane and Data Connection protocols.

# * * * * Second change * * * *

# 8	Conclusions

Principles in clause 7.1.1 are agreed as the conclusion.

# * * * * End of changes * * * *